| 1. Discussion INTERNATIONAL TELECOMMUNICATION UNION |  |  |  | Focus Group On IPTV |
| --- | --- | --- | --- | --- |
| TELECOMMUNICATION STANDARDIZATION SECTOR STUDY PERIOD 2005-2008 |  |  | FG IPTV-C-0141 |  |
|  |  |  | Original: English |  |
| WG(s):3 |  | 2nd FG IPTV meeting: Busan, 16-20 October 2006 |  |  |
| CONTRIBUTION |  |  |  |  |
| Source: | CATR/MII, China |  |  |  |
| Title: | Adding new content to FGIPTV-OD-0015e chapter 5 |  |  |  |

This contribution presents various types of IPTV security threats, based on ID-50 (most contents of this contribution are collected in FGIPTV-OD-0014e and FGIPTV-OD-0015e) proposed in the last Geneva IPTV FG meeting in July.

2. Assets to be protected in IPTV service

The following paragraphs in this chapter are quoted from FGIPTV-OD-0015e. We made some modifications on them according to ETSI TR 102 033v1.1.1.

The service chain of an IPTV service can be divided into four parts: Content Source, Service Integration & Operations, Content Transport and Content Consumption.

- Content Source/Content provider: The entity that own or are licensed to sell content to IPTV service providers. Although the Service Provider is the primary source for the client at Home, a direct logical information flow may be set up between Content Provider and subscriber e.g. for rights management and protection.

- Service Integration & Operations/Service provider: the entity providing a service to the end-user. The Service Provider acquires/licenses content from Content Providers and packages this into a service. In this sense the service provider is not necessarily transparent to the application and content information flow.

- Content Transport/Delivery network: The delivery network usually is composed of access networks and core or backbone networks, using a variety of network technologies. The delivery network is transparent to the IP traffic.

- Content Consumption/Consumer: the domain where the IPTV services are consumed. In this part a single terminal may be used for service consumption, but also a network of terminals and related devices may be present for this purpose.

The assets that need to be protected in IPTV service are:

- Content: broadcast TV content, VOD content, and PVR content.

- IPTV service resource: servers such as media servers, AAA servers, DRM servers, ..., and operation information such as service logs and billing information.

- IPTV bearer network: equipments (e.g. routers, switches), bandwidth, multicast services, and CDN.

- Subscribers’ information: identity information, billing information, and information about subscriber's privacy.

EPG based program

EPG based program

Content Transport

Service Integration & Operation

Living TV

Movie…

Content Provider

TV

Music

Movie

…

IPTV Service Platform

Content Source

Content Consumption

Service Provider

Delivery Network

Consumers

IP based Network

CDN

Figure 1 – IPTV Service chain

3. IPTV security threats

A potential threat to a system is not necessarily harmful unless there is a corresponding weakness in the system and that weakness happen to be exploited. a point in time when that weakness is exploited. Some presumable vulnerabilities of IPTV service are enumerated below:

- Security leaks in the IPTV application software;

- Vulnerabilities of the operation system installed in IPTV servers (media servers, AAA servers, etc.);

- Vulnerabilities of service management, for example the usage of a weak password;

- Security leak in  the IPTV protocols(such as RTSP, RTP, RTCP, …);

- Lack of redundancy scheme of the key equipments and data.

The following chapters list various types of threats to IPTV service from four aspects.

  - Content Security Threat

Threats to the digital content may include:

- Interception: a breach of confidentiality of the digit content by illegal monitoring service networks.

- Loss or corruption of the digital content: Actions to destroy the integrity of the digital content transferred, such as unauthorized deletion, insertion, modification, re-ordering ….

- Unauthorized viewing.

- Unauthorized copying.

  - Service Security Threat

- Unauthorized service access: A user attempts to access IPTV service in violation of the security policy in force.

- Masquerading / spoofing IPTV service provider.

- Malicious threats aiming at the IPTV servers (AAA servers, media servers, etc….):  may include the hacking aiming at security leaks in application software or communication protocol denial of service attack, etc.

- Theft (often use malicious programs, such as Trojan horse) of the subscribers’ information (e.g. identity information, billing information, subscription information).

  - Network Security Threat

- Intentional threats aiming at the network equipments or resources (bandwidth): malicious attacks such as denial of service to the bearer network.

- Security threats to multicast technical used in IPTV bearer network: such as masquerading / spoofing multicast TV sources, or illegitimate multicast group members.

- Malicious attacks (like DOS, hacking) on nodes in Content Distribution Network.

  - Consumer Security Threat

- Unauthenticated terminal devices connecting to the home network.

- The failure of the receiving terminal (hardware and software) caused by malicious attacks or a mass of unwanted traffic from the network.

4. Proposal

We propose to add the contents of this contribution to chapter 5 “Security Threats” in FGIPTV-OD-0015e.