| 1. Abstract INTERNATIONAL TELECOMMUNICATION UNION |  |  |  | Focus Group On IPTV |
| --- | --- | --- | --- | --- |
| TELECOMMUNICATION STANDARDIZATION SECTOR STUDY PERIOD 2005-2008 |  |  | FG IPTV-C-0153 |  |
|  |  |  | English only |  |
| WG(s):2 |  | 2nd FG IPTV meeting: Busan, 16 – 20 October, 2006 |  |  |
| CONTRIBUTION |  |  |  |  |
| Source: | ZTE Corporation |  |  |  |
| Title: | A Proposal for performance monitoring content |  |  |  |

In working document on performance monitoring for IPTV (FGIPTV-OD-0005e, output document of Geneva July meeting), performance monitoring parameters, monitoring points and monitoring methods were discussed. This proposal gives some opinions on service monitoring contents and network status monitoring methods.

2. Discussion

QoS monitoring can be discussed in two aspects. One is the QoS of bearing network and another is QoS of every channel. From the point of user, the latter is more important. The QoS of  every channel should be monitored from several aspects, such as video, audio, network status, etc. QoS of bearing network is different from network status of every channel. QoS of bearing network is on behalf of the entire state of the network and be accurately measured in the physical layer. While network status of every channel can reflect whether it can meet the service need in the application layer.

2.1According to the above analysis, we give the catalogue of monitoring content (in the section 6 of FGIPTV-OD-0005e) as below:

6.1 Service quality monitoring of every channel

6.1.1 Monitoring point deployment

6.1.2 Video performance monitoring method

6.1.3 Audio performance monitoring method

6.1.4 Network status performance monitoring method

6.2 Bearing network monitoring

2.2 A method of network status monitoring

The network status can be divided into three levels as UNLOADED, LOADED or (and) CONGESTED. The receiver sends receiver reports to the sender periodically. The receiver report includes such information as loss packet rate, delay and jitter, etc. By processing loss packet rate, we can estimate which level current network status belongs to.

The method is shown in Figure 1, which includes the following steps.

The sender receives receiver report including packet loss rate

Figure 1 Network state estimation flowchart

Streaming server estimates network status

Filter handles packet loss rate

packet loss rate

The sender sends multimedia data

- Step 1: The sender sends multimedia data at normal rate.

- Step 2: The sender (or the reference point) receives receiver report including loss packet rate from the client (or the sampling point) periodically.

- Step 3: The sender (or the reference point) processes loss packet rate to estimate current network status.

The influence of abrupt change of network bandwidth (ACNB) should be considered. If packet loss is extraordinarily high, we thought that ACNB occurs. Two main reasons may lead to ACNB. One is transient network jitter and the other is network deterioration. ACNB from network jitter only lasts a short time, thus the reference point should ignore the impact.  Therefore, the low-pass filter is used as shown in Figure 2

Unloaded

Loaded

Congested

Loss(%)

low-pass filter

packet loss

Figure 2 Classification of Network State

- Step 4: According to the value of  , we can estimate current network status.

3. Proposal

This proposal gives the catalogue of performance monitoring content and presents a method to estimate network status by loss packet rate from the point of application .We hope that they can be included in the standard document or become a part of document as reference information.

4. References

Schulzrinne, H., Casner, S., Frederick, R., and V. Jacobson, "RTP: A Transport Protocol for Real-Time Applications", RFC 1189, GMD Fokus, Precept Software, Inc., Xerox Palo Alto Research Center, Lawrence Berkeley National Laboratory, January 1996.

- _____________