| INTERNATIONAL TELECOMMUNICATION UNION |  |  |  | Focus Group On IPTV |
| --- | --- | --- | --- | --- |
| TELECOMMUNICATION STANDARDIZATION SECTOR STUDY PERIOD 2005-2008 |  |  | FG IPTV-C-0162 |  |
|  |  |  | English only |  |
| WG(s): 1 |  | 2nd FG IPTV meeting: Busan, 16-20 October 2006 |  |  |
| CONTRIBUTION |  |  |  |  |
| Source: | ZTE Corporation |  |  |  |
| Title: | The Internationalization Requirements of the IPTV service |  |  |  |

- Introduction

This proposal introduces internationalization requirements of the IPTV service.  There are different constrain standards among different countries.  The IPTV service provider should able to satisfy these standards to operate IPTV service in these countries.

- Discussion

- The IPTV service should support various content rating standards to avoid improper access of contents. End users, especially parents of young children, can then use STB to allow or disallow access to contents based on the information declared in the contents by content provider.

- The IPTV service should support various TV standards. There are currently 3 major television standards used throughout the world: NTSC, SECAM and PAL.

| NTSC | PAL | SECAM |
| --- | --- | --- |
| National Television System Committee | Phase Alternating Line | Sequential Couleur Avec Memoire |
| 525 lines | 625 lines | 625 lines |
| 60 frames per second | 50 frames per second | 50 frames per second |

- The IPTV service should support various TV signal resolution.  For example, in NTSC, Standard Definition (SDTV) signals are called 480i/p. High Definition (HDTV) signals can be 720p, or 1080i/p.

- The IPTV service should support various TV aspect ratios.  For example, 4:3, 16:9.

- The IPTV service should support various languages in the country.

- Proposal

It is proposed to add the requirements of internationalization of IPTV service to the WG1’s requirement document.

___________