| INTERNATIONAL TELECOMMUNICATION UNION |  |  |  | Focus Group On IPTV |
| --- | --- | --- | --- | --- |
| TELECOMMUNICATION STANDARDIZATION SECTOR STUDY PERIOD 2005-2008 |  |  | FG IPTV-C-0217 |  |
|  |  |  | English only |  |
| WG(s): 3 |  | 2nd FG IPTV meeting: Busan, 16-20th October 2006 |  |  |
| CONTRIBUTION |  |  |  |  |
| Source: | Cisco Systems Inc. |  |  |  |
| Title: | End-to-End IPTV Security: Assets, Risks and Threats |  |  |  |

Abstract:

This paper examines end-to-end IPTV security based on asset types so providing an overall structure for the Security and Content Protection Working Group (WG3) to work from.

It starts with a top-level end-to-end architecture and distribution model for IPTV and lists the data and service assets at each point in the distribution chain. It considers the risk to those assets including potential attacks on asset integrity, confidentiality and circumvention of access controls. It characterizes the attackers into four broad groups that can potentially exploit these risks and threaten an asset. Finally, mechanisms are suggested for mitigating the risks to the assets.

Proposal:

We propose that this document be added to the “live” list for use as a baseline document.

Table of Contents

Introduction	3

End-to-end IPTV Architecture	4

Assets, Risks and Threats	5

End-to-End IPTV Assets	5

Risks to Assets	6

Who Threatens the Assets?	6

Content Work Assets	7

Cryptographic Assets	8

Metadata Assets	10

Network Service Assets	10

Acquisition Network	10

Core, Metro, Access and Home Networks	10

Metadata Assets	12

Service Assets	12

Terminal Assets	13

Subscriber Assets	15

Security and Protection Mechanisms	16

Content protection mechanisms	17

Network security mechanisms	18

Terminal protection mechanisms	20

Subscriber security mechanisms	21

Acknowledgements	21

Glossary		21

References	24

# Introduction

A complete end-to-end security analysis of an IPTV system is an enormous task, and one that is ultimately unique to a particular service. We don’t intend to perform such an analysis in this document but instead take an overview of the entire chain of an IPTV system. IPTV consists of much more than television over Internet Protocol (IP), for example it contains “last kilometre” IP delivery, efficiencies in the head-end data centre, storage, metropolitan and wide-area network.  End-to-end IPTV security therefore consists of multiple network sub-systems along a distribution chain from producer to distributor to the consumer of multimedia content.  At each step, each network along the delivery chain contains security or content protection issues.

This document identifies five types of “assets,” which have security or content protection issues.

- Content assets, which reside on transit networks controlled by IPTV businesses along the distribution chain, on networks owned by the IPTV consumer and on the IPTV terminal device.

- Network assets, which are networks operated by entities along the distribution chain, and the home network of the residential consumer.

- Service assets, which support network services and IPTV applications.

- Terminal assets, which the residential consumer uses to process and store content works and other information related to the IPTV service.

- Subscriber assets, which consist of information about the subscriber, the subscriber household, and their IPTV transactions that are processed at multiple points along the IPTV delivery chain.

An “asset” is defined to be data or a service resource in the distribution chain. There are different types of risks associated with each asset.  A “risk” is an attack on an asset that only jeopardizes the asset if the risk is exploited through an “attack.”  The exploitation of a risk in an attack is called a “threat”.   A “passive attack” is one in which the attacker merely gains access to the asset.  In an “active attack,” the attacker is able to alter, manipulate, route, or otherwise actively change the asset.  This paper defines key terms such as assets, risks, threats and attacks in the Glossary chapter.

The issues described in this paper include both conventional security challenges that are characteristic of enterprise networks today and also content protection issues that are found in systems that manage access to copyrighted content works.  The problems and technical solutions are different, however, between security and content protection:

- Computer and network security assumes that the endpoint is not under the control of the attacker and that those in control of each endpoint have a vested interest in protecting secrets.

- Whilst Content Protection assumes the endpoint is untrusted and possibly under the control of the attacker.

A true definition of security necessarily includes the physical security of systems and the procedures that people follow when they access those systems.  Content protection systems, however, are widely deployed in consumer environments having little physical security and no human procedures for protecting content works, particularly if the human user at the endpoint wishes to circumvent the copyright protections on a particular content work.  A determined individual who has the end device and its secrets under their control can practically always circumvent the content protection system.  Content protection mechanisms, therefore, keep honest people honest and do not have the mathematical certainties of security technology.  When content works are protected by physical security and security procedures, as in IPTV businesses, this document uses the term “content security” rather than “content protection”.

The IPTV business networks in the distribution chain face security challenges that include breach of confidentiality, denial of service and other attacks common on business networks today.  The distribution of copyrighted content works from third-party providers, however, increases the likelihood of certain types of attacks, such as attacks on data integrity.  In the IPTV case, moreover, a third-party provider of content works may wish to have additional protections, such as a forensic watermark, applied to their titles as a check on the security of their distributors.  In this case, content protection mechanisms may be used in the business environment in addition to content security mechanisms.  In the home environment, security mechanisms are needed on consumer devices in addition to content protection mechanisms:  The consumers’ devices and home networks face threats to privacy and system integrity as well as the novel challenges of content protection.  This document is concerned with both security and content protection along the distribution chain.

Chapter 2 of this paper describes the IPTV security architecture in terms of the IPTV delivery chain, protocols and processes.   A distribution model is presented and chapter 2 characterizes the end-to-end IPTV Assets, Risks and Threats (ART).  The ART for content works, network, terminal and subscriber assets are described in subsequent chapters.  These chapters are followed by a discussion on security and protection mechanisms to be applied to those assets.

# End-to-end IPTV Architecture

Figure 1: End-to-end Distribution Chain

The focus of entertainment content protection is often on the “last kilometre” of a long distribution chain where copyrighted content works might be illegally copied and then redistributed on the Internet.  The greatest economic threat to entertainment content distribution, however, often occurs at the very beginning of the distribution chain shown in Figure 1.   Post-production houses have access to newly released movies before they even reach the movie theatre.  Theft of a content work at this point of the distribution chain enables DVD counterfeiting, which is the greatest threat to movie studio’s revenue.

The work of professional criminals at the earlier stages of the distribution chain can have the greatest economic impact on content revenues.  Unlike the consumer, however, employees in the production and distribution centre can be subject to security procedures and fired for violating them.  Thus, good content-level security is needed in the enterprises that process IPTV content works as well as in the households that purchase or view them.

Professionals can work from a consumer’s residence in cases where a perfect copy can be obtained using only subscriber equipment.  This problem is addressed in IPTV conditional access and DRM systems that are deployed in the home.  As a general rule, these systems should be robust against attempts to obtain an unauthorized copy through circumvention up to the point where other avenues of attack become cheaper and easier.    Whilst it may be impossible to prevent an unauthorized copy from being made, it should be difficult, apparent to the copier that they’re circumventing a control [Marks & Turnbull], and amenable to forensics.  Ideally, unauthorized copying results in a degraded copy of the content work, but this is often not possible to achieve.

Experience has shown that the overwhelming majority of consumers want to enjoy entertainment content works rather than engage in the illegal trade of them.   Content protection, therefore, must not intrude on the customer’s experience, otherwise, content protection will encourage circumvention of content access controls, as explained by the “Darknet” paper [Darknet].   It is also of great importance that the very best service is provided that is free of denial of service attack, attacks on the integrity of the content work, and attacks on the privacy of consumers and the security of their home networks.  These attacks can occur at various points in the distribution chain of Figure 1.  In the core, metro network, access network, and even (rarely) on the consumer’s home network.  The nature of these attacks, the assets that need protection and the protection mechanisms are considered throughout the remainder of this document.

## Assets, Risks and Threats

Table 1 lists the basic definitions that are used throughout this document.

Table 1: Definitions of Asset, Risk and Threat

| TERM | DEFINITION |
| --- | --- |
| Asset | A data resource or service that is of value to supplier, distributor or user in the distribution chain. |
| Risk | Some potential access or action to an Asset that may result in harm to the interests of the supplier, operator or user of the network. |
| Threat | A means to exploit a risk |

## End-to-End IPTV Assets

This document classifies assets according to one of four types.  Each type is defined in Table 2 and has a chapter devoted to it.  Thus, the chapters are organized by the asset type and analyze the type of risk, risk severity, the source of each threat and its likelihood.

Table 2: List of Assets

| ASSET | DEFINITION |
| --- | --- |
| Content work | Streams or files “at rest” or “in motion” on the network. |
| Network | Network bandwidth, intermediate systems such as routers or firewalls, end systems that are vital to a service such as servers and directories. |
| Terminal | End systems that render, store, and process content works |
| Subscriber | Information about the residential consumer and their transactions |

## Risks to Assets

Table 3 contains a list of risks to each asset.  Each risk in the “Risk” column of the table has a severity and a set of threats by those who might attempt to exploit the risk in an attack.  The severity, threats, potential attackers and the likelihood of attack are considered in the chapters that follow for each type of asset.

Table 3: Lists of Risk to Each Type of Asset

| ASSET | RISK |
| --- | --- |
| Content Work | Theft of content Denial of service |
| Network | Theft of service Unauthorized access or modification of messages Denial of service |
| Terminal | Unauthorized relocation or use Unauthorized access Circumvention of protection mechanisms Denial of service |
| Subscriber | Theft of subscriber or transaction information Unauthorized modification of information |

## Who Threatens the Assets?

Ultimately, an Asset, Risks and Threats analysis classifies and lists the potential attacks on service and information assets.  As part of this analysis, it is useful to consider who is in a position to attack an asset in a particular way and with a particular likelihood. Table 4 classifies each type of attacker, describes them and their motivation.

Table 4: The Attackers

| SOURCE | DESCRIPTION | MOTIVATION |
| --- | --- | --- |
| Cracker | Person with skill and opportunity to threaten an Asset | Personal fame, revenge, ideological predilection |
| Professional | Unlawful reseller of an Asset | Gain from crime |
| Insider | Person employed by supplier or operator who threatens an Asset | Gain from organized crime, revenge, desire to hurt employer or help competitor, personal fame or ideological predilection |
| Consumer | Person in a residential household who uses an Asset in an unauthorized manner | Free service access or unauthorized use of a content work |

# Content Work Assets

Table 5 illustrates an important fact with regard to content work assets:  Assets do not have the same value over a lifetime of use.  Content work assets therefore need more protection during the earliest stages of initial or subsequent release to protect the brand of the rights holder and content distributor.

Table 5: Movie Release and Revenues Window [Lieberfarb]

| Window (months) | 3 | 6 | 9 | 12 | 15 | 18 | 21 | 24 | 27 | 30 | 33 | 36 | 39-50 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| % of Total Revenue | 11 | 13 | 21 | 13 | 6 | 9 | 9 | 6 | 3 | 2 | 2 | 2 | 3 |
| US Theatrical | 80% | 20% |  |  |  |  |  |  |  |  |  |  |  |
| US Home Video |  |  | 65% | 35% |  |  |  |  |  |  |  |  |  |
| US Pay TV & VoD |  |  |  |  | 15% | 25% | 25% | 20% | 15% |  |  |  |  |
| US Network TV |  |  |  |  |  |  |  |  |  | 40% | 30% | 30% |  |
| US Syndicated TV |  |  |  |  |  |  |  |  |  | 15% | 15% | 15% | 55% |
| Int’l Theatrical | 15% | 70% | 15% |  |  |  |  |  |  |  |  |  |  |
| Int’l Home Video |  |  |  | 15% | 25% | 25% | 25% | 10% |  |  |  |  |  |
| Int’l Television |  |  |  |  |  | 10% | 10% | 10% | 10% | 10% | 10% | 10% | 30% |

Table 6 lists the assets, risks and threats (ART) for content works.  The severity of an attack and likelihood of an attack on these assets is specific to an IPTV service and may therefore differ for a particular service.  The values in the table are generic and intended to illustrate the concepts of severity and likelihood of attack. We RECOMMEND that an ART analysis be performed not only during the development of standards but also in the design, implementation and deployment of those standards. This chapter gives an overview of what content types need to be protected from what risk and from whom.  Future work needs to be done to add detail to this overview and to consider attacks on content assets.

Home Video
45%

A particular service that implements IPTV security and protection standards, moreover, will likely need to further refine the ART analysis depending on the content types offered by the service.  In the case of some music titles, for example, encryption is not used whereas in cases such as video it usually is.

Table 6: Content Work ART

| ASSET | RISK | SEVERITY | THREAT | LIKELIHOOD |
| --- | --- | --- | --- | --- |
| Compressed, plaintext content work | Unauthorized copy obtained from network, network device or end system | High if the work is within the release window or deemed highly valuable by the provider; Medium to Low depending on the work. | Cracker | High |
|  |  |  | Professional |  |
|  |  |  | Insider |  |
|  |  |  | Consumer | Medium to Low |
| Compressed, encrypted content work | Unauthorized access | Low | Cracker | Low unless the key is obtained and High otherwise |
|  |  |  | Professional |  |
|  |  |  | Insider |  |
|  |  |  | Consumer | Low |
| Any content work | Denial of service attack | High | Cracker | Low |
|  |  |  | Professional | Low |
|  |  |  | Insider | Low |
|  |  |  | Consumer | Low |

## Cryptographic Assets

For encrypted content works, a key management system (KMS) establishes a hierarchy of keys called a “key ladder”, which can use asymmetric and symmetric keys, as shown in Figure ‎01.  The identity keys, which identify and 8nrols8th8te the Terminal, are typically RSA keypairs though DSA,  El Gamal, or Diffie-Hellman keys may be used [MVV].  Public key encryption and private key signature can establish a key encrypting key (KEK), which is typically a 3DES or 128-bit AES key.  The KEK encrypts a “content key” for content encryption or decryption.  A content key might be for a DVB conditional access, SRTP or ISMAcryp content-decrypton key [DVBCA, SRTP, ISMACryp].  In SRTP and ISMACryp, the content key is called a “master key”, which is input to a key derivation algorithm to obtain 128-bit AES decryption and HMAC integrity keys.

The identity key(s) might be pre-installed on the Terminal or installed with a special KMS procedure.  The KEK is installed by a KMS key-establishment protocol [RFC3547].  The master key is “key wrapped” in the KEK or directly installed using an identity keypair, e.g. public-key encryption.

The KMS handles authentication, key installation, pair-wise key establishment, group key establishment, end-system revocation, and identity replacement.

Figure ‎01: Assumed Key Ladder

| ASSET | RISK | SEVERITY | THREAT | LIKELIHOOD |
| --- | --- | --- | --- | --- |
| Identity key | Unauthorized access: Copy obtained from end system | High in broadcast networks; medium in 2-way networks | Cracker | Medium in broadcast networks; medium to low in 2-way networks |
|  |  |  | Professional |  |
|  |  |  | Insider |  |
|  |  |  | Consumer | Low |
| Key encrypting key (KEK) | Unauthorized access: Copied from end system device | High | Cracker | High |
|  |  |  | Professional |  |
|  |  |  | Insider |  |
|  |  |  | Consumer | Low |
| Content key | Unauthorized access: Copied from end-system | High | Cracker | Medium |
|  |  |  | Professional |  |
|  |  |  | Insider |  |
|  |  |  | Consumer | Medium if KEK is obtained; low otherwise |

## Metadata Assets

| ASSET | RISK | SEVERITY | THREAT | LIKELIHOOD |
| --- | --- | --- | --- | --- |
| Information about content assets | Unauthorized access: Disclosure to unauthorized party | Medium | Cracker | Medium |
|  |  |  | Professional | Medium |
|  |  |  | Insider | High |
|  | Unauthorized access: Modification by unauthorized party | Medium | Cracker | High |
|  |  |  | Professional | Low |
|  |  |  | Insider | Low |
| Information about security, cryptography and content protection | Unauthorized access: Disclosure to unauthorized party | Low | Cracker | High |
|  |  |  | Professional | High |
|  |  |  | Insider | High |
|  | Unauthorized access: Modification by unauthorized party | High | Cracker | High |
|  |  |  | Professional | High |
|  |  |  | Insider | High |

# Network Service Assets

We consider that network service assets, risks and threats along with the severity and likelihood of attacks are specific to an IPTV service and may therefore differ for a particular service.  The values in the table are generic and intended to illustrate the concepts of severity and likelihood of attack.  We RECOMMEND that an ART analysis be performed during the development of standards and in the design, implementation and deployment of those standards.  This chapter gives an overview of what in the network needs to be protected from what risk and from whom.  Future work needs to be done to add detail to this overview and to consider attacks on network assets.

## Acquisition Network

It is a widespread practice today for studios and large content providers to use satellite delivery for movie and TV programming files to network distributors.  This practice is likely to continue for the foreseeable future given the economies and existing infrastructure.  Satellite conditional access systems typically provide security for these satellite feeds and this solution has proven to be practically free of attack.  The transport of these feeds, moreover, typically does not use IP or IP routing.  For these reasons, the Acquisition network is not considered further in this document.

## Core, Metro, Access and Home Networks

The core network is a valuable target of crackers who wish to disrupt network operation for the purposes of demonstrating that they can do such a thing.  It is also an attractive target for Professionals who may want to expropriate the network for their uses or disrupt the core network as part of an extortion scheme.  Disruption of the core network disrupts a national service.

Disruption of the metro network disrupts service across a metropolitan area and is therefore an interesting target for crackers and professionals.  In all but the smallest metropolitan areas, it is and will continue to be common in IPTV delivery architectures to connect multiple local access networks to a single backbone network that spans a metropolitan area or region.  This document assumes that the Assets, Risks and Threats for the Metro Network are no different than for the core network.

The following table lists assets, risks and threats for the Core Network, which is shown in Figure 1.

| ASSET | RISK | SEVERITY | THREAT | LIKELIHOOD |
| --- | --- | --- | --- | --- |
| Network bandwidth | Denial of service attack | Medium | Cracker | High |
|  |  |  | Professional | Medium |
|  |  |  | Insider | Low |
|  |  |  | Consumer | Low |
|  | Unauthorized access or use | High | Cracker | Medium |
|  |  |  | Professional | High |
|  |  |  | Insider | Low |
|  |  |  | Consumer | Low |
| Network Messages | Unauthorized access | High | Cracker | High |
|  |  |  | Professional | Medium |
|  |  |  | Insider | Low |
|  |  |  | Consumer | Low |
|  | Unauthorized modification | High | Cracker | High |
|  |  |  | Professional | Medium |
|  |  |  | Insider | Hlowigh |
|  |  |  | Consumer | Low |
|  | Replay | High | Cracker | High |
|  |  |  | Professional | Medium |
|  |  |  | Insider | Low |
|  |  |  | Consumer | Low |
| Routers, intermediate systems, access servers and system hosts | Denial of service attack | High | Cracker | High |
|  |  |  | Professional | Medium |
|  |  |  | Insider | Low |
|  |  |  | Consumer | Low |
|  | Unauthorized use | High | Cracker | High |
|  |  |  | Professional | Medium |
|  |  |  | Insider | Low |
|  |  |  | Consumer | Low |

The network assets are the same for access and home networks, but their value as targets for crackers and professionals, who are unlikely to bother with an concerted attack on a particular home network.  Today, crackers and professionals focus on launching attack on Internet servers and resources from home networks that are “infected” with viruses and other malware.  This topic is further considered in the Terminal chapter below.

Unlike the home network, the access network does have shared resources, servers and hosts that are attractive to crackers and professionals who could use these resources for launching more extensive attacks on other networks such as the metro network and the core.

## Metadata Assets

In addition to network assets on various networks of the distribution chain, there is value and risk to information about these assets.  A cracker or professional can often make use of information about the network service in an active attack in which information is modified.  An attacker with this power can redirect requests up to and including control of routing.  The

| ASSET | RISK | SEVERITY | THREAT | LIKELIHOOD |
| --- | --- | --- | --- | --- |
| Information about network service | Disclosure to unauthorized party | Low | Cracker | Medium |
|  |  |  | Professional | Low |
|  |  |  | Insider | Low |
|  |  |  | Consumer | Low |
|  | Modification by unauthorized party | High | Cracker | High |
|  |  |  | Professional | High |
|  |  |  | Insider | High |
|  |  |  | Consumer | Low |
| Information about security, cryptography and content protection | Disclosure to unauthorized party | Low | Cracker | High |
|  |  |  | Professional | High |
|  |  |  | Insider | High |
|  |  |  | Consumer | Low |
|  | Modification by unauthorized party | High | Cracker | High |
|  |  |  | Professional | High |
|  |  |  | Insider | High |
|  |  |  | Consumer | Low |

# Service Assets

We consider that service assets, risks and threats along with the severity and likelihood of attacks are specific to an IPTV service and may therefore differ for a particular service.  The values in the table are generic and intended to illustrate the concepts of severity and likelihood of attack.  We RECOMMEND that an ART analysis be performed during the development of standards and in the design, implementation and deployment of those standards. This chapter gives an overview of what service assets need to be protected from what risk and from whom.  Future work needs to be done to add detail to this overview and to consider attacks on service assets.

| ASSET | RISK | SEVERITY | THREAT | LIKELIHOOD |
| --- | --- | --- | --- | --- |
| Domain Name Server | Denial of service attack | Medium | Cracker | High |
|  |  |  | Professional | Medium |
|  |  |  | Insider | Low |
|  |  |  | Consumer | Low |
|  | Unauthorized access | High | Cracker | High |
|  |  |  | Professional | Medium |
|  |  |  | Insider | Low |
|  |  |  | Consumer | Low |
| Electronic Program Guide | Denial of service attack | High | Cracker | High |
|  |  |  | Professional | Medium |
|  |  |  | Insider | Low |
|  |  |  | Consumer | Low |
|  | Unauthorized access | High | Cracker | High |
|  |  |  | Professional | Medium |
|  |  |  | Insider | Low |
|  |  |  | Consumer | Low |
| Media Servers and multiplexers | Denial of service attack | High | Cracker | High |
|  |  |  | Professional | Low |
|  |  |  | Insider | Low |
|  |  |  | Consumer | Low |
|  | Unauthorized access | High | Cracker | High |
|  |  |  | Professional | Low |
|  |  |  | Insider | Low |
|  |  |  | Consumer | Low |
| CAS and Subscriber Management | Denial of service attack | High | Cracker | High |
|  |  |  | Professional | Low |
|  |  |  | Insider | Low |
|  |  |  | Consumer | Low |
|  | Unauthorized use | High | Cracker | High |
|  |  |  | Professional | Low |
|  |  |  | Insider | Low |
|  |  |  | Consumer | Low |

# Terminal Assets

This chapter considers assets, risks and threats for the “terminal” or end-system device where content works are stored, rendered, and in some cases forwarded to other devices.  This chapter considers potential risks to those assets as well as the severity and likelihood of attack by each class of attacker.  The severity of an attack and likelihood of an attack on these assets is specific to an IPTV service and may therefore differ for a particular service.  The values in the table are generic and intended to illustrate the concepts of severity and likelihood of attack.  We RECOMMEND that an ART analysis be performed during the development of standards and in the design, implementation and deployment of those standards. This chapter gives an overview of what terminal assets need to be protected from what risk and from whom.  Future work needs to be done to add detail to this overview and to consider attacks on terminal assets.

As with the other assets, a terminal has a set of assets that can be listed in a table.  In addition to assets, a terminal asset typically has the important property of “Localization” because the device is assumed to be close to a subscriber, for example within the physical boundaries of the subscriber’s house, or in the pocket of a subscriber for a mobile terminal device.  Localization is often important to the protection of a content work or service when access is to be controlled to devices that belong to a household.  These assumptions may underlie a variety of future services and content-work business models [DVB CPCM], and this makes “locality” an asset that can be attacked in various ways.

The other terminal assets are the physical resources of the terminal, which are processor, disk, and output interfaces.  In the case of most services, the output interfaces will be protected interfaces such as HDCP or HDMI interfaces (see Glossary for definitions).  Other terminal assets are described in separate chapters of this document:  Content work assets and subscriber assets have their own chapters; these are referenced but not duplicated in this chapter.

Table 2: Terminal

| ASSET | RISK | SEVERITY | THREAT | LIKELIHOOD |
| --- | --- | --- | --- | --- |
| Locality | Fixed-location device that is outside residence is given service | Medium | Cracker | Low |
|  |  |  | Professional | Low |
|  |  |  | Insider | Low |
|  |  |  | Consumer | Medium |
|  | Mobile device owned by non-subscriber is given service | Medium | Cracker | Low |
|  |  |  | Professional | Low |
|  |  |  | Insider | Low |
|  |  |  | Consumer | Medium |
| Processor and Disk | Infection by malware | High | Cracker | Low unless executables downloaded from the network |
|  |  |  | Professional | Low |
|  |  |  | Insider | Low |
|  |  |  | Consumer | Low |
|  | Unauthorized use | High | Cracker | Low |
|  |  |  | Professional | Low |
|  |  |  | Insider | Low |
|  |  |  | Consumer | Low |
| Interface | Subversion of content protection controls (e.g. HDCP, HDMI) | Medium | Cracker | Low |
|  |  |  | Professional | Low |
|  |  |  | Insider | Low |
|  |  |  | Consumer | Low |

# Subscriber Assets

Subscriber information is at risk as recent leaks of corporate and government records have shown.  As with many of the assets discussed in this document, subscriber information can be obtained from multiple points in the delivery chain such as at servers in the metropolitan network or attached to the core network.

This chapter considers potential risks to subscriber assets as well as the severity and likelihood of attack by each class of attacker.  The severity of an attack and likelihood of an attack on these assets is specific to an IPTV service and may therefore differ for a particular service.  The values in the table are generic and intended to illustrate the concepts of severity and likelihood of attack.  We RECOMMEND that an ART analysis be performed during the development of standards and in the design, implementation and deployment of those standards. This chapter gives an overview of what subscriber assets need to be protected from what risk and from whom.  Future work needs to be done to add detail to this overview and to consider attacks on subscriber assets.

Loss of transaction information is also a risk.  As with subscriber information, this asset may be exposed on the Access Network, Metropolitan Network, Core Network, and server repositories.

Subscriber ART with Severity and Likelihood of Attack

| ASSET | RISK | SEVERITY | THREAT | LIKELIHOOD |
| --- | --- | --- | --- | --- |
| Subscriber information | Unauthorized access: Disclosure to unauthorized party | High | Cracker | Medium |
|  |  |  | Professional | Medium |
|  |  |  | Insider | Low |
|  |  |  | Consumer | Low |
|  | Unauthorized access: Modification by unauthorized party | High | Cracker | Medium |
|  |  |  | Professional | Medium |
|  |  |  | Insider | Low |
|  |  |  | Consumer | Low |
| Transaction information | Unauthorized access: Disclosure to unauthorized party | High | Cracker | Medium |
|  |  |  | Professional | Medium |
|  |  |  | Insider | Low |
|  |  |  | Consumer | Low |
|  | Unauthorized access: Modification by unauthorized party | High | Cracker | Medium |
|  |  |  | Professional | Medium |
|  |  |  | Insider | High |
|  |  |  | Consumer | Low |

# Security and Protection Mechanisms

“Content security” offers a higher-level of protection for content works than “content protection”, which lacks policies to ensure physical security of end systems, trained personnel who are motivated to follow security procedures, and auditable processes that find and correct vulnerabilities.  Such security mechanisms and procedures offer a higher level of security than the content protection mechanisms found on customers’ terminals.  These mechanisms and procedures can be implemented on distributors and producers networks but not in the consumer’s home.  One might argue that content protection is the technology used to protect content works on Terminals and the end-to-end distribution chain is no more secure than its weakest link.  In some cases, however, the content work that is provided to the terminal is not of the same quality or form as what a content distributor stores on its servers and other systems.  The content work may be provided to certain terminals only in a lower resolution, or as a stream rather than as a file, or not before a certain date.  Thus, there are often very good reasons for the content distributor and producer to offer a higher level of security for content works than are provided to the terminals of their customers.  These security mechanisms are described under separate bullets below.

- Strong identity and authentication.  A state of the art public key infrastructure can be maintained within a single enterprise or between enterprises that uses at least 2048-bit keys that are securely provisioned on end systems and managed by a certifiably secure credential system.

- Physical security of end systems.  Servers, caches, hosts and other end systems that store plaintext content works or keys to encrypted content works will be in a room or closet that has restricted access to only authorized maintenance personnel.

- Documented and auditable security procedures.  Personnel who manage the content security system will follow documented security guidelines for access and use of the systems that process plaintext content works or keys to encrypted content works.

- Integrity protection of files and streams.  All content works will be uniquely identified in plaintext format by a strong integrity protection system that is at least as strong as the SHA-256 algorithm.

- Encryption of files and streams.  The system should be capable of handling encrypted versions of any plaintext content work in the form of a file of stream.  An obvious exception to this mandate is the case where content works are not encrypted as a matter of policy.

- Selectable level of security.  The system should be robust enough to handle plaintext or encrypted formats of content works, with replaceable algorithms and key sizes.  The content security system should easily accommodate content protection mechanisms such as watermarking and other types of steganography.

Many of the above mechanisms are used by content protection and digital rights management (DRM) systems with the exceptions of physical security of end-systems and the presence of trained personnel who follow security procedures.  Content protection and DRM systems make greater use of steganography, such as digital watermarking, and tamper-resistant key stores.  Throughout this chapter of the document, both content security and content protection mechanisms are considered

## Content protection mechanisms

| ASSET | RISK | SECURITY MECHANISM |
| --- | --- | --- |
| Plaintext content work | Unauthorized modification | Integrity protection of files and streams |
| Plaintext content work | Unauthorized access | Encryption of files and streams Content policy specification Selectable security level |
| Encrypted content work | Unauthorized access | Strong identity and authentication Physical security of end-systems Documented/auditable security system |
| Content work | Denial of service attack | See network security mechanisms |

In cases where the provider of a content work does not trust the security or practices of the recipient of the work, “content protection” mechanisms raise the bar to illegal or unauthorized uses.  These mechanisms are discussed under separate points below.

- Ease and simplicity of use.  According to the “Darknet” paper, cumbersome and intrusive protection mechanisms encourage circumvention of protection mechanisms and unauthorized access to content works [Darknet].  It is therefore important that Terminal applications make it very easy for works to be accessed by the consumer in the desired ways but very hard to access works in unauthorized ways.  For this to have the desired effect, the types of authorized access need to meet the customer’s needs and expectations.

- Limited exposure of plaintext content works.  Terminal, intermediate, and server systems should be capable of processing encrypted content works to the latest point in the rendering process.  This of course is an unnecessary requirement for unencrypted works, but all systems that handle content works should be designed to send, receive, and store encrypted images of those works.

- Integrity protection.  The system needs some means to distinguish an authorized copy of a digital content work from a modified or derived work.

- Stream-level encryption.  Limited exposure of a content work in the form of a stream entails stream-level encryption in which the work is encrypted at the point of transport and decrypted prior to rendering at the terminal.

- File-level encryption.  Limited exposure of a content-work contained in a file entails file-level encryption in which the content work can be downloaded or streamed while encrypted as done in the MPEG-4 Encryption and Authentication standard [ISMACRYP].

- Content policy specification.  The types of access that the residential customer is authorized to perform needs to be specified.  This may be specified per content work, such as an XrML rights specification that is bound to a particular content work.  Or it can be bound to a class of content works or the entire service, as is done in Apple iTunes™.

- Protected outputs. Terminal systems that process encrypted content works must support protected outputs such as HDCP, DTCP, and other standards that are appropriate to the type of interface.

- Digital watermarking.  Forensic and copy-control watermarks are two types of content-level protection mechanism that are widely used today.  Systems that process a watermarked content work must preserve the watermark up to and including Terminal rendering.

- Protected key store.  All keys need appropriate protection and a scalable level of protection needs to be supported.  Home theatre systems that process newly-released movies, for example, may need keys stored on a secure processor or separate token card that are tamper-resistant.  At the other extreme are content works that are distributed for use on PCs, such is the case for practically all music titles today and a secure co-processor is not generally available.  Thus, high-value content works such as newly released movies are unlikely to be licensed for use on a PC today or in the near future.  In the case of a PC, the keystore should at least be inaccessible using the facilities of the operating system such as a text editor.  In general,  the keys to valuable content works in the earlier phases of their release window will likely require hardware-level protections such as a co-processor that does not expose keys on the bus or system memory.

- Scalable level of protection.  Not all works need to be encrypted or watermarked and not all works that are encrypted need to be encrypted in the future.  The Terminal, intermediate and server systems that process content works should support a variety of policies and mechanisms for content protection including no content-protection mechanisms at all.  Possibly the only mechanism that is universally desirable for digital content works is integrity protection, which is a content-security mechanism discussed above.

## Network security mechanisms

| ASSET | RISK | SECURITY MECHANISM |
| --- | --- | --- |
| Network bandwidth | Denial of service | Firewalled perimeter |
|  | Unauthorized use |  |
| Network messages | Unauthorized access | Firewalled perimeter Access Controls |
|  | Unauthorized modification | VPN Access Controls |
|  | Replay | VPN |
| Routers and intermediate systems | Denial of Service | Firewalled perimeter Access controls Intrusion protection Documented and auditable security procedures |
|  | Unauthorized use |  |
| Network service information | Unauthorized disclosure | Access controls Integrity protection Documented and auditable security procedures |
|  | Unauthorized modification |  |

Network security has a set of widely used and commercially available mechanisms that include firewalls, end-system access control, intrusion detection, secure end-systems, certified intermediate systems, and virtual private networks.  In many cases, there is a trade off between security and performance in an IPTV system.  In many cases, transport security is redundant to security at the application layer.  For example, if encryption and integrity protection are applied to a content work, then there is less of a requirement for securing network messages.  In the core and even in the access network, VPN service is often more service than is needed and a drain on system performance.

The security requirements also differ depending on the type of network.  As shown in the Assets, Risks and Threats tables of the preceding chapters, the severity of the risk to a network asset differs between core, access and home network.  For example, the home network is less of a target for denial of service; it is much more effective instead to target shared resources that affect many users and large web sites rather than a single home network.

The basic security mechanisms are similar between different types of networks.  Devices on a home network may use VPNs (virtual private networks) between home network devices.  And home gateways need access controls as do core routers.  These mechanisms are discussed below.

- Firewalled perimeter.  Access to all networks should be through a packet filtering firewall.  In some cases, an application-layer firewall or gateway may also be needed.  In particular, the perimeter of the core network should be protected by firewalls to restrict access to authorized devices and networks.

- Access controls.  Access controls are needed on network devices, multicast streams, and media files.  Device access controls serve to authorize access both at the console as well as over a network.  For devices, strong passwords are needed at minimum and two-factor authentication, such as by password and token device, is preferred; this is particularly true of security critical devices such as core routers, servers and intermediate systems.  Multicast access controls perform authorization on receiving devices that attempt to join the multicast group and or senders who attempt to send to a multicast group.  File access controls require authorization as a pre-requisite to decrypting or receiving a file.

- VPN (Virtual Private Network).  On some networks, it is desirable to use an encrypted and integrity protected connection between devices or between Terminal application programs.  On IPTV core and access networks, however, the overhead of VPNs is often inconsistent with the performance demands of IPTV applications and unneeded when content-level security is used. On these networks, packet-filtering firewalls are preferred to VPN tunnels between devices.

- Intrusion detection and prevention.  When intrusion by hackers or professionals is considered to be a major threat, intrusion-control mechanism might be used on access networks.  These systems correlate access patterns with different types of attacks and take defensive actions.

- Integrity protection.  Records and other information that are critical to network configuration, management, and operation typically need to be integrity protected on the device as well as on the network.

- Documented and auditable security procedures. Personnel who manage the network system need to follow documented security guidelines for access and use of the systems that process plaintext content works or keys to encrypted content works.

## Terminal protection mechanisms

| ASSET | RISK | SECURITY MECHANISM |
| --- | --- | --- |
| Terminal locality | Unauthorized relocation of fixed-location device | End-system localization Visible Fingerprinting |
|  | Unauthorized sharing of mobile device | Household localization |
| Processor and Disk | Unauthorized access such as by malware or remote access | Signed code Secure Bootloader Secure Keystore Network access controls |
| Terminal interface | Circumvention of content protection controls through a Terminal interface | Approved copy-protected outputs Approved outputs to authenticated devices |

When content works are licensed to a household, it is desirable that the content work be accessible only by members of the household or visitors to the household.  In an extreme and probably rare case, a fixed-location device might be moved outside of a household.  Or the secret identity key that identifies and authenticates the device is copied to a remote device, possibly to circumvent regional controls on broadcast content or simply to share content works beyond the household.  It also might  threaten the provider’s business model if content works are shared across a home network to another home network.  “Localization mechanisms” address these threats by associating a Terminal with a household.  Localization can be done once at the time of Terminal installation by an installer, or it can be done by the consumer, possibly over the telephone.  Often localization can be accomplished by a physical procedure involving a dongle or a 20nrols20th device that requires physical proximity.  Localization also applies to mobile devices.  A localization of a mobile device, for example, might require that the device be brought back into the home periodically where it automatically registers with a proxy or other fixed-location device in the household.  These localization techniques are listed in the table below.

| Method | Localization |
| --- | --- |
| Installation | Employee of IPTV service installs device in subscriber’s home. |
| Telephone | Subscriber 20nrols device over the phone through a provider’s interactive voice response system to identify a device as belonging to the subscriber or subscriber’s household. |
| Dongle | Subscriber uses a USB drive or other dongle device to identify the device as belonging to the subscriber or household. |
| Proxy | A device such as a settop device, network gateway or modem associates multiple devices using some localization method such as a dongle or Bluetooth. |

Localization is of interest to a number of content business models.  It is important to note, however, that the greatest security threats on the Internet today are from malware such as computer viruses and Trojan horses that the user unknowingly installs on their personal computer.  If the Terminal device downloads executable code such as Java, it will be vulnerable to malware attacks, which can propagate throughout the home network and the IPTV network.  The IPTV provider can mitigate this threat by not supporting downloadable executables or by requiring that all Terminal software be signed code in which a hash of the software is signed by the provider and checked by the Terminal before the code is downloaded or used.  This procedure would be implemented by the secure bootloader in which all software and keys are integrity protected and securely downloaded to the device, which checks their integrity and performs the necessary decryption.  These keys will typically be kept in secure storage in which the identity and key decrypting keys are not exposed on external interfaces such as a system bus.  In some cases, even content decryption keys might be so stored.  When there is such variability in the security capabilities of a Terminal device, some means may be needed to communicate the secure key-storage capabilities of the device at the time of bootloading.

In addition to signed code and secure bootloading, network access to the Terminal device needs to be strictly controlled and authorized by the Terminal and the IPTV provider.  A secure connection such as a VPN connection may serve the purpose of authenticating and authorizing remote access to the device.

In addition to controlling inputs to the Terminal such as code and remote commands, rights holders of content works typically require that outputs be protected as well.  An “approved output” is an interface that has appropriate encryption and integrity protection.  These mechanisms are specific to the interface device.  High-bandwidth Digital Content Protection (HDCP) is used for High Definition Multimedia Interface (HDMI) devices.  Digital Transmission Content Protection (HDCP) is used on USB and IEEE 1394 devices, and CPRM is used for recordable DVDs, to name only a few.

## Subscriber security mechanisms

| ASSET | RISK | SECURITY MECHANISM |
| --- | --- | --- |
| Subscriber information | Unauthorized access | Terminal access control Transaction VPN Integrity protection of subscriber and transaction information |
|  | Unauthorized modification |  |
| Transaction information | Unauthorized access |  |
|  | Unauthorized modification |  |

As described in the previous chapter, access to the terminal over the network needs to be strictly controlled to prevent unauthorized remote access to subscriber information.  Depending on the nature of the service, it may be desirable to restrict the subscriber’s information from local access to the Terminal from the console, if there is a console on the terminal device.  In general, it is prudent to use encrypted and integrity-protected connections (such as a VPN) between the Terminal and the service provider’s equipment when information about the subscriber and the subscriber’s transactions is carried over the network.

# Acknowledgements

# Glossary

| AACS | Advanced access content system for DVD and network media, see http://www.aacsla.com/home |
| --- | --- |
| Asset | A data resource or property that is of value to the supplier, distributor, or consumer in the content-work distribution chain. |
| AVC | H.264/AVC codec and payload type |
| CAS | Conditional access service that controls access to a service by excluding non subscribers to the service. |
| CE | “Consumer electronic” or “consumer electronics”. |
| CPRM | Content Protection for Recordable Media standard for writable DVDs. |
| Content protection | Mechanisms that control access to a content work when the end device is not physically secured or under the control of personnel who are trained and managed to obey a set of security procedures. |
| Content security | Mechanisms that control access to a content work when the end-system devices are physically secured and/or managed by trained personnel who are trusted to follow security procedures. |
| Content title | See “content work”. |
| Content work | A video, musical or other work, which is typically copyrighted and distributed as a stream or file. |
| Distribution chain | The sequence of networks and devices that span one or more businesses and terminate in a household that is a subscriber of the last business in the distribution chain. Content works, subscriber information, network data, and other resources are stored and transmitted across the distribution chain. |
| DRM | Digital rights management is a technology for rights transfer between a business and a consumer or consumer household for a copyrighted title. In general, any system that protects content independently of its transmission or storage format is termed “DRM”. This paper uses a narrow definition of rights transferral between two entities, the provider and the consumer/customer. |
| DTLA | Digital Transmission License Authority, see http://www.dtcp.com/ |
| DTCP | Digital Transmission Content Protection specification of the DTLA |
| DVD CCA | DVD Copy Control Association, see http://www.dvdcca.org/ |
| File encryption | An MPEG-4, Quicktime or other media file containing ciphertext media and plaintext metadata. Files that are enveloped using Cryptographic Message Syntax (CMS) may be encrypted as well as integrity-protected using CMS, but CMS does not leave certain parts of the file in plaintext, such as plaintext metadata. |
| HDCP | High-bandwidth Digital Content Protection standard for link-level authentication and encryption over HDMI. |
| HDMI | High Definition Multimedia Interface for uncompressed audio/video streams including standard, enhanced, and high-definition video. |
| Household | A set of devices associated with a subscriber and authorized to play certain titles depending on the business model and DRM of the title provider. |
| Identity key | An RSA or some other key that allows an entity to prove that it is in the possession of a secret, such as the private key belonging to an RSA public/private keypair. |
| Indemnification | A fine or fee that is incurred when a device vendor fails to comply with its agreement with a License Authority. |
| Internet content provider | An aggregator or other content distribution business that does not necessarily own the network or access network to their customer homes nor control the their customers’ end-systems. |
| IPTV | TV services over Internet Protocol include the distribution of content works at any point in the distribution chain regardless of whether every point in the distribution chain uses IP transport. |
| Key ladder | A set of related keys with a “master key” to access a stream or file, a “key-encrypting key” to access a master key, and “identity keys” used to encrypt, sign, or decrypt the key-encrypting key |
| KMS | Key Management Service for establishing a key ladder in a device. |
| License authority | A body that licenses devices that comply with a particular specification or set of specifications. |
| Licensed content | See “Licensed Data” |
| Licensed data | Data from a copyrighted work that are restricted by prevailing law to reside or be rendered on a licensed device. |
| Licensed device | A device that compliance with the specifications or requirements of a particular license authority. |
| LMI LLC | Licensed Management International, LLC is a professional device licensing business that administers the licensing associated with a particular content protection standard such as CPRM. |
| Robustness | A property of a device that makes it difficult to circumvent its access controls or to gain access to device secrets, such as its identity key. A device that can be revoked or that can be renewed with a new secret is also said to be “robust”. |
| RTCP | The control protocol for RTP. |
| RTP | Real-time Transport Protocol. |
| RTSP | Real-Time Streaming Protocol. |
| SDP | Session Description Protocol Library. |
| SRTP | Secure Real-time Transport Protocol |
| Stream | A sequence of packets carrying video or other continuous-time media that is typically rendered upon reception or else stored using a media ingestion system. |
| Title | See “Content work”. |
| Value chain | See “Distribution chain”. |

# References

[Content Guard] Content Guard Licensing, http://www.contentguard.com/patents.asp

[DVBCPCM]  DVB Copy Protection and Copy Management, http://www.dvb.org/technology/dvb-cpcm/.

[Darknet] P. Biddle et. Al., Darknet and the Future of Content Distribution, http://msl1.mit.edu/ESD10/docs/darknet5.pdf, 2002.

[DVBCA] ETSI ETR 289 ed.1 (1996-10): “Digital Video Broadcasting (DVB);Support for use of scrambling and Conditional Access (CA) within digital broadcasting systems”

[ISMACRYP] ISMA 1.1 Encryption and Authentication, Internet Streaming Media Alliance, 2006

[Lieberfarb] W. Liebarfarb, private communication, 2005.

[MPEG-LA] MPEG License Authority Programs in Development, DRM, http://www.mpegla.com/pid/drm/

[MVV] A.J. Mendes, P.C. vonOorschot, S.A. Vanstone, Handbook of Applied Cryptography CRC Press LLC, 1997.

[OMADRM2] Open Mobile Alliance DRM Version 2, http://www.openmobilealliance.org/release_program/drm_v2_0.html

[WAX] R.J. Anderson, et. Al., Secure Books: Protecting the Distribution of Knowledge, http://www.cl.cam.ac.uk/~rja14/Papers/wax-securebook.pdf

[Marks & Turnbull] D.S. Marks, B.H. Turnbull, Technical protection measures:  The intersection of technology, law, and commercial licenses, Workshop on Implementation Issues of the WIPO Copyright Treaty (WCT) and the WIPO Performances and Phonograms Treaty (WPPT), World Intellectual Property Organization, Geneva, December 6 and 7, 1999 (http://www.wipo.org/eng/meetings/1999/wct_wppt/pdf/imp99_3.pdf).

___________