| INTERNATIONAL TELECOMMUNICATION UNION |  |  |  | Focus Group On IPTV |
| --- | --- | --- | --- | --- |
| TELECOMMUNICATION STANDARDIZATION SECTOR STUDY PERIOD 2005-2008 |  |  | FG IPTV-C-0240 |  |
|  |  |  | English only |  |
| WG(s): 1 |  | 2nd FG IPTV meeting: Busan, October 16-20 2006 |  |  |
| CONTRIBUTION |  |  |  |  |
| Source: | Telekomunikacja Polska; Research and Development Department |  |  |  |
| Title: | Proposal of concept IMS based modular IPTV architecture |  |  |  |

- Main requirements for the architecture:

- A lot of different terminals (STB, PC, mobile phone, palmtop, etc.)

- Many different interactive services. Very important is easy integration of new services  even services that not exist now

- Standard protocols

- Open architecture for many different developers

- Communication between platform and customer’s terminals:

There are on the customers terminals every data, which is responsible for layout . Customer   should have a few different templates of layout. When it is necessary this data is updated from the platform.

SIP – control protocol. Two approaches: extending SIP protocols or encapsulating RTSP into SIP

SCP – updating templates of layout and dedicated terminals firmware

XML – protocol uses for transfer meta data between platform and STB

SNMP ver. 3 – management protocol

DHCP – protocol in the access network to give for terminals IP address

- Communication between functional modules of the platform.

SIP – control protocols

SNMP ver. 3 – management protocol

SSH – protocols used to some administration procedures

- Content protection.

For ensure content protection in the IPTV should be implemented DRM system based on public key architecture.

- Architecture of the IPTV

IPTV architecture should be modular, flexible and scalable. It is necessary to easy integration of new services.

Access Network

DSL

Ethernet

Mobile network

Content Delivery Network

Video servers

Game servers

Other content servers

Core Network

Terminals

Terminals

Terminals

IMS based platform with SIP proxy

Live TV Head-end

Local Live TV Head-end

SIP

SIP

SIP

SIP

Network and maintenance servers

Central content dystrybution servers

DRM based content protection system

- IMS based platform.

CSCF

HSS

Biling system based on Parlay OSA

Telephony server and telephony gateways

Application non telephony servers

Media servers

Media gateway

- Content Delivery Network

CDN should consist:

  - master content servers for each services

  - local video servers with redundant server

  - local game servers with redundant server

  - possibility of integration now other content servers in the future

- Argumentation

IPTV IMS based architecture ensures:

  - SIP protocol dedicated to multimedia

  - Modular architecture

  - One central databases shared between many different services

  - Easy integration with old telecommunication networks and exists services

  - Central billing system based on Parley OSA solution.

  - Third-party services

_____________