| INTERNATIONAL TELECOMMUNICATION UNION |  | Focus Group On IPTV |
| --- | --- | --- |
| TELECOMMUNICATION STANDARDIZATION SECTOR STUDY PERIOD 2005-2008 |  | FG IPTV-C-0303 |
|  |  | Original: English |
| WG(s): 6 |  | 3rd FG IPTV meeting: Mountain View, USA, 22-26 January 2007 |
| CONTRIBUTION |  |  |
| Source: | China Netcom Group, Peking University, Tsinghua University |  |
| Title: | Reply OLS to the FG-IPTV on “Second FG IPTV meeting activities and achievements to AVS” for IPTV Advanced Coding of Audio and Video Part 2: Video standard |  |

# Introduction

Based on the reviewed contributed document FG IPTV-IL-0008 in the second of FG IPTV meeting by FG IPTV-R-0013, WG6 AV subgroup requests Advanced Coding of Audio and Video Part 2: Video standard. This document will outline the standard structure (GB/T200090.2, China) briefly.

AVS Workgroup is an A4 organization of ITU-T. China Netcom Group, Peking University, Tsinghua University are on behalf of AVS Workgroup.

# Content of AVS part 2: Video standard document

For the AVS P2 standard content, please check Annex 1.

# Standard document key content instruction

## 3.1 Purpose

This standard is established in response to the growing need for better compression of moving pictures for various applications such as digital television broadcasting, digital storage media, Internet streaming, and communication.

## 3.2 Applications

This standard is designed to cover a broad range of applications including but not limited to the following:

CATV		Cable TV on optical networks, copper, etc.

DBS		Direct broadcast satellite video services

DSL 		Digital subscriber line video services

DTTB		Digital terrestrial television broadcasting

ISM		Interactive storage media (optical disks, etc.)

MMM		Multimedia mailing

MSPN		Multimedia services over packet networks

RTC		Real-time conversational services (videoconferencing, videophone, etc.)

RVS		Remote video surveillance

## 3.3 Profiles and Levels

This Standard can serve a wide range of bit rates, resolutions and qualities. Considering the practicality of implementing the full syntax of this Specification, however, a limited number of subsets of the syntax are also stipulated by means of “profiles” and “levels”.

A “profile” is a subset of the entire bitstream syntax that is specified by this Standard.

A “level” is a specified set of constraints imposed on values of the syntax elements in the bitstream, which is specified within each profile.

## 3.4 Overview of the Design Characteristics

In this Standard, a number of techniques may be used to achieve highly efficient compression, including inter prediction, intra prediction, transform, quantisation, entropy coding, etc. Inter coding uses motion vectors for block-based inter prediction to exploit temporal statistical dependencies between different pictures. Intra coding uses various spatial prediction modes to exploit spatial statistical dependencies in the source signal for a single picture. The prediction residual is then further compressed using a transform to remove spatial correlation inside the transform block before it is quantised, producing an irreversible process that typically discards less important visual information while forming a close approximation to the source samples. Finally, the motion vectors or intra prediction modes are multiplexed with the quantised transform coefficient information and encoded using entropy coding.

## 3.5 Predictive Coding

Intra coding needs not refer to other pictures; Intra coded pictures can provide random access points to the coded sequence.

Inter coding need to refer to prior coded pictures, and the order of the decoding process can be different from the order of the source picture capture process in the encoder or the output order from the decoder for display. In inter coding, motion vector precision can be up to ¼ pixel, and motion vectors are coded by predictive coding.

## 3.6 Picture Partitioning into Macroblocks and Smaller Partitions

In this Standard, the basic processing unit of the video decoding process is a macroblock, which consists of a 16x16 block of luma samples and corresponding blocks of chroma samples. A macroblock can be further partitioned into sub-macroblocks for prediction. The minimum size of a sub-macroblock is 8x8.

## 3.7 Transform and Quantisation

The transform block-size is 8x8. Transform coefficients are scalar quantised.

## 3.8 How to Read This Specification

It is suggested that the reader starts with clause 1 (Scope) and jumps to clause 3 (Definitions). Clause 6 (Structure of Coded Bitstream) should be read for the definition of coding bitstream construction. Clause 7 (Bitstream Syntax and Semantics) specifies syntax and semantics of bitstream. Subclause 7.1 (Syntax Description) describes the syntax, which defines the order of appearance of syntax elements in the bitstream. Subclause 7.2 (Semantics Description) describes the semantics, which specifies the scope, restrictions, and conditions that are imposed on the syntax elements. The actual parsing for most syntax elements is specified in clause 8 (Parsing). Finally, clause 9 (Decoding) specifies how the syntax elements are mapped into decoded samples. Throughout reading this specification, the reader can refer to clauses 2 (Normative), 4 (Abbreviations), 5 (Conventions) and the annex as needed.

## 3.9 AVS Patent Pool

To be done

# AVS standard IPR and licensing policy

AVS video standard is approved by China government, and AVS group members hold the IPR. For any non-member users who want to get the official copy, they need AVS group’s license permission.  Otherwise, AVS group haven’t obligation to keep it update and guarantee its correctness.

## 4.1   Explanation for the correlative patents

The issuer of this section draws attention to the fact that it is claimed that compliance with this document may involve the use of patents. We take no position concerning the evidence, validity and scope of this patent right. The holder of this patent right has assured that he is willing to negotiate licences under reasonable and non-discriminatory terms and conditions with applicants throughout the world. In this respect, the statement of the holder of this patent right is registered with the releasing organization.

The patent permission involves in this part, follows the《Intellectual Property Policy》 which was constituted by the draft organizer AVS Standard Workgroup of China and signed by the committee, Permissions should be carried on through the AVS patent and concrete permission provisions are decided by the patent management committee. Standard carrier could obtain any permission of the patent from patent management committee or it's authorized organizations.

Patent management committee is sponsored by the"AVS Standard Workgroup of China", and you may contact the secretariat of "AVS Standard Workgroup of China" for related matters you concerned.

Contact man	: Tiejun Huang (Secretary-general of AVS Standard Workgroup of China)

Mail			: 2704 box 31 sub-box, Beijing

Zip code		: 100080

E-mail		: tjhuang@ict.ac.cn

Tel		: +86 10 58858303，+86 10 58858300-303

Fax			: +86 10 58858301

Website		: www.avs.org.cn

# Proposal

Consideration of AVS P2 as one of video standard options for IPTV application.

# ANNEX 1 Content of AVS part 2: Video standard document

EMBED Photoshop.Image.7 \s

_______________