| Introduction INTERNATIONAL TELECOMMUNICATION UNION |  |  |  | Focus Group On IPTV |
| --- | --- | --- | --- | --- |
| TELECOMMUNICATION STANDARDIZATION SECTOR STUDY PERIOD 2005-2008 |  |  | FG IPTV-C-0333 |  |
|  |  |  | English only |  |
| WG(s): 1 |  | 3rd FG IPTV meeting: Mountain View, USA, 22-26 January 2007 |  |  |
| CONTRIBUTION |  |  |  |  |
| Source: | CATR/MII, China Telecom |  |  |  |
| Title: | Proposed Interpretation to Figure 3 in the FG IPTV-ID-0261 |  |  |  |

In Busan Meeting, the initial draft of the output document on IPTV Architecture (FG IPTV-D-0056) was produced by WG1. During the electronic meeting the document FG IPTV-D-0056 was discussed and revised and then submitted to this meeting as FG IPTV-ID-0261. In the sub-section 8.1.1 of FG IPTV-ID-0261, a high level architecture was illustrated in Figure 3, while it is inconvenient to understand this diagram since the absence of descriptive text to it. In this contribution some detailed descriptions to figure 3 are proposed.

# Proposed Descriptions of IPTV Functional Architecture

Each function group can be described as following,

- Content Provision: IPTV contents, including the VoD programs, TV channel streams, as long as their metadata, are prepared by the Content Provision functions through some adapting processes before or during the service offering. First the contents are ingested from content sources into the Content Provision, then converted into the designated format when necessary, and encrypted for right management by the Content Provision Functions. Meanwhile, some simple editing work upon the contents can be done within the Content Provision if it is permitted, e.g. Ad inserting, subtitle or logo superposing etc. After the adapting process aforementioned, then contents are transferred to the Content Delivery functions.

The Content Provision functions also act as the root authority of the contents’ right management to empower the IPTV Control functions to issue the right description and keys to the end users before content is to be consumed.

- IPTV Control:  IPTV Control functions are responsible to IPTV service preparation control as well as service offering control. During the service preparation process, IPTV Control may request contents from Content Provision, and pack the contents into services, generate content distribution policy, and then publish these deliverable services through navigating / EPG components. In the service offering process, IPTV Control will delivery the service information (e.g. in the form of EPG) to IPTV users, therefore the users can select specific service/program. With the empowerment of Content Provision, IPTV Control can also responsible to issue the content licence to the users according to their subscription. In order to support the service control functions, at least three kinds of information base should be maintained by IPTV Control functions, that is content profile, service profile and user profile.

- Content Delivery: Contents which are packed in IPTV services are delivered to End System by the Content Delivery functions. The content data is injected to the Content Delivery functions from Content Provision before or during the service offering process. In order to support the services like VoD and PVR, and for the sake of transport efficiency, contents shall be stored /cached within the Content Delivery. When the end user requests for a piece of content, it will be directed by the IPTV Control functions to the Content Delivery functions to obtain content data. Content Delivery functions also provide capability to facilitate interaction between the users and programs such as playback control. Physically, Content Delivery sub-system employs the resources of IP bear networks for the content conveying.

- End System: Functionally End System is composed of a set of service execution functions of IPTV system in custom premise. End System is responsible to collect control command from user, and interact with IPTV Control functions to get service information (e.g. EPG), content license and key for decryption. End System also provides the capability for content acquisition, content decrypting and decoding.

- IPTV System Management & Security: System Management & Security functions is in charge of system overall status monitoring, configuration, and security aspects. This set of functions may be deployed in a centralized manner, or otherwise in a distributed manner.

# Proposal

We propose WG1 to add the description about each function group in section 2 into the working document on IPTV Architecture behind current Figure 3.