| Abstract INTERNATIONAL TELECOMMUNICATION UNION |  |  |  | Focus Group On IPTV |
| --- | --- | --- | --- | --- |
| TELECOMMUNICATION STANDARDIZATION SECTOR STUDY PERIOD 2005-2008 |  |  | FG IPTV-C-0343 |  |
|  |  |  | English only |  |
| WG(s): 1 |  | 3rd FG IPTV meeting: Mountain View, USA, 22-26 January 2007 |  |  |
| CONTRIBUTION |  |  |  |  |
| Source: | NTT Corporation |  |  |  |
| Title: | Requirements for optical access |  |  |  |

Though the current working document “IPTV Service Requirements” mentions optical access, it does not list either BPON or GPON, which are being idenitified in SG13.  This contribution proposes adding BPON and GPON access to the working document.

1. Introduction

The latest working document of “IPTV Service Requirement” is FG IPTV-C-0260. This mentions optical access in item IPTV_NET_084 in section 5.4.6 Home Network. However, the text describes only IEEE 802.3 standard specifications used for EPON access. This contribution proposes adding G.983 for BPON and G.984 for GPON to the text. It also proposes moving the text to section 5.4.1.2 Access Network to improve its readability.

2. Discussion

2.1 Readability

The terms “IPI-4”, “IPI-4a”, “IPI-4b”, “IPI-4c”, and “IPI-4d” are used without definitions for delivery network access interface in section 5.4.6 Home Network.  More explanatory terms such as “delivery network access interface”, “metallic delivery network access interface”, “optical delivery network access interface”, “coaxial delivery network access interface”, and “wireless delivery network access interface” are needed to improve the document readability.

Though some of the requirements of delivery network access could be discussed from home network viewpoint, most of the requirements should be considered from the access network viewpoint. Therefore, some of the requirements should be moved from 5.4.6 Home Network to 5.4.1.2 Access Network.

2.2 Requirements for optical access

The current working document mentions only IEEE 802.3 standard specifications for optical access. However, ITU-T SG13 identifies G.983 for BPON and G.984 for GPON as standard specifications for optical access. Therefore, these two specifications should be included in the text.

3. Proposal

Based on  the above discussion, this contribution proposes moving some of the requirements in section 5.4.6 Home Network to section 5.4.1.2 Access Network. It also proposes deleting the current IPTV_NET_084 and including the following text in section 5.4.1.2 Access Network.

“The optical access interface, if provided, shall comply with G.983, G.984, or the appropriate IEEE 802.3 standard specifications. [IIF.ARCH.HOME.11]”

___________