| INTERNATIONAL TELECOMMUNICATION UNION |  |  |  | Focus Group On IPTV |
| --- | --- | --- | --- | --- |
| TELECOMMUNICATION STANDARDIZATION SECTOR STUDY PERIOD 2005-2008 |  |  | FG IPTV-C-0351 |  |
|  |  |  | English only |  |
| WG(s): 6 |  | 3rd FG IPTV meeting: Mountain View, USA, 22-26 January 2007 |  |  |
| CONTRIBUTION |  |  |  |  |
| Source: | NTT Corporation |  |  |  |
| Title: | IPTV Middleware Application requirements |  |  |  |

This contribution describes IPTV middleware function (hereinafter referred to as IMF) requirements on interfaces that would enable IPTV services. We propose a set of  requirements for IPTV application/middleware interfaces. These requirements on APIs concern both those used by browsers as well as by “resident” middleware and applications.

- Content related requirement

  - IMF should support API for playing IPTV content, such as VOD, IP broadcasting etc.

  - IMF should support API for playing the audio resource on the IPTV terminal.

  - IMF should support API for locking screen, which disables the displayed image to change for a certain amount of time until, e.g., all the resources needed to be shown are ready and can be synchronized.

  - MF should support API for starting IP channel service.

- IPTV Communication

- IMF should support API for transmitting data over TCP/IP.

- IMF should support API for obtaining content by HTTP/POST.

- IMF should support API for getting information from non-volatile storage if it is present.

- IMF should support API for querying metadata.

- Security

- IMF should support API for getting security information such as DRM information, CAS card ID, etc.

- IMF should support API for identifying the client device.

- IMF should support API for getting registration information that has to be set on the terminal device.

- IMF should support API for storing the user identification information in the terminal device.

- IMF should support API for storing subscription licence in the terminal device.

- IMF should support API for obtaining parental control related information from the terminal, if necessary.

- IMF should support API for storing the licence for content the user purchased in the terminal device

- IMF should support API for updating the purchased content licences in the terminal device.

- IMF should support API for storing content purchasing information in the terminal device.

______________