| With reference to document 260 we would like to suggest the following changes to the Working Document: INTERNATIONAL TELECOMMUNICATION UNION |  |  |  | Focus Group On IPTV |
| --- | --- | --- | --- | --- |
| TELECOMMUNICATION STANDARDIZATION SECTOR STUDY PERIOD 2005-2008 |  |  | FG IPTV-C-0366 |  |
|  |  |  | English only |  |
| WG(s): 1 and 6 |  | 3rd FG IPTV meeting: Mountain View, USA, 22-26 January 2007 |  |  |
| CONTRIBUTION |  |  |  |  |
| Source: | RNIB (Royal National Institute of the Blind, UK) |  |  |  |
| Title: | Suggested changes to IPTV Services Requirements: metadata and home networking |  |  |  |

## Using an EPG to make recordings

When browsing a list of programmes on an EPG, the user should be able to instruct the terminal to record the highlighted programme. The actual PVR that is set to record may be a central one on a home network.

In section 5.6.2 Enhanced EPG, Channel and Menu Processing INSERT:

The user should be able to select the highlighted programme on an EPG to be recorded on a PVR.

In section 5.4.6. Home Network INSERT:

If using a home network, it should be possible to instruct a central PVR to record a programme from a remote terminal, eg by selecting it in an EPG.

## Filtering EPG listings according to accessibility features required

It is a considerable benefit to some users who are constrained by the provision of accessibility features to be able to view only the programmes that are accessible to them. For example, perhaps a deaf person can only watch programmes that are provided with suitable captions.

In section 5.6.2 Enhanced EPG, Channel and Menu Processing INSERT:

The user should be able to filter or prioritise the EPG display by accessibility feature available for each programme.

In section 5.6.4 Metadata

The metadata shall indicate which accessibility features are available for a TV programme, ie captions, subtitles in various languages, supplementary video and descriptive audio.

____________