| INTERNATIONAL TELECOMMUNICATION UNION |  |  |  | Focus Group On IPTV |
| --- | --- | --- | --- | --- |
| TELECOMMUNICATION STANDARDIZATION SECTOR STUDY PERIOD 2005-2008 |  |  | FG IPTV-C-0367 |  |
|  |  |  | English only |  |
| WG(s): 1, 6 |  | 3rd FG IPTV meeting: Mountain View, USA, 22-26 January 2007 |  |  |
| CONTRIBUTION |  |  |  |  |
| Source: | Samsung Electronics |  |  |  |
| Title: | EPG Metadata Architecture for Fast EPG Launching Time Support |  |  |  |

By the definition of IPTV, any IP-enabled terminal can access the services provided by IPTV service providers regardless of its underlying access technology.

Depending on the characteristics of the underlying access technology, the available bandwidth to IPTV terminals varies from several Mbps (typical to xDSL) to several hundred kbps (typical to wireless link). The wide dynamic range of the available bandwidth raises a multitude of challenges to the development of IPTV specifications.

One of the most important available bandwidth related parameters that have direct effects on user experiences is the launching time of IPTV terminals. The launching time can be defined as the time required from the power-on operation to the final EPG display that allows a user to choose a program.

Displaying EPG to a user requires a whole set of metadata that represents all the services provided by a service provider. The size of a whole set of metadata can be large up to several tens of Mbytes depending on the number of programs included and detailed information contained. In order to reduce the launching time, a lot of techniques can be used: caching may be the most preferred one, which allows the download of incremental changes from the cached one. When no cached metadata is available, however, thus the whole set of metadata has to be downloaded in full, the initial launching will be unbearably long. This can be happen when IPTV terminals boot for the first time, when the cached metadata is corrupted, when a new service provider is tried, when a major change in programs happens, etc.

To mitigate the long initial launching time issues, the EPG metadata shall be structured in a proper way to support fast launching regardless of the size of full EPG metadata. The proposed method is dividing the metadata into two parts (or possibly many parts). There is a portion of metadata which can be called basic metadata table. The basic metadata table contains very basic and essential data required to display EPG for user selection. The basic metadata table may contains the title, start time, brief description, purchasing method of the programs available. The remaining portion of metadata consists of all the details of the programs available.

By dividing the EPG metadata into two parts—basic metadata table and the rest—and delivering the basic part first, the launching time can be very short and constant regardless of the full size of EPG metadata.

Definition:

1. EPG Launching Time

The time required from power-on operation to EPG display for user program selection

Requirements:

- The EPG metadata shall be structured to maintain the EPG launching time as small and constant as possible.

- The EPG metadata shall be delivered in a way that maintains the EPG launching time as small and constant as possible.

___________