| INTERNATIONAL TELECOMMUNICATION UNION |  |  |  | Focus Group On IPTV |
| --- | --- | --- | --- | --- |
| TELECOMMUNICATION STANDARDIZATION SECTOR STUDY PERIOD 2005-2008 |  |  | FG IPTV-C-0368 |  |
|  |  |  | English only |  |
| WG(s): 3 |  | 3rd FG IPTV meeting: Mountain View, USA, 22-26 January 2006 |  |  |
| CONTRIBUTION |  |  |  |  |
| Source: | China Telecom |  |  |  |
| Title: | A proposal for IPTV authentication framework under HN environment |  |  |  |

- Abstract

In response to output document “IPTV-DOC-0044” about IPTV security aspects, this proposal focused on authentication framework and authentication relationships of IPTV service under HN environment.

- 1. Abbreviations

HN                                      Home Network

HG                                      Home Gateway

CCD                                   Content Consumption Device

HN HG-ACF                      HN Home Gateway Access Control Function

IPTV S-ACF                      IPTV Service Access Control Function

IPTV PAM-ACF                IPTV Presence Availability Management Access Control Function

IPTV CC-ACF                   IPTV Conditional Content Access Control Function

IPTV CPCM-F                   IPTV Content Protection and Copy Management Function

- 2. IPTV General Authentication Framework

The whole authentication requirement could be generalized to a general framework. We suggested a general framework is shown as Figure 1 below.

Figure 1 shows, in vertical dimension, authentication requirements derive from three different layer of network control, service control and Content control, in horizontal dimension, entities that authentication needed are categorized  to Devices, Subscribers and end users.

Network Control

Layer

Service Control

Layer

Content Control

Layer

Device Oriented

Subscriber Oriented

User Oriented

IPTV Device

STB and/or CCD

IPTV

Subscriber

IPTV

End Users

HG Device

HN Service

Subscriber

Figure 1   Layered Authentication Framework Model under HN environment

- 2. Illustration of IPTV Authentication relationships on HN environment.

IPTV

End user

Identity

HN

HG-ACF

IPTV CC-ACF

IPTV PAM-ACF

IPTV S-ACF

IPTV CPCM-F

IPTV

Subscriber

Identity

IPTV

STB Device

Identity

STB

CCD

HG

HN related

Identities

ACFs

DLNA

Identity

IGRS

Identity

Figure 2   Illustration of authentication relationships of IPTV in HN environment

As shows in Figure 2 above, in purpose to support flexible device nomadicity and user mobility, many authentication relationships of IPTV may be existed in HN environment. Those relationships include, but are not limited to following items:

(1) Authentication relationship between ‘STB Device and/or Subscriber’ and ‘HN-ACF’;

(2) Authentication relationship between ‘STB Device and/or Subscriber’ and ‘S-ACF’;

(3) Authentication relationship between ‘end user’ and ‘PAM ACF’;

(4) Authentication relationship between ‘end user’ and IPTV ‘CC-ACF’;

(5) Authentication relationship between ‘CCD and/or STB Device’ and ‘CPCM-F’.

- 3. Proposal

In this contribution, suggested authentication framework could complement section 7.2 of document FG-IPTV-DOC-044, and suggested authentication could complement section 7.1 of document FG-IPTV-DOC-044.

At same time, we also think the sequence of section 7.1 and section 7.2 should be swapped.

___________