| Introduction INTERNATIONAL TELECOMMUNICATION UNION |  |  |  | Focus Group On IPTV |
| --- | --- | --- | --- | --- |
| TELECOMMUNICATION STANDARDIZATION SECTOR STUDY PERIOD 2005-2008 |  |  | FG IPTV-C-0389 |  |
|  |  |  | English only |  |
| WG(s): 4 |  | 3rd FG IPTV meeting: Mountain View, USA, 22-26 January 2007 |  |  |
| CONTRIBUTION |  |  |  |  |
| Source: | ETRI, Republic of Korea |  |  |  |
| Title: | Requirements on QoS/QoE parameter adjustment capabilities for Linear TV service |  |  |  |

The IPTV network should support both High Definition (HD) and Standard Definition (HD) profiles for Linear TV service.   And IPTV service operators expect to achieve higher level of user satisfaction with HD contents.  But in many countries including Korea, HD migration is on going and HD programs are not common yet.  So, HD content alternates with SD content even on a HD channel.

# Problem Statements

On the assumption that service signalling will specify only channel, not individual content on a channel, Linear TV service have following characteristics.

- Transition between SD and HD may cause QoS/QoE parameter adjustment.

- The adjustment will happen in already established sessions.

- The adjustment will influence every session for the channel, at the same time.

- The effect of transition will influence all network elements along the path from the content source to the user terminal, and last a few minutes or hours.

- The network is able to estimate the time of adjustment with the broadcast schedule.

We can consider 3 scenarios.

  - Static Provisioning – each channel can be configured to use specific QoS/QoE parameters (e.g. SD only channels and HD only channels).   In this scenario, inefficient bandwidth usage may occur.  Especially it is more critical in Mobile IPTV.

  - Network-only dynamic adjustment –The Network controls its resources internally with the known schedule.  But the home network and the user terminal may not be reconfigured.

  - Fully dynamic adjustment – when the adjustment is required, the network controls its resources and signals to the related user terminals.  If the user terminals initiate whole adjustment process, the concurrent adjustment requests may overload the network.

# Proposal

We propose that the following requirements for “IPTV Network Control Aspects”.

- The IPTV network should provide mechanisms to support QoS/QoE parameter adjustment due to changes of content characteristics on a channel.

- The stream session control protocol should support QoS/QoE adjustment procedure due to changes of content characteristics on a channel.

# ____________