| Abstract INTERNATIONAL TELECOMMUNICATION UNION |  |  |  | Focus Group On IPTV |
| --- | --- | --- | --- | --- |
| TELECOMMUNICATION STANDARDIZATION SECTOR STUDY PERIOD 2005-2008 |  |  | FG IPTV-C-0404 |  |
|  |  |  | English only |  |
| WG(s): 1 |  | 3rd FG IPTV meeting: Mountain View, USA, 22-26 January 2007 |  |  |
| CONTRIBUTION |  |  |  |  |
| Source: | NEC Corporation |  |  |  |
| Title: | Clarification of “Streaming for Broadcast & VoD” on IMS-based IPTV architecture |  |  |  |

This contribution proposes to clarify the location of “Streaming for Broadcast & VoD” on IMS-based IPTV architecture.

1. Introduction

In FG IPTV-C-0261[1], three IPTV architectures are identified, ie. (1) “NGN-based” and “IMS-based” architecture, (2) “NGN-based” and “non-IMS-based” architecture and (3) “Non-NGN-based” architecture. This Contribution focuses on the (3) “Non-NGN-based” architecture especially. The FG IPTV-C-0261[1] presents two architectures (and one example) for IMS-based architecture for IPTV, but there is some part which requires more consistency.

2. Discussion

It is an inconsistent part that is a function of “Streaming for Broadcast & VoD”. In Figure 9 of FG IPTV-C-0261[1], ”Streaming for Broadcast & VoD” is located in NGN Service Oriented Sub-system. However, in Figure 10 of FG IPTV-C-0261[1], “Streaming for Broadcast & VoD” is not shown explicitly, and it can be read that “Streaming for Broadcast & VoD” is expressed in separation of MRFC and MRP-FE, where the MRFC is located in Core-IMS  and the MRP-FE is located in IP Transport in Figure 10 of FG IPTV-C-0261[1]. However two figures are referenced for the same purpose, it could lead some confusion that these two figures include an inconsistency.

As a reference, in section 5 of Draft ETSI TS 182 027[2], IMS-based IPTV high level architecture is shown in Figure 1. In this figure, it can be read that “IPTV Media Functions” is equivalent to “Streaming for Broadcast & VoD” in Figure 9 of FG IPTV-C-0261[1]. However the Figure 1 is the architecture that enables Service Provider can provide the VoD service because the “IPTV Media Functions” are provided by either NGN operator or Service Provider, while MRFC and MRP-FE are provided only by the NGN operator.

3. Proposal

It could be one of possible reasons for the inconsistency that there is no consideration of differences in service models such as Broadcast service and VoD service in IMS-based architecture on Figure 10 of FG IPTV-C-0261[1]. For example, the architecture in Figure 10 of FG IPTV-C-0261[1] could not be applicable to such service model where Service Provider (non NGN operator) provides VoD. Therefore, it is proposed to discuss architecture in each of Broadcast service and VoD service, and to change Figure10 of FG IPTV-C-0261[1] to be consistent.

Figure 1 High Level Architecture from TISPAN

References

[1] FG IPTV-C-0261[1]   Working Document: IPTV Architecture

[2] Draft ETSI TS 182 027 v0.0.4(2006-12) IPTV Architecture; IPTV functions supported by the IMS subsystem

_____________