| INTERNATIONAL TELECOMMUNICATION UNION |  |  |  | Focus Group On IPTV |
| --- | --- | --- | --- | --- |
| TELECOMMUNICATION STANDARDIZATION SECTOR STUDY PERIOD 2005-2008 |  |  | FG IPTV-C-0412 |  |
|  |  |  | English only |  |
| WG(s): 2 |  | 3rd FG IPTV meeting: Mountain View, USA, 22-26 January 2007 |  |  |
| CONTRIBUTION |  |  |  |  |
| Source: | Telchemy Incorporated |  |  |  |
| Title: | Packet Loss Distribution |  |  |  |

The distribution of packet loss has a material impact on the quality of a decoded IP video stream.  It is important to note that packet loss occurs in three forms:-

- Sparse bursts

Sparse bursts (described in [1] and [2]) are periods of high packet loss, analogous to severely errored seconds.  These may be modelled using multistate Markov Models or Gilbert-Elliott models.  A sparse burst is a period that begins and ends with a lost (or discarded) packet during which some constraint is satisfied.  In [2] the defined constraint is that within a burst there must be less than Gmin consecutively received packets.  Gmin is selected such that the minimum effective loss rate within a burst corresponds to the lowest packet loss rate at which some noticeable distortion occurs within the decoded media stream.  Sparse burst are often due to network congestion, RED and related effects.

- Continuous bursts

Continuous bursts are periods during which every packet is lost.  Contiguous losses may occur due to packetization (i.e. packing several transport packets within an IP packet), to link failures within an IP network or other phenomena.

- Isolated losses

Isolated lost packets typically occur due to bit errors in transmission or excessive collisions on local area networks.

Sparse bursts are extremely common in IP networks, and certain approaches to reporting packet loss metrics do not represent these.  In practical measurements made on IP traces, bursts tended to be 1-3 seconds in length with loss densities of 20-30%.

It is proposed that the IPTV FG incorporate a sparse burst model into any set of transport metrics, and consider using the model described in [2] for this purpose.

References

[1] ITU-T COM12 D.97 Packet Loss Distributions and Packet Loss Models

[2] IETF RFC3611 RTCP Extended Reports (VoIP Metrics block)

_________