| 1. Introduction INTERNATIONAL TELECOMMUNICATION UNION |  |  |  | Focus Group On IPTV |
| --- | --- | --- | --- | --- |
| TELECOMMUNICATION STANDARDIZATION SECTOR STUDY PERIOD 2005-2008 |  |  | FG IPTV-C-0416 |  |
|  |  |  | English only |  |
| WG(s): 1 |  | 3rd FG IPTV meeting: Mountain View, USA, 22–26 January 2007 |  |  |
| CONTRIBUTION |  |  |  |  |
| Source: | Korea (Republic of) |  |  |  |
| Title: | Requirements for interworking between IPTV and Cable/Terrestrial TV |  |  |  |

In order to support IPTV services over widely deployed existing networks, interworking between IPTV and Cable/Terrestrial TV should be considered. If we apply interworking between IPTV and Cable/Terrestrial TV, it would allow IPTV subscribers to access existing network services without any addition. Consequently, IPTV subscribers can use the Cable/Terrestrial TV services.

2. Proposal

In this contribution, we proposes to insert some interworking requirements between IPTV and Cable/Terrestrial TV.

We hope that these requirements will be inserted on the relevant section in the section 5.5 of the IPTV service requirements document

_______________

< Requirements for interworking between IPTV network and existing networks(Cable/ Terrestrial TV network, IP Network, PSTN/ISDN and  2G/3G network, etc.) >

1. Requirements for interworking between IPTV and Cable/Terrestrial TV network

1.1 Interworking at transport level

- IPTV_ESI_XXX : The IPTV Architecture should provide IP connectivity between hosts attached to the cable modems (the hosts attached to the CMs can communicate only via the CMTS) and between the CM and the core data network.

- IPTV_ESI_XXX : The IPTV Architecture should act as a bridge or router: it performs data forwarding in transparent bridging or network-layer forwarding mode, or a combination of the two.

- IPTV_ESI_XXX : The IPTV Architecture should support real-time encoded live programming and pre-encoded stored programming

- IPTV_ESI_XXX : The IPTV Architecture should allocate upstream bandwidth in accordance with CM requests and network QoS policies.

- IPTV_ESI_XXX : The IPTV Architecture should provide the required QoS to the CM based upon DOCSIS requests which are checked against policy.

- IPTV_ESI_XXX : The IPTV Architecture should classify each arriving packet from the network-side interface and assigning it to a QoS level based on defined filter specifications.

- IPTV_ESI_XXX : The IPTV Architecture should perform traffic shaping and policing as required by the flow specification.

- IPTV_ESI_XXX : The IPTV Architecture should forward downstream packets to the DOCSIS network using the assigned QoS.

- IPTV_ESI_XXX : The IPTV Architecture should forward upstream packets to the backbone network devices using the assigned QoS.

1.2 Interworking at service level

- IPTV_ESI_XXX : The IPTV Architecture should use a unique set of APIs on all platforms.

- IPTV_ESI_XXX : The IPTV Architecture should be reauthoring the content and applications for each platform.

- IPTV_ESI_XXX : The IPTV Architecture should use a common content format, with automatic transformation capability to various legacy APIs

1.3 Interworking via end user terminal level

- IPTV_ESI_XXX : IPTV end user terminal should be capable of adequately receiving all channels allocated by the Cable/Terrestrial broadcaster.

- IPTV_ESI_XXX : Random access to channels should be possible and IPTV end user terminal should provide fast channel search.

- IPTV_ESI_XXX : IPTV end user terminal should be able to select either 16:9 or 4:3 display mode either by manual selection or by responding to the transmitted status signal.

- IPTV_ESI_XXX : IPTV end user terminal should use standardized interfaces for connection to peripheral equipment.

- IPTV_ESI_XXX : IPTV end user terminal should have sufficient transmission capacity.

- IPTV_ESI_XXX : IPTV end user terminal shall either use the NIT(Network Information Table) information or the scanning procedure for retrieving the services available on the cable network.

- IPTV_ESI_XXX : IPTV end user terminal shall provide a scanning procedure over the whole Cable TV frequency bands. During this scanning procedure all relevant information about the available digital and analogue services shall be accessible by the application.

- IPTV_ESI_XXX : IPTV end user terminal shall include one tuner/demodulator for reception of signals from terrestrial transmitters.

- IPTV_ESI_XXX : IPTV end user terminal shall provide a function to perform an automatic search that finds all of the multiplexes and services in the whole frequency range.

- IPTV_ESI_XXX : IPTV end user terminal shall only display a service once in the service list, even if the same service is received from multiple transmitters

2. Requirements for interworking between IPTV and IP Network

- IPTV_ESI_XXX : TBD

3. Requirements for interworking between IPTV and PSTN/ISDN

- IPTV_ESI_XXX : TBD

4. Requirements for interworking between IPTV and  2G/3G network

- IPTV_ESI_XXX : TBD

Reference

[1] ITU-T Recommendation J.160, Architectural framework for the delivery of time-critical services over cable television networks using cable modem.

[2] NorDig Requirements for the Basic TV profile for Digital Integrated Receiver Decoders for use in cable, satellite, terrestrial and IP-based networks.

[3] Internet-Draft, DOCSIS 3.0 Requirements for IPv6 support

[4] DVB DOCUMENT A003.

_______________