| Abstract INTERNATIONAL TELECOMMUNICATION UNION |  |  |  | Focus Group On IPTV |
| --- | --- | --- | --- | --- |
| TELECOMMUNICATION STANDARDIZATION SECTOR STUDY PERIOD 2005-2008 |  |  | FG IPTV-C-0455 |  |
|  |  |  | English only |  |
| WG(s): 1 |  | 4th FG IPTV meeting: Bled, Slovenia, 7-11 May 2007 |  |  |
| CONTRIBUTION |  |  |  |  |
| Source: | Nortel Networks (Canada) |  |  |  |
| Title: | Comments on sub-clause 5.7 of FG IPTV- DOC-0060 |  |  |  |

This contribution proposes changes to sub-clause 5.7 of FG IPTV - DOC-0060, working document on IPTV services requirements.

Background

At the most recent FG IPTV meeting, i.e. 22-26 January 2007, significant progress was made and a better document was produced.

Discussion

This contribution proposes some changes which is hoped to enhance accuracy of texts in this sub-clause.

Reference

FG IPTV- DOC-0060 (22-26 January 2006), working document: IPTV services requirements

Proposal

It is proposed to replace content of sub-clause 5.7 with the following.

## 5.7	Public interest requirements

### 5.7.1 Accessibility Requirements

Provide accessibility for people with disabilities or special needs and meet the minimum regulatory requirements.

Additionally, these requirements will provide accessibility for people with temporary environmental dysfunctions to include people with varied or different languages, to those with learning difficulties and to those whose capabilities are diminished by age and degeneration By 2015, it is estimated that 50% of the population will be over the age 50* and require accessibility features, therefore making such features mainstream requirements.  It is also noted that some of these accessibility features are presently mainstream in their use and have been included in other clauses in this document. Appropriate references to those other clauses for such accessibility features are included in the list below as an aide for the implementer, as national regulation may apply.

(Note: Some of the following requirements below may have regulatory implications in some countries and may not be required for all video IPTV applications, this may change over time. National regulations may place specific additional requirements that shall be honored).

As the high level requirements, the architecture and services shall provide:

IPTV_PUB_001:

The IPTV Architecture shall provide mechanisms to support closed captioning [IIF.ARCH.SERVICE.43].

IPTV_PUB_999: The IPTV Architecture shall support the ability of the ITF to decode and display closed captioning information [IIF.ARCH.SERVICE.44].

IPTV_PUB_002: The IPTV Architecture shall support the ability to view the captions and vary its presentation in a separate window. Reference needed

IPTV_PUB_003: The IPTV Architecture shall support the ability to select and receive two (related) video sources <<Simultaneously>>.. (e.g. one with sign language translation) Reference needed

IPTV_PUB_004: The IPTV Architecture shall support the ability to select and receive two audio sources. (e.g. one with audio description) << what is audio description>>)  Reference needed

IPTV_PUB_005: The audio transmission and reproduction should be of good quality to make it possible for people to perceive the sound well. Reference need

IPTV_PUB_006: Video should be transmitted with sufficient quality for sign language perception if there is sign language in the contents [1] Referenced needed

IPTV_PUB_007: Video should be transmitted with sufficient quality for lip reading perception. Reference needed

IPTV_PUB_008: The ITU-T accessibility checklist should be applied to the work on IPTV [2]. Reference will change after AAP process for SG16 for version 5, <<This is a very wide requirement>>

IPTV_PUB_009: IPTV applications and equipments shall be designed using principles of Universal Design (<<what is universal design>>  so that a wider population with users of varying capabilities can access such applications and equipment Reference needed

IPTV_PUB_010: Meta-data shall contain information about the provision of Accessibility Features.

IPTV_PUB_011: Any recordings that the IPTV terminal makes shall also include the appropriate Accessibility Features. The recording of Accessibility Features should be done in such a way that, when watching the recording, they can be both switched on and switched off. Therefore, it is a good idea to record all Accessibility Features streams and link them to the service as a whole.

IPTV_ESI_104: The IPTV architecture shall provide the end-user ability to be notified and receive regulatory information services (unassociated with TV channels) whenever the IPTV Terminal is active (e.g. displaying an EPG or a TV channel).

IPTV_ESI_105: The reception of regulatory information services shall trigger mechanisms to draw the customer attention.

IPTV_ESI_157: The IPTV Terminal should process emergency messages if they are available in the transmission infrastructure.

IPTV_MID_013: IPTV Terminal should support presentation of captions.

### 5.7.2 Emergency telecommunications

IPTV_PUB_012: The IPTV Architecture shall support Emergency Alert Service per the relevant national regulations -- e.g., ANSI J-STD-042-2002 [IIF.ARCH.SERVICE.36].

IPTV_PUB_013: The IPTV Architecture shall provide appropriate interfaces to support the ingestion of suitably authorized Emergency Alert information [IIF.ARCH.SERVICE.37].

IPTV_PUB_014: The IPTV Architecture shall support the ability for the ITF to decode and display emergency alert information [IIF.ARCH.SERVICE.38].

IPTV_PUB_015: Meta-data shall contain information about the provision of Accessibility Features.

- IPTV_PUB_016: Any recordings that the equipment makes shall also include the appropriate Accessibility Features. The recording of the Accessibility Features should be done in such a way that, when watching the recording, they can be both switched on and switched off. Therefore, it is a good idea to record all relevant Access Service streams and link them to the service as a whole.

- IPTV_PUB_017: Applications and equipment shall be designed using principles of Universal Design* and cater for the needs of the users of such applications and equipment.

### 5.7.3 Provider Selection and Portability Requirements

IPTV_PUB_018: The IPTV architecture should support a mechanism for customers to select IPTV network providers, IPTV service providers, and IPTV content providers with their preference.

IPTV_PUB_019: The IPTV architecture may support number portability capabilities.

From ATIS

Note: Remove the diagram Figure 1 (already in the architecture)

# Appendix I: Bibliography

[EN 1332-4] 	CEN Identification Card Systems - Man-Machine Interface - Part 4: Coding of user requirements for people with special needs.

_____________

*	 J M Gill, Ms S A Perera: Accessible Universal Design of Interactive Digital Television;  http://www.tiresias.org/reports/brighton.htm