| 1. Introduction INTERNATIONAL TELECOMMUNICATION UNION |  |  |  | Focus Group On IPTV |
| --- | --- | --- | --- | --- |
| TELECOMMUNICATION STANDARDIZATION SECTOR STUDY PERIOD 2005-2008 |  |  | FG IPTV-C-0456 |  |
|  |  |  | English only |  |
| WG(s): 3, 5 |  | 4th FG IPTV meeting: Bled, Slovenia, 7-11 May 2007 |  |  |
| CONTRIBUTION |  |  |  |  |
| Source: | LG Electronics (Republic of Korea) |  |  |  |
| Title: | IPTV metadata requirements for content management and protection |  |  |  |

IPTV systems need to distinguish the type of content in accordance with the usage rule for content and the content rights policy of content/service provider. This document proposes the meta information that is necessary for content management and protection considering the usage right of IPTV contents for content store and redistribution.

2.	Discussion

The information for content management and protection of IPTV contents stored in IPTV Client Device storage is to be transferred through the IPTV service network. Then, IPTV content/service provider can define the rights and protection scheme of content in that information. This document also defines some approaches for content management to take into account the device capability and the interoperability.

[Figure 1.] Content Management and Protection : Use-cases

Phase 1 in this document defines the rights information for content store and redistribution and describes appropriate content protection approaches using the information. Following Phase 2 will propose a series of use cases and associated requirements of content store and redistribution, and some information for the interoperability and the capability (resolution, bit-rate etc.) for the content.

IPTV service provider transfers the content with rights information (for content store and distribution rights, content protection scheme, etc.) as the meta information based on the usage rights of content. Using this meta information, IPTV Client Device would process content store and distribution with the content protection scheme.

  - Rights information for content store/distribution

The type of content which can be stored in IPTV Client Device is classified according to the usage rights (copy, broadcasting, transmission, redistribution, secondary modification… etc.) with store/distribution point of view.

- “Freely Store/Distribute” content which is not governed by usage rights.

- “Store-Only” Content can be stored locally and is not available to distribute (use only in IPTV Client Device)

- “Distribute-Only” content can be distributed and is not available to store the content locally.

- “Store and Distribute” the content can be stored locally and distributed to other devices if a suitable license is available which permits these operations.

- “No Store and No Distribute” content cannot be locally stored or distributed to other devices. This type of content can only be used to view or render.

Note. Secondary modification to be discussed.

  - Content protection approach based on usage right.

- “Freely Store/Distribute”  : Unencrypted content

- “Store-Only” : Securely-store content

- “Distribute-Only” : Securely-distribute content with usage rule.

- “Store and Distribute” : Securely-store or Securely-distribute content with usage rule.

- “No Store and No Distribute” : only viewing.

Metadata information associated with the content SHALL specify the above usage rights.

3.	Proposals

The requirements identified from these use cases are as follow:

3.1	Requirements

REQ1 : IPTV content SHALL support “Freely Store/Distribute” contents.

REQ2 : IPTV content SHALL support “Store-Only” contents.

REQ3 : IPTV content SHALL support “Distribute-Only” contents.

REQ4 : IPTV content SHALL support “Store and Distribute” contents.

REQ5 : IPTV content SHALL support “No Store and No Distribute” contents.

REQ6: The information for content management and protection SHALL include the information relative to the capability and interoperability of IPTV content.

REQ7: The information for content management and protection SHALL be securely delivered to IPTV Client Device.

REQ8: The information for content management and protection SHALL contain information regarding usage rights of the content. (pertaining to content store, distribution and protection schemes.)

REQ9: The information for content management and protection MAY include the information of the digital watermarking or fingerprinting to track the illegal use of content.

________