| INTERNATIONAL TELECOMMUNICATION UNION |  |  |  | Focus Group On IPTV |
| --- | --- | --- | --- | --- |
| TELECOMMUNICATION STANDARDIZATION SECTOR STUDY PERIOD 2005-2008 |  |  | FG IPTV-C-0467 |  |
|  |  |  | English only |  |
| WG(s): 1 |  | 4th FG IPTV meeting: Bled, Slovenia, 7-11 May 2007 |  |  |
| CONTRIBUTION |  |  |  |  |
| Source: | NEC Corporation |  |  |  |
| Title: | Proposal for IMS-based architecture for IPTV |  |  |  |

1. Introduction

In WG1 Working Document: IPTV Architecture (FG IPTV-DOC-0062[1]), “Non-IMS-based” and “IMS-based” are defined as “NGN-based IPTV Architecture”.  This contribution proposes to clarify “IMS-based IPTV Architecture” consistent with High-level architecture.

2. Discussion

In the third FG-IPTV meeting (Mountain View, 22-26 January 2007), following issues were discussed about “IMS-based architecture for IPTV” (Fig 8-17 on FG IPTV-DOC-0062[1]), but it did not reach to conclusion.

- Does function of “MRFC” and “MRP-FE” really fit for IPTV case ?

IPTV architecture may need some modification.

- Is it appropriate to define interface between “MRFC” and “MRP-FE” as a standard ?

Many of existing IPTV servers implement proprietary interface between control function and streaming function.

- “IMS based” interface is not clear enough.

Is delivery control (such as RTSP) separated from session control (such as SIP), or embedded within SIP (extension) protocol ?

ETSI TISPAN document Draft ETSI TS 182 027 V0.0.4 (2006-12), which is included in the liaison documents from TISPAN (FG IPTV-IL-0041 [3]), gives an answer to the discussion above. In this document,

- “Service Control Function”, “Media Control Function” and “Media Delivery Function” are identified.

- Interface between “Media Control Function” and “Media Delivery Function” is not defined as a standard and left as a product implementation matter.

- “IMS based IPTV architecture” defines three interfaces. i.e.: “Session control”, “Media control” and “Stream Delivery”.

Figure 1 show the IMS based architecture for IPTV quoted from Draft ETSI TS 182 027 V0.0.4 (2006-12).

Figure 1.  IMS-based architecture for IPTV

3. Proposal

It is proposed to modify Figure 8-17 on FG IPTV-DOC-0062[1] as the Figure 2 shown below, which adds media control connection between STB (UE) and MRFC and moves MRFC from inside core IMS to outside core IMS, as IMS-based architecture for IPTV.

Figure 2.  IMS-based architecture for IPTV (modified from Figure 8-17)

4. REFERENCE

[1] FG IPTV-DOC-0062: “Working Document: IPTV Architecture”

[2] FG IPTV-C-0261: “Working Document:  IPTV Architecture”

[3] FG IPTV-IL-0041: “Information to ITU-T IPTV FG about progress of IPTV work inside ETSI TISPAN”, Draft ETSI TS 182 027 V0.0.4 (2006-12)

[4] FG IPTV-C-0404: “Clarification of “Streaming for Broadcast & VoD” on IMS-based IPTV architecture”

_________