| INTERNATIONAL TELECOMMUNICATION UNION |  |  |  | Focus Group On IPTV |
| --- | --- | --- | --- | --- |
| TELECOMMUNICATION STANDARDIZATION SECTOR STUDY PERIOD 2005-2008 |  |  | FG IPTV-C-0482 |  |
|  |  |  | English only |  |
| WG(s): WG 1 |  | 4th FG IPTV meeting: Bled, Slovenia 07-11 May 2007 |  |  |
| CONTRIBUTION |  |  |  |  |
| Source: | Huawei Technologies Co., Ltd. |  |  |  |
| Title: | Discussion on IPTV unicast service establishment procedure |  |  |  |

# Introduction

During 3rd FG-IPTV meeting, a description of the unicast IPTV service is shown as the italic texts below. This contribution illustrates the indicated procedures for discussion.

The following sequence represents one of a number of possible operational models for IPTV. It assumes that all provisioning and initial connection activities have been completed.

- Customer’s network gateway obtains an  IP Address and configuration details, via DHCP or PPP, from network

- IPTV Terminal obtains IP Address and configuration details, via DHCP, from the Gateway

- IPTV Terminal runs IPTV Application and obtains a logical URL for the content that the customer wishes to receive

IPTV Terminal resolves the logical URL of content into the physical URL of content using the IPTV server

  - IPTV server determines the URL of the best server to deliver the content by passing IPTV Terminal’s IP address and content reference to the Content Location service

    - Content Location service uses the IPTV Terminal’s IP address to determine its geographical location

    - Content Location service uses information about the distribution of content in the network to find a server that has the requested content and free resources to deliver it

    - Content Location service uses sever and terminal information to reserve delivery capacity on physical server

    - Content Location service returns URL of physical server and content to the IPTV server

  - IPTV server sets up network resources for path from server to network core

  - IPTV server sets up network resources for path from network core to client

  - IPTV server returns URL of physical server and content to IPTV Terminal

- IPTV Terminal uses its Media Client to connect to and control the delivery of the Content

- Media Client receives Content

# Discussion

Based on the description above, the illustrated procedures are as follows:

1: Session Request

The Terminal initiates a session request carrying a content identifier indicating the desire content and SDP offer with control and content channel parameters, the content identifier maybe a logical URL, e.g. a SIP URL.

2: Interaction with RACF for Qos reservation

Core IMS, more specifically, the P-CSCF interacts with RACF for Qos reservation.

3,4: Session Request

The session request is forwarded to Content Location via Application Server.

5: Media Delivery Selection

Content Location selects a suitable Media Delivery for the request by resolving the logical URL to a physical URL which can identify the selected Media Delivery based on some criteria, e.g. the terminal’s capability, the load of the Media Delivery.

6: Session Request

The session request is forwarded to Content Delivery.

7: Session Response

Content Delivery responses with Session Response carrying the SDP answer with control and content channel parameters.

8,9: Session Response

The session response is forwarded to Core IMS via Application Server.

10: Interaction with RACF for Qos commit

Core IMS, more specifically, the P-CSCF interacts with RACF for Qos commit.

11: Session Response

The session response is forwarded to Terminal.

12: establishment of control and content channels

The control channel and content channel are established.

13: Content Delivery

The content data are delivered for Content Delivery to terminal.

# Proposal

It is proposed to discuss and accept the content in chapter 2 as the high level descriptions of the procedures for unicast IPTV service.

__________