| INTERNATIONAL TELECOMMUNICATION UNION |  |  |  | Focus Group On IPTV |
| --- | --- | --- | --- | --- |
| TELECOMMUNICATION STANDARDIZATION SECTOR STUDY PERIOD 2005-2008 |  |  | FG IPTV-C-0509 |  |
|  |  |  | English only |  |
| WG(s): 2 |  | 4th FG IPTV meeting: Bled, Slovenia, 7-11 May 2007 |  |  |
| CONTRIBUTION |  |  |  |  |
| Source: | Pixelmetrix Corporation |  |  |  |
| Title: | End-to-End monitoring of IPTV networks |  |  |  |

Introduction: The contribution is to describe the parameters that need to be monitored at different stages of an End-to-End IPTV delivery chain. Specific parameters are applicable to only few domains (Domains as described in FG IPTV-DOC-0066 – “Performance monitoring for IPTV”) , but all parameters listed are equally significant for monitoring the true performance of IPTV delivery.

It is essential to monitor the IPTV delivery network End-to-End from ingest of content into the network to final delivery to the customer.

Proposal: To include monitoring parameters in document FG IPTV-DOC-0066 – “Performance monitoring for IPTV”,

Table: Monitoring parameters for End-to-End IPTV network

| Monitoring Parameters | Domain A | Domain B (PT1- PT2) | Domain C (PT2- PT3) | Domain D (PT3- PT4) | Domain E (PT4- PT5) |
| --- | --- | --- | --- | --- | --- |
| RF Integrity | Yes | No | No | No | No |
| Meta Data verification (parental control/ EPG/ Subtitles) | Yes | Yes | No | No | No |
| Packet Loss metrics | Yes (Only applicable to content over IP contribution) | No | Yes | Yes | Yes |
| Jitter | Yes (Only applicable to content over IP contribution) | No | Yes | Yes | Yes |
| Transport stream parameters (TR 101 290) | Yes Priority 1, 2 and 3 | Yes Priority 1 and 2 | Yes Priority 1 | Yes Priority 1 | Yes Priority 1 |
| Video/ Audio Bandwidth | Yes | Yes | Yes | Yes | Yes |
| IGMP join/ leave time | No | No | No | Yes | Yes |
| Channel Zap time | No | No | No | No | Yes |
| Channel line-up verification | Yes | Yes | No | No | Yes |
| Available Bandwidth | No | No | Yes | Yes | Yes |
| Audio/ Video quality | Yes | Yes | No | No | Yes |
| Blackout detection | Yes | Yes | No | No | Yes |
| Freeze Frame detection | Yes | Yes | No | No | Yes |
| Audio Loss/ Presence | Yes | Yes | No | No | Yes |
| Video on demand request performance | No | No | No | No | Yes |

Conclusion:

All parameters proposed are equally significant and need to be monitored to ensure performance and integrity of IPTV delivery. Each parameter has specific attached importance to the domain is has to be monitored in.