| INTERNATIONAL TELECOMMUNICATION UNION |  |  |  | Focus Group On IPTV |
| --- | --- | --- | --- | --- |
| TELECOMMUNICATION STANDARDIZATION SECTOR STUDY PERIOD 2005-2008 |  |  | FG IPTV-0511 |  |
|  |  |  | English only |  |
| WG(s): 6 |  | 4th FG IPTV meeting: Bled, Slovenia, 7-11 May 2007 |  |  |
| CONTRIBUTION |  |  |  |  |
| Source: | China Netcom, Huawei, Tsinghua University, Peking University, Zhe Jiang University, SVA GROUP |  |  |  |
| Title: | Transport of GB 20090.2 video data over ITU-T Rec. H.222.0 ¦ ISO/IEC 13818-1 |  |  |  |

Following steps are suggested to transport GB20090.2 over the ITU-T Rec. H.222.0 | ISO/IEC 13818-1.

Notes:

- ITU-T Rec. H.222.0 | ISO/IEC 13818-1 specify the Transport Stream and the Program Stream.

- The ‘bbv’ in AVS is an equivalent of ‘vbv’ in ITU-T Rec. H.222.0 | ISO/IEC 13818-1.

## Replace table 2-18 in subclause 2:

“

Table 2-18  Stream_id assignments

| stream_id | Note | Stream coding |
| --- | --- | --- |
| 1011 1100 | 1 | program_stream_map |
| 1011 1101 | 2 | private_stream_1 |
| 1011 1110 |  | padding_stream |
| 1011 1111 | 3 | private_stream-2 |
| 110x xxxx |  | ISO/IEC 13818-3 or ISO/IEC 11172-3 or ISO/IEC 13818-7 or GB/T 20090.3 audio stream number x xxxx |
| 1110 xxxx |  | ITU-T Rec. H.262 ¦ ISO/IEC 13818-2, ISO/IEC 11172-2, ISO/IEC 14496-2, or GB/T 20090.2 video stream number xxxx |
| 1111 0000 | 3 | ECM_stream |
| 1111 0001 | 3 | EMM_stream |
| 1111 0010 | 5 | ITU-T Rec. H.222.0 ¦ ISO/IEC 13818-1 Annex A or ISO/IEC 13818-6_DSMCC_stream |
| 1111 0100 | 6 | ITU-T Rec. H.222.1 type A |
| 1111 0101 | 6 | ITU-T Rec. H.222.1 type B |
| 1111 0110 | 6 | ITU-T Rec. H.222.1 type C |
| 1111 0111 | 6 | ITU-T Rec. H.222.1 type D |
| 1111 1000 | 6 | ITU-T Rec. H.222.1 type E |
| 1111 1001 | 7 | ancillary_stream |
| 1111 1010…1111 1110 |  | reserved data stream |
| 1111 1111 | 4 | program_stream_directory |
| The notation x means that the values '0' or '1' are both permitted and results in the same stream type. The stream number is given by the values taken by the x’s. NOTES 1 PES packets of type program_stream_map have unique syntax specified in 2.5.4.1. 2 PES packets of type private_stream_1 and ISO/IEC_13552_stream follow the same PES packet syntax as those for ITU-T Rec. H.262 ¦ ISO/IEC 13818-2 video and ISO/IEC 13818-3 audio streams. 3 PES packets of type private_stream_2, ECM_stream and EMM_stream are similar to private_stream_1 except no syntax is specified after PES_packet_length field. 4 PES packets of type program_stream_directory have a unique syntax specified in 2.5.5. 5 PES packets of type DSM-CC_stream have a unique syntax specified in ISO/IEC 13818- 6. 6 This stream_id is associated with stream_type 0x09 in Table 2-29. 7 This stream_id is only used in PES packets, which carry data from a Program Stream or an ISO/IEC 11172-1 System Stream, in a Transport Stream (refer to 2.4.3.7). |  |  |

”

## 2) Replace table 2-29 in subclause 2:

“

Table 2-29 -- Stream type assignments

| Value | Description |
| --- | --- |
| 0x00 | ITU-T ¦ ISO/IEC Reserved |
| 0x01 | ISO/IEC 11172 Video |
| 0x02 | ITU-T Rec. H.262 ¦ ISO/IEC 13818-2 Video or ISO/IEC 11172-2 constrained parameter video stream |
| 0x03 | ISO/IEC 11172 Audio |
| 0x04 | ISO/IEC 13818-3 Audio |
| 0x05 | ITU-T Rec. H.222.0 ¦ ISO/IEC 13818-1 private_sections |
| 0x06 | ITU-T Rec. H.222.0 ¦ ISO/IEC 13818-1 PES packets containing private data |
| 0x07 | ISO/IEC 13522 MHEG |
| 0x08 | ITU-T Rec. H.222.0 ¦ ISO/IEC 13818-1 Annex A DSM CC |
| 0x09 | ITU-T Rec. H.222.1 |
| 0x0A | ISO/IEC 13818-6 type A |
| 0x0B | ISO/IEC 13818-6 type B |
| 0x0C | ISO/IEC 13818-6 type C |
| 0x0D | ISO/IEC 13818-6 type D |
| 0x0E | ITU-T Rec. H.222.0 ¦ ISO/IEC 13818-1 auxiliary |
| 0x0F | ISO/IEC 13818-7 Audio with ADTS transport syntax |
| 0x10 | ISO/IEC 14496-2 Visual |
| 0x11 | ISO/IEC 14496-3 Audio with the LATM transport syntax as defined in ISO/IEC 14496-3 / AMD 1 |
| 0x12 | ISO/IEC 14496-1 SL-packetized stream or FlexMux stream carried in PES packets |
| 0x13 | ISO/IEC 14496-1 SL-packetized stream or FlexMux stream carried in ISO/IEC14496_sections. |
| 0x14 | ISO/IEC 13818-6 Synchronized Download Protocol |
| 0x15 | Metadata carried in PES packets |
| 0x16 | Metadata carried in metadata_sections |
| 0x17 | Metadata carried in ISO/IEC 13818-6 Data Carousel |
| 0x18 | Metadata carried in ISO/IEC 13818-6 Object Carousel |
| 0x19 | Metadata carried in ISO/IEC 13818-6 Synchronized Download Protocol |
| 0x1A | ITU-T Rec. H.264 ¦ ISO/IEC 14496-10 Video |
| 0x1B-0x41 | ITU-T Rec. H.222.0 ¦ ISO/IEC 13818-1 Reserved |
| 0x42 | GB/T 20090.2 Video |
| 0x43 | GB/T 20090.3 Audio |
| 0x44-0x7E | ITU-T Rec. H.222.0 ¦ ISO/IEC 13818-1 Reserved |
| 0x7F | IPMP stream |
| 0x80-0xFF | User Private |

“

## 3) Replace table 2-39 in subclause 2:

“

Table 2-39 -- Program and program element descriptors

| descriptor_tag | TS | PS | Identification |
| --- | --- | --- | --- |
| 0 | n/a | n/a | Reserved |
| 1 | n/a | n/a | Reserved |
| 2 | X | X | video_stream_descriptor |
| 3 | X | X | audio_stream_descriptor |
| 4 | X | X | Hierarchy_descriptor |
| 5 | X | X | registration_descriptor |
| 6 | X | X | data_stream_alignment_descriptor |
| 7 | X | X | target_background_grid_descriptor |
| 8 | X | X | Video_window_descriptor |
| 9 | X | X | CA_descriptor |
| 10 | X | X | ISO_639_language_descriptor |
| 11 | X | X | System_clock_descriptor |
| 12 | X | X | Multiplex_buffer_utilization_descriptor |
| 13 | X | X | Copyright_descriptor |
| 14 | X |  | Maximum_bitrate_descriptor |
| 15 | X | X | Private_data_indicator_descriptor |
| 16 | X | X | Smoothing_buffer_descriptor |
| 17 | X |  | STD_descriptor |
| 18 | X | X | IBP_descriptor |
| 19-26 | X |  | Defined in ISO/IEC 13818-6 |
| 27 | X | X | MPEG-4_video_descriptor |
| 28 | X | X | MPEG-4_audio_descriptor |
| 29 | X | X | IOD_descriptor |
| 30 | X |  | SL_descriptor |
| 31 | X | X | FMC_descriptor |
| 32 | X | X | External_ES_ID_descriptor |
| 33 | X | X | MuxCode_descriptor |
| 34 | X | X | FmxBufferSize_descriptor |
| 35 | X |  | MultiplexBuffer_descriptor |
| 36 | X | X | AVC-video descriptor |
| 37-62 | n/a | n/a | ITU-T Rec. H.222.0 ¦ ISO/IEC 13818-1 Reserved |
| 63 | X | X | AVS_Video_descriptor |
| 64-255 | n/a | n/a | User Private |

“

## 4) Add after subclause 2.6.35:

“

### 2.6.36 AVS video descriptor

For GB/T20090.2 Video streams, the AVS video descriptor provides basic information for identifying coding parameters of such video elementary streams.

Table 2-62 – AVS  video descriptor

| Syntax | No. of bits | Mnemonic |
| --- | --- | --- |
| AVS_video_descriptor () { descriptor_tag descriptor_length profile_id level_id multiple_frame_rate_flag frame_rate_code AVS_still_present chroma_format sample_precision reserved } | 8 8 8 8 1 4 1 2 3 5 | uimsbf uimsbf uimsbf uimsbf bslbf uimsbf bslbf uimsbf uimsbf bslbf |

### 2.6.37  Semantic definition of fields in AVS video descriptor

profile_id-- This 8-bit field shall identify the profile of the GB/T 20090.2 Video streams.

level_id-- This 8-bit field shall identify the level of the GB/T 20090.2 Video streams.

multiple_frame_rate_flag –This is a 1 bit field which when set to '1' indicates that multiple frame rates may be present in the video stream. When set to a value of '0' only a single frame rate is present.

frame_rate_code-- This is a 4-bit field as defined in GB/T 20090.2, except that when the multiple_frame_rate_flag is set to a value of '1' the indication of a particular frame rate also permits certain other frame rates to be present in the video stream, as specified below:

Table 2-41 -- Frame rate code

| coded as | also includes |
| --- | --- |
| 23,976 |  |
| 24,0 | 23,976 |
| 25,0 |  |
| 29,97 | 23,976 |
| 30,0 | 23,976 24,0 29,97 |
| 50,0 | 25,0 |
| 59,94 | 23,976 29,97 |
| 60,0 | 23,976 24,0 29,97 30,0 59,94 |

AVS_still_present -- This is a 1 bit field, which when set to '1' indicates that the video stream contains only still pictures. If the bit is set to '0' then the video stream may contain either moving or still picture data.

chroma_format -- This is a 2-bit field which is set to the same value as the chroma_format fields in the GB/T 20090.2 video stream.

sample_precision   -- This is a 3-bit field, which is set the same value as the sample_precision fields in the GB/T 20090.2 video stream.

“

## 5) Add after subclause 2.4.2.6:

#### 2.4.2.7  T-STD extensions for carriage of GB/T 20090.2 Video

For decoding of GB/T 20090.2 Video data carried in a Transport Stream the T-STD model is extended. T-STD parameters for decoding GB/T 20090.2 elementary streams are defined in clause xxxx.

## 9)Add clause 2.11 immediately after clause 2.10:

## Carriage of GB 20090.2

### Introduction

An ITU-T Rec. H.222.0 | ISO/IEC 13818-1 stream may carry GB 20090 Video. Typically, a GB 20090.2 Video stream will be an element of an ISO/IEC 13818-1 program, as defined by the PMT in a Transport Stream.

### Carriage in PES packets

GB 20090.2 Video is carried in PES packets as PES_packet_data_bytes, using one of the 16 stream_id values assigned to video, while signaling the GB 20090.2 Video stream by means of the appropriate stream-type value. In addition, such carriage shall be signaled by the AVS_video descriptor, as defined in 2.6.36. This descriptor shall be conveyed in the descriptor loop for the respective elementary stream entry in the Program Map Table in case of a Transport Stream,