| Abstract INTERNATIONAL TELECOMMUNICATION UNION |  |  |  | Focus Group On IPTV |
| --- | --- | --- | --- | --- |
| TELECOMMUNICATION STANDARDIZATION SECTOR STUDY PERIOD 2005-2008 |  |  | FG IPTV-C-0517 |  |
|  |  |  | English only |  |
| WG(s): 1 |  | 4th FG IPTV meeting: Bled, Slovenia, 7-11 May 2007 |  |  |
| CONTRIBUTION |  |  |  |  |
| Source: | ZTE Corporation |  |  |  |
| Title: | Review comments on IPTV Services Requirements |  |  |  |

This contribution provides review comments on “FG IPTV-DOC-0060 (22-26 January 2007), Working Document: IPTV Services Requirements”.

- Discussion

Reference

FG IPTV-DOC-0060 (22-26 January 2007), Working Document: IPTV Services Requirements

- Proposal

It is proposed to modify “FG IPTV-DOC-0060 (22-26 January 2007), Working Document: IPTV Services Requirements” as below.

| No. | Requirements | Review Comments, Proposed Changes |
| --- | --- | --- |
| 1 | IPTV_ARC_005: The IPTV architecture should provide capability enabling service providers may to integrate different contents into a content bundle, or arrange schedule for the program. So the service management function is very important and should be supported. | IPTV_ARC_005: The IPTV architecture should provide service management capability enabling service providers to integrate different contents into a content bundle, or arrange schedule for the program. |
| 2 | IPTV_ARC_012: Dynamic loading and run of applications no issues | IPTV_ARC_012: The IPTV architecture should support dynamic loading and running of applications. |
| 3 | IPTV_ARC_998: The IPTV architecture should support user registration. | IPTV_ARC_998: The IPTV architecture should support user subscription and unsubscription. |
| 4 | 5.1.2.3 Revenue Collection TBD | 5.1.2.3 Revenue Collection IPTV_ARC_XXX: The IPTV Architecture shall support the Collection of Revenue on service consumption by the Service Provider. |
| 5 | IPTV_ARC_127: There are some mature services in the operating network, such as POTS telephone, video conference etc. It’s expected that IPTV service can be integrated with the existing services and it is requested that all these services can be provided through a unified service platform. | IPTV_ARC_127: The IPTV Architecture shall support integration with other existing services with a unified service platform. |
| 6 | IPTV_ARC_073: The IPTV architecture should provide capability to the content provider to manage the content management should be provided to content providers, include uploading content, deleting content or modifying the relevant attributes of content. | IPTV_ARC_073: The IPTV architecture should provide capability to the content provider to manage the content, include uploading content, deleting content or modifying the relevant attributes of content. |
| 7 | 5.1.4.1.4 Customer User Requirements | 5.1.4.1.4 Customer User Requirements IPTV_ARC_XXX: The IPTV Architecture shall support the users to filter unwanted contents which were pushed by the Service Provider. |
| 8 | IPTV_ARC_103: The IPTV architecture should provide consistent QoS for the duration of the service. IPTV_ARC_104: The IPTV architecture should provide consistent QoS for the duration of the service. | IPTV_ARC_103: The IPTV architecture should provide consistent QoS for the duration of the service. (IPTV_ARC_104 is identical to IPTV_ARC_103, and it should be removed.) |
| 9 | IPTV_ARC_124: The IPTV server should interact with the Message server, or integrate the message capability, to provide some message related services | IPTV_ARC_124: The IPTV Architecture should support interaction or integration with message related services. |
| 10 | 5.1.5.2 Service transparency (from network) Service Integration | 5.1.5.2 Service transparency IPTV_ARC_XXX: The IPTV Architecture shall support service without the awareness of the network be used. |

________________