| 1. Introduction INTERNATIONAL TELECOMMUNICATION UNION |  |  |  | Focus Group On IPTV |
| --- | --- | --- | --- | --- |
| TELECOMMUNICATION STANDARDIZATION SECTOR STUDY PERIOD 2005-2008 |  |  | FG IPTV-C-0521 |  |
|  |  |  | English only |  |
| WG(s): 3 |  | 4th FG IPTV meeting: Bled, Slovenia, 7-11 May 2007 |  |  |
| CONTRIBUTION |  |  |  |  |
| Source: | ZTE Corporation |  |  |  |
| Title: | Comments and Modifications on Security Threats |  |  |  |

This contribution gives a review to Working Document of WG3, i.e. FG IPTV-DOC-0067, and gives some comments and modifications on Security Threats.

- 2.	Discussion

During the past 3 meetings of FG IPTV, WG3 got a lot of contributions and excerpted lots of security threats, security requirements into Working Document, and products an updated Working Document, i.e. FG IPTV-DOC-0067.

After the review to Working Document offline, I found that some threats and requirements are not applicable to their place, or duplicated with each other within different Chapter or Sub-Clause, or not according to security wont.

Hereby, I suggest all of us to review and comment WG3 Working Document based on the major technical concepts of ITU-T Recommendations of X.800 and X.805, ITU-T SG17 latest security related specifications and ITU-T SG13 NGN-Security Requirement (Release 1.0 or latest draft version), and give some proposals of refinement such as deletion, mergence, modification and moving the place.

- Comments and Proposed Modification

[Contributor Notes]The following texts refer to X.800[X1], X.805[X3] and ITU-T SG13 NGN-Security.

## 3.1 Comments are as following:

In Security Threats part, there are some texts which are not belong to threats of security and it seem as disorderly and unsystematic, so I propose to reorganize, reorder, merge , remove and modify some of them,  and move some of them to Annex.

## 3.2 Proposed Modifications are as following:

X  Security Threats

This section describes a set of identified security threats that some requirement or mechanism of this document addresses.

The security threat model and other fundamental materials have been addressed in the following ITU-T Recommendations:

•    Recommendation X.800 [X1] defines the general security-related architectural elements which can be applied appropriately in the circumstances for which protection of communication between open systems is required.

(This Recommendation establishes, within the framework of the Reference Model of X.200 [X2], guidelines and constraints to improve existing Recommendations or to develop new Recommendations in the context of OSI in order to allow secure communications and thus provide a consistent approach to security in OSI.)

•    Recommendation X.805 [X3] defines network security architecture for providing end-to-end network security.

(The architecture can be applied to various kinds of networks where the end-to-end security is a concern and independently of the network’s underlying technology. This Recommendation defines the general security-related architectural elements that are necessary for providing end-to-end security. The objective of this Recommendation is to serve as a foundation for developing the detailed recommendations for the end-to-end network security.)

Parties interested in security considerations related to IPTV are invited to read these base security documents, as it is assumed the reader of this document is aware of the information presented in those Recommendations.

The Recommendation X.800 [X1] and X.805 [X3] identify the following security threats to the networks, and which are also security threats to service and content application that are applicable to IPTV:

a) Destruction of information and/or other resources;

b) Corruption or modification of information;

c) Theft, removal or loss of information and/or other resources;

d) Disclosure of information; and

e) Interruption of services.

The table 1 from the Recommendation X.805 [X3] lists the Security Dimensions and describes mapping of Security Dimensions to security threats: the letter 'Y' in a cell formed by the intersection of the table's columns and rows designate that a particular security threat is opposed by a corresponding security dimension.

Table 1 – Mapping of security dimensions to security threats

| Security dimension | Security threat |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Destruction of information or other resources | Corruption or modification of information | Theft, removal or loss of information and other resources | Disclosure of information | Interruption of services |
| Access control | Y | Y | Y | Y |  |
| Authentication |  |  | Y | Y |  |
| Non-repudiation | Y | Y | Y | Y | Y |
| Data confidentiality |  |  | Y | Y |  |
| Communication security |  |  | Y | Y |  |
| Data integrity | Y | Y |  |  |  |
| Availability | Y |  |  |  | Y |
| Privacy |  |  |  | Y |  |

Figure 1 illustrates the security architecture with the architectural elements shown and indicates the security threats described above. The figure depicts the concept of protecting a network by security dimensions at each security plane of each security layer in order to provide a comprehensive security solution. It should be noted that, depending on a given network’s security requirements, it might not be necessary to have all architectural elements implemented (that is, to have a complete set of the security dimensions, security layers and security planes).

Figure 1 – Security architecture for end-to-end network security

## 3.3 Proposed Deletions are as following:

#1:

The following conceptual threat model provides a classification of the related threats to the IPTV system. This threat model requires significantly more refinement to define the complete list of known risks and mitigations. This model does not include network security threats which exist in all IP networks and assumes that such threats have been mitigated.

Content Theft

Service Theft

Information

Theft

Service Disruption

System Control

Compromise IPTV Solution

Figure Error! No text of specified style in document.-1 - Compromise IPTV Solution Attack Tree

#2:

Service Chain

The service chain of an IPTV service can be divided into four parts: Content Source, Service Integration & Operations, Content Transport and Content Consumption.

- Content Source/Content provider: The entities that own or are licensed to sell content to IPTV service providers.

- Service Integration & Operations/Service provider: the entity providing a service to the end-user.

- Content Transport/Delivery network: The delivery network usually is composed of access networks and core or backbone networks, using a variety of network technologies.

- Content Consumption/Consumer: The domain where the IPTV services are consumed.

## 3.4 Proposed Moving to Security Objectives and be merged are as following:

#1:

Security Objectives

- Only legitimate actors can access and operate on the assets of IPTV services after be authorized.

- The IPTV service should be protected against unauthorized deletion, insertion, modification or replay attack.

## 3.5 Proposed Moving to Annex as informative reference are as following:

#1: Table5-1 and its descriptions

Table Error! No text of specified style in document.-1 - Content Assets, Risks, and Threats

| ASSET | RISK | SEVERITY | THREAT | LIKELIHOOD |
| --- | --- | --- | --- | --- |
| Compressed, plaintext content work | Unauthorized copy obtained from network, network device or end system | High if the work is within the release window or deemed highly valuable by the provider; Medium to Low depending on the work. | Cracker | High |
|  |  |  | Professional |  |
|  |  |  | Insider |  |
|  |  |  | Consumer | Medium to Low |
| Compressed, encrypted content work | Unauthorized access | Low | Cracker | Low unless the key is obtained and High otherwise |
|  |  |  | Professional |  |
|  |  |  | Insider |  |
|  |  |  | Consumer | Low |
| Any content work | Denial of service attack | High | Cracker | Low |
|  |  |  | Professional | Low |
|  |  |  | Insider | Low |
|  |  |  | Consumer | Low |

#2: Table5-2 and its descriptions

Table Error! No text of specified style in document.-2 - Service Assets, Risks, and Threats

| ASSET | RISK | SEVERITY | THREAT | LIKELIHOOD |
| --- | --- | --- | --- | --- |
| Domain Name Server | Denial of service attack | Medium | Cracker | High |
|  |  |  | Professional | Medium |
|  |  |  | Insider | Low |
|  |  |  | Consumer | Low |
|  | Unauthorized access | High | Cracker | High |
|  |  |  | Professional | Medium |
|  |  |  | Insider | Low |
|  |  |  | Consumer | Low |
| Electronic Program Guide | Denial of service attack | High | Cracker | High |
|  |  |  | Professional | Medium |
|  |  |  | Insider | Low |
|  |  |  | Consumer | Low |
|  | Unauthorized access | High | Cracker | High |
|  |  |  | Professional | Medium |
|  |  |  | Insider | Low |
|  |  |  | Consumer | Low |
| Media Servers and multiplexers | Denial of service attack | High | Cracker | High |
|  |  |  | Professional | Low |
|  |  |  | Insider | Low |
|  |  |  | Consumer | Low |
|  | Unauthorized access | High | Cracker | High |
|  |  |  | Professional | Low |
|  |  |  | Insider | Low |
|  |  |  | Consumer | Low |
| CAS and Subscriber Management | Denial of service attack | High | Cracker | High |
|  |  |  | Professional | Low |
|  |  |  | Insider | Low |
|  |  |  | Consumer | Low |
|  | Unauthorized use | High | Cracker | High |
|  |  |  | Professional | Low |
|  |  |  | Insider | Low |
|  |  |  | Consumer | Low |

#3: Table5-3 and its descriptions

Table Error! No text of specified style in document.-3 - Network Assets, Risks, and Threats

| ASSET | RISK | SEVERITY | THREAT | LIKELIHOOD |
| --- | --- | --- | --- | --- |
| Network bandwidth | Denial of service attack | Medium | Cracker | High |
|  |  |  | Professional | Medium |
|  |  |  | Insider | Low |
|  |  |  | Consumer | Low |
|  | Unauthorized access or use | High | Cracker | Medium |
|  |  |  | Professional | High |
|  |  |  | Insider | Low |
|  |  |  | Consumer | Low |
| Network Messages | Unauthorized access | High | Cracker | High |
|  |  |  | Professional | Medium |
|  |  |  | Insider | Low |
|  |  |  | Consumer | Low |
|  | Unauthorized modification | High | Cracker | High |
|  |  |  | Professional | Medium |
|  |  |  | Insider | High |
|  |  |  | Consumer | Low |
|  | Replay | High | Cracker | High |
|  |  |  | Professional | Medium |
|  |  |  | Insider | Low |
|  |  |  | Consumer | Low |
| Routers, intermediate systems, access servers and system hosts | Denial of service attack | High | Cracker | High |
|  |  |  | Professional | Medium |
|  |  |  | Insider | Low |
|  |  |  | Consumer | Low |
|  | Unauthorized use | High | Cracker | High |
|  |  |  | Professional | Medium |
|  |  |  | Insider | Low |
|  |  |  | Consumer | Low |

#4: Table5-4 and its descriptions

Table Error! No text of specified style in document.-4 - Terminal Device Assets, Risks, and Threats

| ASSET | RISK | SEVERITY | THREAT | LIKELIHOOD |
| --- | --- | --- | --- | --- |
| Locality | Fixed-location device that is outside residence is given service | Medium | Cracker | Low |
|  |  |  | Professional | Low |
|  |  |  | Insider | Low |
|  |  |  | Consumer | Medium |
|  | Mobile device owned by non-subscriber is given service | Medium | Cracker | Low |
|  |  |  | Professional | Low |
|  |  |  | Insider | Low |
|  |  |  | Consumer | Medium |
| Processor and Disk | Infection by malicious software | High | Cracker | Low unless executables downloaded from the network |
|  |  |  | Professional | Low |
|  |  |  | Insider | Low |
|  |  |  | Consumer | Low |
|  | Unauthorized use | High | Cracker | Low |
|  |  |  | Professional | Low |
|  |  |  | Insider | Low |
|  |  |  | Consumer | Low |
| Interface | Subversion of content protection controls (e.g. HDCP, HDMI) | Medium | Cracker | Low |
|  |  |  | Professional | Low |
|  |  |  | Insider | Low |
|  |  |  | Consumer | Low |

#5: Table5-5 and its descriptions

Table Error! No text of specified style in document.-5 - Subscriber Assets, Risks, and Threats

| ASSET | RISK | SEVERITY | THREAT | LIKELIHOOD |
| --- | --- | --- | --- | --- |
| Subscriber information | Unauthorized access: Disclosure to unauthorized party | High | Cracker | Medium |
|  |  |  | Professional | Medium |
|  |  |  | Insider | Low |
|  |  |  | Consumer | Low |
|  | Unauthorized access: Modification by unauthorized party | High | Cracker | Medium |
|  |  |  | Professional | Medium |
|  |  |  | Insider | Low |
|  |  |  | Consumer | Low |
| Transaction information Credit card info | Unauthorized access: Disclosure to unauthorized party | High | Cracker | Medium |
|  |  |  | Professional | Medium |
|  |  |  | Insider | Low |
|  |  |  | Consumer | Low |
|  | Unauthorized access: Modification by unauthorized party | High | Cracker | Medium |
|  |  |  | Professional | Medium |
|  |  |  | Insider | High |
|  |  |  | Consumer | Low |

## 3.6 Promotion with modification as outline of Security Threats Instance of IPTV roles

Figure Error! No text of specified style in document.-2 - Threat Attack Points

## 3.7 Refine the modified Security Threats of Working Document

For Content Security Threat, Service Security Threat, Network Security Threat, Terminal Security Threat and Subscriber Security Threat parts, they are needed to be abridged and refined.

- Proposals

This contribution proposes to refine the Working Document, and it also gives some comments and some proposals on how to dispose the Security Threats of Working Document.

________