| INTERNATIONAL TELECOMMUNICATION UNION |  |  |  | Focus Group On IPTV |
| --- | --- | --- | --- | --- |
| TELECOMMUNICATION STANDARDIZATION SECTOR STUDY PERIOD 2005-2008 |  |  | FG IPTV-C-0523 |  |
|  |  |  | English only |  |
| WG(s): 1,2 |  | 4th FG IPTV meeting: Bled, Slovenia, 7-11 May 2007 |  |  |
| CONTRIBUTION |  |  |  |  |
| Source: | Cisco |  |  |  |
| Title: | Change to current section 7 (IPTV Reference Model) of the FG IPTV Architecture document, DOC-0062 |  |  |  |

Abstract

This contribution proposes changes to current section 7 of the FGIPTV DOC-0062 from the Mountain View meeting, titled “IPTV Reference Model”.  The proposed figures and textual descriptions attempt to better align the reference model to a full service IPTV Architecture discussed throughout document DOC-0062.

Introduction

In the previous IPTV FG meeting in Mountain View, the material in section 7 of the IPTV Architecture document DOC- 0062 regarding the IPTV related roles was enhanced to include new text.  However, the enhancements to the figures and related text do not entirely encompass the comprehensive applicability of the IPTV architecture in the rest of the document.

This contribution provides further enhancements to the figures and related text, as well as editorial clarifications where required.

Discussion

The IPTV functional architecture being developed in IPTVFG WG 1 is intended to support all envisaged IPTV services and use cases, and this aspect should also be reflected in the reference model for IPTV roles.  However, the current Figure 7-1 appears to only show certain capabilities, and leaves out others which may be equally important.

As this may lead to misleading interpretation of the respective roles, it is desirable to generalize the figures and related text to clarify the wider roles that were implied in the original diagram.

This contribution suggests improved version of the figures in section 7 and editorial enhancements and clarification to the material, to bring it more in line with the subsequent sections and provide more logical flow to the description.

Proposal

The attached document is proposed to replace current section 7 of FGIPTV DOC-0062 from the Mountain View meeting.  The diagrams have been slightly updated and renamed, but no major informational/technical changes have been proposed.  The modified text in section 7 of the attached DOC-0062 from the Mountain View meeting indicates the proposed changes.

# Reference Model for IPTV Roles

The following figure shows the main roles that are involved in provisioning and providing IPTV Service.

Figure 7-1: Reference Model for IPTV Roles

The four roles are as described below:

Content Provider: The entity that owns or is licensed to sell media and data.

Service Provider: The entity that provides the IPTV Service to the Customer.  The Service Provider acquires or licenses content from the Content Provider and packages it into a service that is sold to the Customer.

Network Provider (IP Transport): The Network Provider (which makes use of IP Transport) transports media and control information from the Service Provider to the Customer and vice-versa.  The Network Provider provides the connection from the Service Provider to the Customer.

Note: The Service Provider and the Network Provider may be one entity.  For simplicity, the two are shown as functionally separate entities in the included figures.

Customer: The entity that consumes and pays for the IPTV Service

.

The functional elements shown in each of the four roles above will be described in more detail in section 8, “IPTV Architecture”.

## Typical IPTV Service Scenarios and Use Cases

The following are useful for the description of the basic service scenarios and possible use cases of an IPTV Service.  This is by no means and exhaustive list of potential scenarios and use cases, but is instead meant to serve as a reference to a few potential uses and scenarios of an IPTV Service.

### Linear IPTV with Local PVR

A key part of a typical IPTV Service is the delivery of Linear TV.  Linear TV is received from the existing broadcast transmission from a Content Provider, trans-coded by the Service Provider, and delivered by the Network Provider through IP Transport, via the Customer’s Residential Gateway (Home Network) to the IPTV Client.  The Residential Gateway and IPTV Client may be the same device.  The Residential Gateway is a form of IPTV Client, whose role is to act as a gateway between the Network provider and the Customer Home Network, including the other IPTV Clients in the Customer’s premise.

With a Local PVR, the IPTV Client will have an internal and/or external means of storage, which allows the Customer to time-shift linear TV.  The Customer can record Linear TV onto the Storage medium for later viewing, and can time-shift real-time TV (pause, rewind, fast forward, etc…) for more personalized interaction with real-time TV streams.

Figure 7-2: Linear IPTV with Local PVR Use Case

### Content on Demand

Another typical use case and service scenario of an IPTV Service is the delivery of On Demand Content (VOD).  The content is received from the Content Provider over normal distribution means or in the form of physical media, such as digital videotape.  The content is then trans-coded and stored (in Storage media physically located at the Service Provider site) by the Service Provider and made available as unicast streams to the Customer.  When requested by the IPTV Client, the unicast streams are delivered by the Network Provider through IP Transport via the Customer’s Residential Gateway (Home Network) to the IPTV Client.

Figure 7-3: Content On Demand Use Case

### Hybrid: Online and Over-the-air (OTA) TV Delivery

An alternative and/or complementary approach to the delivery of Linear TV is to enable the IPTV Client to directly receive Over-the-air (OTA) broadcast signals from the Content Provider (shown below in Figure 7-4 with a satellite/antenna connection from the Customer directly to the Content Provider).  This received content may be combined with online content delivered through normal means (from the Service Provider and Network Provider through IP Transport and the Customer’s Residential Gateway to the IPTV Client) at the IPTV Client.

Figure 7-4: Hybrid: Online and Over-the-air (OTA) TV Delivery Service Scenario

## IPTV Definitions

In the description of IPTV Services a number of different terms, such as service, application, and component are often used. For consistency the following definitions are to be used

The IPTV Service is the total proposition offered to the customer, it offers a set of

Applications that provide different Service features to the customer

Live TV

On Demand Video

Communications

Games

These make use of a set of underlying support services that provide common facilities these include.

Unicast Video Delivery

Multicast Video Delivery

An IPTV component provides part of a service, examples are

Video streamer

[Editor’s note:  Whether the definitions of the IPTV service and Application is consistent with NGN document needs to be checked.]

[Editor’s note: It was suggested during the scenarios that Content Aggregation Entity to be considered.]

[Editor’s note: The table of content is provisional and will include more sections]

Based in part on the definitions in the ATIS IIF Architecture Requirements document ATIS-0800002