| INTERNATIONAL TELECOMMUNICATION UNION |  |  |  | Focus Group On IPTV |
| --- | --- | --- | --- | --- |
| TELECOMMUNICATION STANDARDIZATION SECTOR STUDY PERIOD 2005-2008 |  |  | FG IPTV-C-0618 |  |
|  |  |  | English only |  |
| WG(s): 1 & 3 |  | 5th FG IPTV meeting: Geneva, 23 – 31 July 2007 |  |  |
| CONTRIBUTION |  |  |  |  |
| Source: | Nortel Networks (Canada) |  |  |  |
| Title: | Comments on sub-clause 6.3 of FG IPTV-DOC-0083 |  |  |  |

Abstract

This contribution provides replacement text for sub-clause 6.3 of “FG IPTV-DOC-0083 (7-11 May 2007), Working Document: IPTV Services Requirements” in order to enhance its accuracy and quality.

Background and discussion

At the most recent FG IPTV meeting, i.e. 7-11 May 2007, significant progress was made on the IPTV Services Requirements document. However, more work is needed to enhance its accuracy and appearance. This contribution proposes text replacement for sub-clause 6.3 for the aforementioned reasons and purposes.

For this contribution and the companion contributions addressing FG IPTV-DOC-0083, the following convention has been proposed for ease of understanding also to help placing requirements under proper heading when revising the document. This is also to facilitate examination of each requirement as for correctness of its stated intention.

Requirements have been rearranged and been placed under three headings as:

- ……………. is required to (e.g. The IPTV architecture is required to)

- …………… is recommended to (e.g. The IPTV architecture is recommended to)

- …………… can optionally (e.g. The IPTV architecture can optionally)

Reference

FG IPTV-DOC-0083 (7-11 May 2007), working document: IPTV Services Requirements

Proposal

It is proposed that sub-clause 6.3 of FG IPTV-DOC-0083 be replaced with the following.

Contributor’s note: In sub-clause 6.3 the phrase “IPTV receiving device” is used. Was this meant to be identical to an “IPTV terminal device”? If so, then appropriate changes should be made, viz replace IPTV receiving device with IPTV terminal device. Otherwise, a definition of an “IPTV receiving device” is required and should be generated during his meeting and inserted into clause 3 before it is used.

6.3	Requirements for Service Security and Contents Protection Aspects

### 6.3.1	Digital Rights Management

### 6.3.2	Content

The IPTV architecture is required to

- IPTV_NET_036: support a mechanism for the service provider to authenticate the source of content [IIF.ARCH.OPERATOR.22].

The IPTV architecture is recommended to

- IPTV_SEC_007: provide the capability to service providers and/or content providers to present end-user content related information. Contributor’s note: Needs to be written in a more explicit way

- WG6R-0074-010: support secure delivery of the content

- IPTV_NET_899: support security capabilities for multicast enabled networks to protect IPTV services from the unintended service forgery or abuse.

The IPTV architecture can optionally

- IPTV_SEC_004: include the ability for applications to interact with and be managed by the content management and protection capabilities [IIF.ARCH.OPERATOR.32].

- IPTV_SEC_C_116: support the inclusion of content tracing information  in distributed signals. This content tracing information can optionally contain the operator ID, content owner ID, IPTV Receiving Device ID, and other information.

### 6.3.3	Security

IPTV_SEC_013: The IPTV Architecture is recommended to support security management capabilities, such as user authentication, network security, software download authentication, etc.

### 6.3.4	Content protection

The IPTV architecture is required to

- IPTV_SEC_C_001: support content protection, which is defined as “Ensuring that an end-user can only use content they have previously acquired in accordance with the rights that they have been granted by the rights holder.”

- IPTV_SEC_C_101: provide sufficient bandwidth to support protection of content that is distributed simultaneously to a very high number of subscribers.

- IPTV_SEC_C_102: support protection of content that is transferred over multicast and/or over unicast streams.

- IPTV_SEC_C_103: make possible rights specifications and protect content for multiple common existing content formats (e.g. MP-3, MPEG-4, etc.) and/or common computing platforms.

- IPTV_SEC_C_105: support the securing of DVR content. Contributor’s note: By what means and under what conditions?

- IPTV_SEC_C_106: support the service provider to apply robust content tracing to content in real-time (e.g., broadcast content).

- IPTV_SEC_C_107: support the service provider to apply robust content tracing to content in an offline manner (e.g. VoD content).

- IPTV_SEC_C_113: pass through and not interfere with the distribution of any content tracing technology encoded within the content. Such content tracing technology can optionally be designed to uniquely identify the supplier of content.

- IPTV_SEC_C_114: support the application of content tracing technology within the output of an IPTV Receiving Device for the purpose of uniquely identifying a session (e.g., channel, time/date), IPTV Receiving Device and/or network operator. Examples of such content tracing technology can optionally include visible and invisible information.

- IPTV_SEC_C_115: allow the retrieval of all the content tracing information from the content.

The IPTV scrambling algorithm is required to

- IPTV_SEC_C_108: have sufficiently large key entropy to protect the content fully from plausible threats (e.g., at least 128 bits for the AES symmetric algorithms).

- PTV_SEC_C_109: be built using publicly-available and standardized cryptographic algorithms

Contributor’s note: In the following there is a need to specify what is meant by terms Simulcrypt or Multicrypt approaches before accepting the following requirement

- IPTV_SEC_C_111: not preclude support for either Simulcrypt or Multicrypt approaches. The purpose is to support multiple DRM vendors within a single network platform when required by the content distributor.

Contributor’s note: In the following need to specify where the 10 year figure is coming from and the criterion (criteria) upon which it is based.

- IPTV_SEC_C_112: be designed to last a minimum of 10 years with suitable, variable key lengths defined by the appropriate experts. The technical solution is required to allow configurable, variable parameters (such as key length, crypto periods, etc.) allowing for scalable and future proof levels of security in the same stream. Given the current rate of technology advances in processor, storage, etc., the designers of a secure scrambling algorithm must consider the capabilities of offline descramblers to eventually decrypt the content within a reasonable amount of time, using “brute force” attacks. The rationale for the scrambling algorithm designers must be a cost/benefit analysis of the cost of implementing a solution for today that will still be valid at least 10 years out in time.

- IPTV_SEC_T_105:	be efficiently implementable for both hardware and software implementations.

The IPTV scrambling algorithm can optionally

- IPTV_SEC_C_104: provide stronger cryptographic measures for certain content types to protect against theft.

### 6.3.5	Service Security

The IPTV architecture is required to

- IPTV_SEC_S_001:	  support service protection, which is defined as “Ensuring that an end-user can only acquire content that is entitled to receive.”

- IPTV_SEC_S_101:	  support a mechanism to signal the IPTV Receiving Device to utilize a specified scrambling algorithm based on a standardized framework.

- IPTV_SEC_S_102:	  have the ability to use standard key management systems (e.g. MIKE, EMM/ECM), to the extent that this is required for interoperability. Contributor’s note: Need to spell out MIKE, EMM and ECM.

- IPTV_SEC_S_103:	  provide the capability to the server side DRM interfaces to update and query the DRM System concerning the IPTV scrambling algorithm and any other operator-selected scrambling algorithms.

- IPTV_SEC_S_104:	 support rights management that is independent of specific content formats or standards. Contributor’s note: Should indicate what is meant by “standards”

- IPTV_SEC_S_105:	 support a mechanism for secure delivery of entitlements to the IPTV Receiving Devices.

- IPTV_SEC_S_106:	 support content usage control (e.g. re-play entitlements).

- IPTV_SEC_S_107:	 support different modes of re-play entitlements, e.g., limit on number of plays, time limit on plays, restriction of fast forward or rewind.

IPTV_SEC_S_108:	 support impulse transactions. Contributor’s note: A definition is required for impulse transactions and it should be added during this meeting.

- IPTV_SEC_S_109:	 support a mechanism for transmitting signalling messages between the DRM server and the IPTV Receiving Device DRM component.

- IPTV_SEC_S_110:	 support a mechanism to allow for the confidentiality of signalling messages.

- IPTV_SEC_S_111:	 support a mechanism to allow for the authentication of signalling messages.

- IPTV_SEC_S_112:	 support a mechanism to allow for the integrity of signalling messages.

- IPTV_SEC_S_115:	 support a mechanism to allow the IPTV operator to securely retrieve the parameters (e.g. configuration, status) of an IPTV Receiving Device DRM component.

- IPTV_SEC_S_116:	 support a mechanism to allow IPTV operator to securely update the parameters (e.g. configuration) of DRM Components.

- IPTV_SEC_S_117:	 support the operator to turn on and off content tracing function with flexibility (e.g. based on time, an event, asset, or channel).

The IPTV architecture is recommended to

- IPTV_SEC_S_114:	 support a “blackout mechanism”: a mechanism for limiting viewing-rights of certain programs to certain groups of subscribers e.g., block viewing by residents of a specific area (for example, this can optionally be useful for sporting events).

The IPTV architecture can optionally

- IPTV_SEC_S_113:	 enable content confidentiality, i.e. provide a capability to the head-end to support multiple scrambling algorithms. When this capability is used, the IPTV operator is required to be able to select and/or support/disable any of the supported scrambling algorithms at the head-end.

6.3.6	Network Security Requirements

IPTV architecture is required to

IPTV_SEC_N_101: work well, even in the presence of the following: NAT/NAPT, Firewalls, and DHCP.

Editor’s note: indicate what is meant by “work well”

### 6.3.7	Terminal Security

- The IPTV architecture is required to

- IPTV_SEC_T_001:	support terminal device protection, which is defined as “Ensuring that a device employed by a end-user in the reception of a service can reliably and securely use content while enforcing the rights of use granted for that content, and while physically and electronically protecting the integrity of the device.”

- IPTV_SEC_T_101:	 support a method to authenticate IPTV Receiving Devices.

- IPTV_SEC_T_102:	 support physical tamper-resistance.

- IPTV_SEC_T_103:	 support a means to detect when physical tampering has occurred.

- IPTV_SEC_T_104:	 support secure download and installation of the DRM operating code to IPTV Receiving Devices.

### 6.3.8	Subscriber Security

The IPTV architecture is required to

- IPTV_SEC_E_001:	 support end-user privacy protection, which is defined as “Ensuring that information considered to be private (or confidential) by a end-user be maintained in confidence, while remaining subject to mandatory disclosure due to legal processes.”

- IPTV_SEC_E_102:	 support a mechanism to allow an IPTV Receiving Device DRM component to authenticate the DRM servers.

- IPTV_SEC_E_104:	 support a mechanism for restricting access to programs/system by allowing the subscriber to set an access code (i.e. password).

- IPTV_SEC_E_105:	 support a mechanism for parental rating i.e., facilities for rating programs according to content.

- IPTV_SEC_E_106:	 support subscribers’ flexible usage controls to limit access to content by content type (such as VoD, TV, WWW, etc.).

- IPTV_SEC_E_107:	  be capable of indicating why access to content has been denied to the user.

- IPTV_SEC_E_108:	 allow for a mechanism for a Subscriber to obtain extensions (e.g. more plays, more play-time) to digital rights associated with specific content instances.

The IPTV architecture is recommended to

- IPTV_SEC_E_101:	 allow the end user (as rights allow) to change, i.e. replace an IPTV Receiving Device without inherently affecting its rights to consume content.

- IPTV_SEC_E_103:	 not significantly degrade the performance of the IPTV services.

_______________________