| Abstract INTERNATIONAL TELECOMMUNICATION UNION |  |  |  | Focus Group On IPTV |
| --- | --- | --- | --- | --- |
| TELECOMMUNICATION STANDARDIZATION SECTOR STUDY PERIOD 2005-2008 |  |  | FG IPTV-C-0650 |  |
|  |  |  | English only |  |
| WG(s): 1 |  | 5th FG IPTV meeting: Geneva, 23-31 July 2007 |  |  |
| CONTRIBUTION |  |  |  |  |
| Source: | KDDI Corporation |  |  |  |
| Title: | Basic session setup procedure for roaming case in clause 9.1 of FG.IPTV- DOC-0084 |  |  |  |

This contribution proposes to clarify the basic procedure for the roaming case, Figure 9-2 in clause 9.1 of FG.IPTV-DOC-0084.

Background and discussion

At the 4th FG IPTV meeting, three possible architecture scenarios were proposed and accepted in the “IPTV Interconnection over NGN” section. This contribution proposes a basic session setup procedure for the clarification of the roaming case, Figure 9-2 in clause 9.1 of FG.IPTV-DOC-0084.

Reference

FG.IPTV- DOC-0084, working document: IPTV architecture

Proposal

It is proposed to clarify the basic procedure and adopt into right after Figure 9-2 in clause 9.1, “IPTV Interconnection over NGN”.

Figure 9-2-1 illustrates a basic session setup procedure for NGN IMS based IPTV case in Figure 9-2.

Figure 9-2-1: Basic setup procedure for IMS based IPTV in Figure 9-2

The steps in Figure 9-2-1 are described below.

- The IPTV Terminal attaches to the Access Network and requests the user authentication and authorization in order to obtain the valid IP address and establish the link.

- The Access Network sends the user authentication request to the home NACF through the visited NACF.

- After the completion of the user authentication and authorization, the Access Network assigns the valid IP address to the IPTV Terminal and establishes the link.

- The Access Network obtains the user policy rules from the home RACF through the visited RACF and opens the gate for signalling messages (e.g., SIP port number) based on the provider’s agreement.

- The IPTV Terminal executes the SIP registration mechanism with the Core IMS including the User Profile.

- The IPTV Terminal sends a SIP INVITE message to the Core IMS in order to request the IPTV media session. The Core IMS then sends it to the IPTV Application.

- The Core IMS sends the IPTV media information (e.g., IP address, port number, media type and bandwidth) to the home RACF so that the home RACF can request the visited RACF to open the gate on the Access Network for the IPTV media. QoS rules may be allocated to the Access Network.

- The IPTV Terminal receives the IPV media from the IPTV Application.

Figure 9-2-2 illustrates a basic session setup procedure for NGN non-IMS based IPTV case in Figure 9-2.

Figure 9-2-2: Basic setup procedure for non-IMS based IPTV in Figure 9-2

The steps in Figure 9-2-2 are described below.

- The IPTV Terminal attaches to the Access Network and requests the user authentication and authorization in order to obtain the valid IP address and establish the link.

- The Access Network sends the user authentication request to the home NACF through the visited NACF.

- After the completion of the user authentication and authorization, the Access Network assigns the valid IP address to the IPTV Terminal and establishes the link.

- The Access Network obtains the user policy rules from the home RACF through the visited RACF and opens the gate for signalling messages (e.g., IPTV control port number) based on the provider’s agreement.

- The IPTV Terminal requests the IPTV media to the IPTV Service Control including the User Profile.

- The IPTV Service Control requests the IPTV media to the IPTV Application.

- The IPTV Service Control sends the IPTV media information (e.g., IP address, port number, media type and bandwidth) to the home RACF so that the home RACF can request the visited RACF to open the gate on the Access Network for the IPTV media. QoS rules may be allocated to the Access Network.

- The IPTV Terminal receives the IPV media from the IPTV Application.

__________