| Introduction INTERNATIONAL TELECOMMUNICATION UNION |  |  |  | Focus Group On IPTV |
| --- | --- | --- | --- | --- |
| TELECOMMUNICATION STANDARDIZATION SECTOR STUDY PERIOD 2005-2008 |  |  | FG IPTV-C-0654 |  |
|  |  |  | English only |  |
| WG(s): 1 |  | 5th FG IPTV meeting: Geneva, 23-31 July 2007 |  |  |
| CONTRIBUTION |  |  |  |  |
| Source: | CATR/MII, China, China Telecom, Huawei, ZTE Corporation |  |  |  |
| Title: | Proposed Modifications to IPTV Functional Architecture |  |  |  |

This contribution proposes some modifications to the first level functional group decomposition of IPTV architecture in the working document produced in Bled meeting (FG IPTV-DOC-0084e).

# Discussion

## Digital right management functionality

Since digital right management of IPTV content is a necessary functionality of IPTV system, so the architecture needs a set of functional entities to fulfil this crucial requirement. According to the definition in current document (section 7, Application Functions), it is suitable to allocate the DRM server side function into Application Functions.

While on the 1st level decomposition of Application Functions (section 8.1), only three function blocks was identified: IPTV Applications, Application Profile Function, and Content Preparation. According the descriptive text to those function blocks, neither of them can cover the DRM function.

Consequently, in the IPTV Terminal Functions, there will be a DRM Client to handle the interaction with the Right & Key Management function. The function of DRM Client was described in 8.1.1.1.1. It is proposed to draw a independent DRM Client block within IPTV Terminal Functions, as Figure- 1 of this contribution.

## Content stream injection

There is a common practice that data flow and control flow are separated.  Therefore it is proposed to inject the content stream (or file) directly into the Content Delivery & Storage function other than Content Delivery Control Function, see Figure- 1 of this contribution.

Figure-  proposed modifications on the 1st level of function group decomposition (modifications: blocks and lines in blue)

# Proposal

- Proposal #1:  It is proposed to add a new block into and Application Functions (as Figure-1 of this contribution), described as following

8.1.2.4  Right & Key Management

This function is responsible to acquire the content license (or content rights, originated from Content Provider) from Content Preparation function, and key management for Content encryption and de-encryption. When the end user is to consume a piece of content, it generates the right object cooperate with the Application Profile Function and then delivers it to the end user.

- Proposal #2:  It is proposed to draw an independent DRM Client block within IPTV Terminal Functions, as Figure- 1 of this contribution.

- Proposal #3:  It is proposed to inject the content stream (or file) directly into the Content Delivery & Storage function other than Content Delivery Control Function, see Figure- 1 of this contribution.

- Proposal #4:  It is proposed to modify Figure 8-1/2/3/4/5 of architecture working document according to proposal #1, 2, 3 above.

Contributors

| Company | Contributor | Email |
| --- | --- | --- |
| CATR of MII, China | Wei Kai | weikai@catr.com.cn |
| CATR of MII, China | He Baohong | hebaohong@mail.ritt.com.cn |
| China Telecom | Meng Janting | mengjt@sttri.com.cn |
| Huawei | Will Li | lijincheng@huawei.com |
| ZTE Corporation | Hu Jie | Hu.jie@zte.com.cn |

__________