| This contribution proposes modifications on the service requirements regarding Home Network. INTERNATIONAL TELECOMMUNICATION UNION |  |  |  | Focus Group On IPTV |
| --- | --- | --- | --- | --- |
| TELECOMMUNICATION STANDARDIZATION SECTOR STUDY PERIOD 2005-2008 |  |  | FG IPTV-C-0676 |  |
|  |  |  | English only |  |
| WG(s): 1, 5 |  | 5th FG IPTV meeting: Geneva, 23-31 July 2007 |  |  |
| CONTRIBUTION |  |  |  |  |
| Source: | NTT Corporation |  |  |  |
| Title: | Proposed modifications to the service requirements: 6.4.6 Home Network |  |  |  |

Proposal on the service requirement in 6.4.6	Home Network

Original: IPTV_NET_082: The Home Network (HN) is required to support at least one IPI-4 interface with sufficient bandwidth and QoS to support the IPTV services. This is required to include the simultaneous transport of multiple video streams, in addition to voice and data traffic [IIF.ARCH.HOME.09]. In the case that multiple video streams are provided, the requirement shall be met.

Proposal: IPTV_NET_082: The Home Network (HN) is required to support at least one IPI-4 interface with sufficient bandwidth and QoS to support the IPTV services [IIF.ARCH.HOME.09].

Reason - The first sentence is generic and sufficient.  The second sentence states that the simultaneous transport of multiple video streams is mandatory.  However, the last sentence seems to state this requirement is conditional and should be understood as optional.  Hence, there is a concern that the second and last sentence may cause confusion regarding the level of requirement for multiple video streams.  In order to avoid such confusion, it is proposed to delete the second and last sentence and keep the first sentence.

Original: IPTV_NET_086: The RF carrier(s) utilized to carry the digital information over the IPI-4c Interface is required to not interfere with the standard CATV RF plan within the residence. To clarify further, the new RF carrier(s) used for the IPTV services can optionally either operate below the CATV-defined RF spectrum or above it [IIF.ARCH.HOME.13].

Proposal: Deletion

Reason – IPTV_NET_82 already states a generic requirement covering various IPI-4 interfaces applicable to the coaxial case.  The requirement specific to cable access network seems to be unnecessary.  Furthermore, the requirement of frequency allocation on the coaxial cable, as stated in the second sentence, may need the regulatory consideration.  Thus, it is proposed for deletion.

Original: IPTV_NET_087: The IPTV Architecture is required to specify one or more fixed wireless instantiations of the IPI-4d interface. [IIF.ARCH.HOME.14]

Proposal: Deletion

Reason – IPTV_NET_82 already states a generic requirement covering various IPI-4 interfaces.  The requirement specific to wireless access seems to be unnecessary.  Furthermore, this requirement does not state any issue specific to wireless access.  Thus, it is proposed for deletion.

Original: IPTV_NET_095: The IPTV Architecture is required to define mechanisms to support the authentication procedures required by the network and service providers for home network elements [IIF.ARCH.HOME.22].

Proposal: IPTV_NET_095: The home network elements supporting IPTV services are recommended to support the authentication procedures required by the network and service providers [IIF.ARCH.HOME.22].

Reason - According to the original requirement, the home network elements are required to have authentication mechanism by the network and service providers.  Considering the fact that the each home network elements such as router and hub are usually owned and managed by the end-user, it is not plausible that all the home network elements have such an authentication mechanism with the providers.  Also, there may be a privacy concern for such an authentication mechanism.  Thus, this item is proposed to be an optional item.

Original: IPTV_NET_111: The IPTV Architecture is required to support the ability to collect status information from the DNG and ITF remotely [IIF.ARCH.HOME.57].

Proposal: IPTV_NET_111: The DNG and ITF are recommended to support the status information report to the service provider [IIF.ARCH.HOME.57].

Reason - There may be a privacy concern for such status information function in DNF and ITF.  Thus, this item is proposed to be an optional item.  Also, it is desirable to specify the type of information and the relevant circumstance associated with this requirement.

Original: IPTV_NET_113: The IPTV Architecture is required to identify circumstances under which a frequency distribution protocol can optionally be required to support aspects of IPTV services [IIF.ARCH.HOME.59].

Proposal: # Deletion

Reason – The role of frequency distribution protocol is not clear and may not be necessary for the home network entities. It is proposed for deletion unless the clarification is provided.

______________