| INTERNATIONAL TELECOMMUNICATION UNION |  |  |  | Focus Group On IPTV |
| --- | --- | --- | --- | --- |
| TELECOMMUNICATION STANDARDIZATION SECTOR STUDY PERIOD 2005-2008 |  |  | FG IPTV-C-0694 |  |
|  |  |  | English only |  |
| WG(s): 2 |  | 5th FG IPTV meeting: Geneva, 23-31 July 2007 |  |  |
| CONTRIBUTION |  |  |  |  |
| Source: | China Telecom |  |  |  |
| Title: | Proposal on parameters for IPTV monitoring points |  |  |  |

# 1. Discussion

The working document of IPTV FG WG2(FG IPTV-DOC-0089) has defined and identified the parameters across different domains in Table 1 , section 7. But it seems that there is still further work to be done for specifying the undetermined ones.

This contribution  is based on the Table 1 - Monitoring points and parameters and proposes an improved Table.

All the modification is marked as italics blue, e.g.”yes”

| Monitoring Parameters | Domain A | Domain B (PT1- PT2) | Domain C (PT2- PT3) | Domain D (PT3- PT4) | Domain E (PT4- PT5) |
| --- | --- | --- | --- | --- | --- |
| RF Integrity | Yes |  |  |  |  |
| Meta Data verification (parental control/ EPG/ Subtitles) | Yes | Yes |  |  | yes |
| Metadata Validity | Yes | yes |  |  |  |
| Metadata Integrity | Yes | yes |  |  |  |
| AAA Success Rate |  | yes |  |  |  |
| Packet Loss metrics | Yes (Only for content over IP contribution) |  | Yes | Yes | Yes |
| Jitter | Yes (Only for content over IP contribution) |  | Yes | Yes | Yes |
| Transport stream parameters (TR 101 290) | Yes Priority 1, 2 and 3 | Yes Priority 1 and 2 | Yes Priority 1 | Yes Priority 1 | Yes Priority 1 |
| Video/ Audio Bandwidth | Yes | Yes | Yes/NO | Yes/NO | Yes |
| Connection Success Rate |  | Yes | NO | Yes |  |
| IGMP join/ leave time |  |  | No | Yes | Yes |
| Channel Zap time |  |  | No | No | Yes |
| Connection Time |  | Yes | ?? | Yes | Yes |
| Channel line-up verification | Yes | Yes | No | No | Yes |
| Available Bandwidth | No | No | Yes | Yes | Yes |
| Streaming Jitter | Yes | Yes | Yes | Yes | Yes |
| Video quality | Yes | Yes |  |  | Yes |
| Audio quality | Yes | Yes |  |  | Yes |
| Blackout detection | Yes | Yes |  |  | Yes |
| Freeze Frame detection | Yes | Yes |  |  | Yes |
| Audio Loss/ Presence | Yes | Yes |  |  | Yes |
| Video on demand request performance | No | No | No | No | Yes |
| Buffering delay |  |  |  |  | Yes |
| ##### Decoding delay |  |  |  |  | Yes |

Table 1 - Monitoring points and parameters

# 2. Proposal

- It is proposed that the modification for Table 1 in FG IPTV-DOC-0089 would be discussed in WG2 and the accepted modification by all should be added into the Table 1 of working document of  WG2.

Contributor List

| Name | Company | Tel. | Email |
| --- | --- | --- | --- |
| Siqing Luo | China Telecom | +86 21 58755452 | luosq@sttri.com.cn |
| Baohua Lei | China Telecom | +86 10 58552305 | leibh@ctbri.com.cn |
| Jianting Meng | China Telecom | +86 21 58755057 | mengjt@sttri.com.cn |
| Haining Yan | China Telecom | +86 21 28970207 | yanhn@sttri.com.cn |
| Li Jiang | China Telecom | +86 21 58750441 | jiangli@sttri.com.cn |

____________