| INTERNATIONAL TELECOMMUNICATION UNION |  |  |  | Focus Group On IPTV |
| --- | --- | --- | --- | --- |
| TELECOMMUNICATION STANDARDIZATION SECTOR STUDY PERIOD 2005-2008 |  |  | FG IPTV-C-0697 |  |
|  |  |  | English only |  |
| WG(s): 1 |  | 5th FG IPTV meeting: Geneva, 23 – 31 May 2007 |  |  |
| CONTRIBUTION |  |  |  |  |
| Source: | BT, UK. |  |  |  |
| Title: | High-Level Functional Architecture for Cable delivered IPTV |  |  |  |

Context

This contribution proposes changes to the working document: “IPTV Architecture” FG IPTV-DOC-0084 produced at the 4th FG meeting in May.

Discussion

In the ITU-T SG.9 there is work on “Proposed Draft New Recommendation J.iptvfra” that looks at the delivery of IPTV over Secondary Distribution networks, such as cable networks.

This contribution extends the high-level architecture figures in the FG “IPTV Architecture” document to include a figure that includes a ‘Cable’ Network layer. This figure closely aligns with ‘Figure 11: IPTV Functional Architecture’ in T05-SG09-070614-TD-GEN-0518!R2. It should be noted that the referenced figure contains more details than the figure below, where the level of detail is consistent with the other figures in the same section of the FG’s document..

Proposal

### The following additions are proposed to section 8.X of FG IPTV-DOC-0084 after the other high-level functional architecture figures.

### 8.X Cable Delivered IPTV Architecture

The following figure shows the function architecture for IPTV where the network layer is provided by a HFC Cable network. Detail decomposition of the diagram is shown in Draft Recommendation J.iptvfra

Figure 8-X High-Level Functional Architecture for Cable delivered IPTV

### HFC Access Network

### The Hybrid Fibre Coax, HFC, access network is defined as the network between the CMTS and the Cable Modem.  Attributes of the HFC access network include:

### –	Supported or required DOCSIS versions

### –	DOCSIS Set top box Gateway (DSG)

### –	Edge QAMs

### –	Modular CMTS

### –	Optical Distribution Network

### –	RF Network

CMTS

### The cable modem termination system, CMTS, provides support for IP based data services, such as Internet or Voice over IP.

DNS / DHCP Servers

Dynamic Name Service and Dynamic Host Control Protocol servers.

IPTV Service Requirements and Framework for Secondary Distribution, figure 11.