| 1. Abstract INTERNATIONAL TELECOMMUNICATION UNION |  |  |  | Focus Group On IPTV |
| --- | --- | --- | --- | --- |
| TELECOMMUNICATION STANDARDIZATION SECTOR STUDY PERIOD 2005-2008 |  |  | FG IPTV-C-0708 |  |
|  |  |  | English only |  |
| WG(s): 2 |  | 5th FG IPTV meeting: Geneva, 23-31 July 2007 |  |  |
| CONTRIBUTION |  |  |  |  |
| Source: | ZTE Corporation |  |  |  |
| Title: | Modifications for the description of the generalized monitoring method |  |  |  |

This contribution gives two proposals of modifying the description of the generalized monitoring method in the section 8.1 of FG.IPTV-Doc-0089.

- 2. Discussion

2.1 In the section 8.1 of FG IPTV-DOC-066, the output document of 3rd FG IPTV meeting, a generalized monitoring method was described. This method mainly included three stages:

1. After the reference point get the whole copies of sender’s data, the relations between transmission packet number and the characteristics are established and saved. For example, we want to monitor video performance, so video parameters are saved, such as video frame number, macro-block number and location, etc.

2. At the sampling point, received packets are sorted according to packet numbers. We can find out which packets are lost by checking the integrity of packets then feed back to the reference point.

3. At the reference point, according to loss packet numbers from the sampling points, we can retrieve loss characteristics according to the stored relationship between the transmitted packet number and their characteristics. Consequently we can judge which grade current service belongs to. The graded means and details are not discussed in this proposal.

But in the section 8.1 of FG IPTV-DOC-0089, the output document of 4th FG IPTV meeting, the stage 2 and the stage 3 disappeared that made this method incomplete.

2.2 Figure 4 is a typical index structure, so it isn’t exact to call the Figure 4 a example.

3. Proposal

Proposal 1: In the section 8.1 of IPTV Performance Monitoring ,the description of  that “One example of establishing the index is shown in Error: Reference source not found4 ”is suggested to be modified by that “One typical index structure is shown in Figure 4”.

Proposal 2: After the introduction of typical index structure, the following description should be added:

At the sampling point, received packets are sorted according to packet numbers. We can find out which packets are lost by checking the integrity of packets then feed back to the reference point.

At the reference point, according to loss packet numbers from the sampling points, we can retrieve loss characteristics according to the stored relationship between the transmitted packet number and their characteristics. Consequently we can judge which grade current service belongs to. The graded means and details are not discussed in this proposal.

________________