| INTERNATIONAL TELECOMMUNICATION UNION |  |  |  | Focus Group On IPTV |
| --- | --- | --- | --- | --- |
| TELECOMMUNICATION STANDARDIZATION SECTOR STUDY PERIOD 2005-2008 |  |  | FG IPTV-C-0735 |  |
|  |  |  | English only |  |
| WG(s): 6 |  | 5th FG IPTV meeting: Geneva, 23-31 July 2007 |  |  |
| CONTRIBUTION |  |  |  |  |
| Source: | hanarotelecom, TVSTORM |  |  |  |
| Title: | Requirement on the locator for IPTV |  |  |  |

1. Introduction

There are many instances in which middleware application will want to refer to a piece of content, such as when it is playing media, loading a file, or displaying a new service. In many cases, APIs will allow applications to use several different types of content, and we need standardized way of describing how the receiver can access that content. In legacy broadcasting, the locator is used to represent a reference to some content. But the locator for resource referencing in legacy broadcasting has some feature that does not suitable for IPTV. And IPTV service has much richer sources and contents.

This contribution considers the legacy locator and proposes the requirement for the locator to reference diverse and richer contents and services in IPTV.

2. Discussion

The locator is an object that represents a reference to some content in many cases such as when the middleware application is playing media, loading a file, or displaying a new service. The locators have a standardized string representation known as an external form. The external form is a string representing a URL. The types of content that can be referred by using the locator in legacy data broadcasting system are as follows:

- Transport Streams

- Services

- Elementary streams within a service

- Events

- Files in a broadcast file system

- Video drips

IPTV services can have diverse source and content from terrestrial, satellite, cable and the internet. Therefore IPTV middleware is required to support the locator to refer some content and service from variety of sources. And it is needed to consider the harmonization with the locator of the legacy broadcasting to accommodate various contents and services.

Locators for streaming content in a DVB system use the “dvb://” URL format. OCAP uses its own protocol called “ocap://” for its locator and ACAP follows the OCAP locator as well. Both DVB URL and OCAP URL are used to refer a service, elementary stream within a service, a specific event on a service or a file in a broadcast file system. But, in case of OCAP Locator, there are some considerations when we accommodate it into IPTV system as follows:

- There is the locator which uses the frequency and modulation for referencing a service. This is available for the terminal which has a tuner, but it is not applicable to IPTV.

- The application or service using the frequency locator can not be performed on IPTV terminal, and we may get the interoperability problems for the application and service.

- In that case, we need to contrive other locator mechanism and substitute the legacy locator to the corresponding locator for IPTV, or we need to recommend not to use the frequency and modulator locator for the case in which the application or service to be used in IPTV.

3. Proposal

We propose the following requirement to be adopted by FG WG6 for the requirement of IPTV middleware.

- The IPTV middleware is required to support the locator for referencing various content and service from terrestrial, satellite, cable and the internet.