| INTERNATIONAL TELECOMMUNICATION UNION |  |  |  | Focus Group On IPTV |
| --- | --- | --- | --- | --- |
| TELECOMMUNICATION STANDARDIZATION SECTOR STUDY PERIOD 2005-2008 |  |  | FG IPTV-C-0777 |  |
|  |  |  | English only |  |
| WG(s): 2 |  | 5th FG IPTV meeting: Geneva, 23-31 July 2007 |  |  |
| CONTRIBUTION |  |  |  |  |
| Source: | Digital Fountain |  |  |  |
| Title: | DVB-IPI Application Layer FEC for ITU-T FG IPTV Application Layer FEC |  |  |  |

ABSTRACT

This document resolves all remaining issues to move from a provisional endorsement of the DVB-IPI Application Layer FEC solution as the ITU FG IPTV Application Layer FEC. It is proposed to fully endorse the solution by integrating this into the working document for Application Layer Reliability Solutions.

# 1	Introduction

During the fourth ITU-T FG IPTV meeting, the following agreement was reached with respect to the application layer FEC solution:

<BEGIN AGREEMENT>

In the case that a network can not fulfil the requested IPTV service requirements tbd, the use of the DVB-IPI AL-FEC solution [Annex E, DVB-A086] is provisionally endorsed as the ITU-T AL-FEC solution.

  - The details on what parts of the DVB-IPI AL-FEC solution are required, and what is recommended, or what can optionally be used, is tbd.

  - The relation of the AL-FEC solution to the QoS classes in Y.1541 [ITU-Y.1541], especially recommended parameters settings for different QoS classes and different services is tbd.

Provision to the endorsement was made due to the following two reasons:

- The specification [Annex E, DVB-A086] and the accompanying evaluation document [DVB-A0115] was made available to the ITU-T IPTV FG only 3 days prior to the 4th meeting of the ITU-T FG on IPTV. Therefore, some more thorough checking of the details is necessary for some ITU-T IPTV FG participants.

- As DVB is not among the Organizations Qualified for Including References in ITU-T Recommendations under Rec. A.5 Procedures, it formally not possible to add this endorsement to the Working document. The group agreed that an individual company submission at the next meeting with copy&paste of the solution specified in [Annex E, DVB-A086] or to await ETSI publication of the DVB specification will be able to solve this formal issue.”

<END AGREEMENT>

This agreement was only added to the Living List for application layer reliability solutions, but not to the working document because DVB is not among the Organizations Qualified for Including References in ITU-T Recommendations under Rec. A.5 Procedures, it formally not possible to add this endorsement to the Working document.

Section 2 discusses the remaining open issue and proposes a resolution of this problem.

Furthermore, only provisional agreement was achieved due to the late availability of the solution before the last meeting. However, during the last three two months, this information was available for full study in public such that this aspect is resolved.

Clarification on the other two aspects, namely the details on mandatory and optional and some further background on the usage of the AL-FEC solution as well as the relation of the solution to the Y.1541 QoS classes is provided in two other input contributions to this meeting.

# 2	Referencing DVB-IP AL-FEC Solution in ITU-T

To resolve the referencing issue, two options have been considered during the 4th meeting of the ITU-T FG IPTV:

- An individual company submits at the next meeting a “copy&paste” of the solution specified in [Annex E, DVB-A086], or

- The ITU-T FG IPTV awaits ETSI publication of the DVB specification to solve this formal issue.

Obviously, the second proposal would be preferred to avoid significant editing work. Therefore, the status of the publication process of [DVB-A086] within ETSI was tracked. At this point of time, the specification is not yet published, but has assigned an ETSI WI reference for the draft TS 102 034 V1.3.1, namely RTS/JTC-DVB-206. Therefore, it is expected that TS 102 034 V1.3.1 will be published in fall this year and the ETSI reference will be available in for the 6th meeting of the ITU-T FG IPTV.

Therefore, to progress the work, Digital Fountain has decided to submit a copy&paste of the solution specified in [Annex E, DVB-A086] which is identical to Annex E of draft TS 102 034 V1.3.1. The MoU between ETSI and ITU-T also permits the use of draft or adopted ETSI deliverables and other documents provided by ETSI technical bodies. It is proposed to append this specification to the WD on application layer reliability in an Annex. The Annex includes an editor’s note, which explains the process and proposes to remove the Annex as soon as ETSI has published TS 102 034 V1.3.1. The text proposal is provided in Section 4.

# 3	Conclusion

With the background information here, we propose to endorse the DVB-IPI Application Layer FEC solution as the ITU FG IPTV Application Layer FEC. To reflect this proposal, the text to be added to ITU-T FG IPTV WDs is provided in section 4.

# 4	Proposed Modifications to WD on Application Layer Reliability Solutions

It is proposed to add the following subsection to the working document on application layer reliability solutions.

<BEGIN MODIFICATION>

# 10	Conclusions

…

In the case that a network cannot fulfil the requested IPTV service requirements tbd, the use of the DVB-IPI AL-FEC solution as defined in Annex A of this document  is endorsed as the ITU-T AL-FEC solution.

- …

# Annex A (normative)

Editor’s note: The following text is a cut and paste of part of the references as well as Annex E of the ETSI draft TS102 034 v1.3.1 under the study item reference RTS/JTC-DVB-206. Note that ETSI draft TS102 034 v1.3.1 has been approved by the DVB project. As soon as this Technical Specification, from which the text has been cut and pasted, has been approved by ETSI, this Annex will be removed and all pointers to this Annex within this document will be replaced by [Annex E, ETSI TS102 034].

# References

…

[1] 	3GPP TS 26.346 – 3rd Generation Partnership Project; Technical Specification Group Services and System Aspects; Multimedia Broadcast/Multicast Services (MBMS); Protocols and codec’s (Release 6).

[2]	ETSI TS 102472 “Digital Video Broadcasting (DVB);IP Datacast over DVB-H: Content Delivery Protocols”

[3]	RFC3452 - “Forward Error Correction (FEC) Building Block” () and update draft-ietf-rmt-fec-bb-revised-03.

[4]	SMPTE specification 2022-1: Forward Error Correction for Real-time Video/Audio Transport Over IP Networks.

[5]	SMPTE specification 2022-2: Unidirectional transport of constant bit rate MPEG-2 Transport Streams on IP Networks.

# …

# Annex E (normative): Application Layer Forward Error Correction

# E.1	Introduction

This Annex defines an optional protocol for Application Layer FEC (AL-FEC) protection of streaming media for DVB-IP services. This AL-FEC protocol is a layered protocol based on a combination of the following two forward error correction codes:

- a simple packet-based interleaved parity code, equivalent to a subset the code defined in [4]

- the Raptor code, as defined in [1] and [2]

Note that the code defined in [4] is only applicable to the case of media carried within a single RTP flow. In this case, FEC repair packets may be sent in one (or more) layers, the first layer containing packets generated by the interleaved parity code and the optional second and subsequent layers containing packets generated by the DF Raptor code. Receivers process only packets from the layer or layers they support. A key property of the code defined in this specification is that simultaneous support of multiple layers is possible and FEC packets from these multiple layers can be combined at the receiver to achieve error correction performance which is better than any single layer alone.

Section E.3 of this Annex defines the first layer, based on [4].

Section E.4 of this Annex defines the subsequent layers, based on [1] and [2].

Section E.5 of this Annex describes hybrid decoding procedures which can make use of packets from all layers of the code.

Finally, Section E.6 of this Annex defines complete FEC protocols for multicast and unicast video with both MPEG-2 Transport Stream encapsulation and direct transport of audio and video over RTP, constructed using the components described in the previous sections.

# E.2	Terms and Acronyms

| Term/Acronym | Definition/Description |
| --- | --- |
| Bundle | Collection of Streams (a.k.a. Flows) that are collected into a single Source Block, and used to generate a single stream of Repair Symbols. For example, a low-bitrate audio stream might be bundled with a high-bitrate stream, providing better FEC protection than if it hadn’t been bundled. |
| Flow | Another term for “Stream”, used in the context of Bundles. |
| Intermediate Block | A block of data derived from the original Source Block data in the case of DF Raptor Encoder or the combination of Received Source Symbols and Repair Symbols in the case of DF Raptor Decoder. |
| Repair Symbol | A Symbol generated by the DF Raptor Encoder that is derived from Source Symbols. |
| Source Block | A block of source data over which the DF Raptor Encoder provides FEC repair information. |
| Source Symbol | The unit of data from a Source Block. All Source Symbols within a source block are the same size. |
| FEC | Forward Error Correction |
| Encoding Symbol | A source symbol or a repair symbol |
| Source Packet Information (SPI) | Information included in a source block related to or from a source packet |
| FEC Streaming Configuration Information | Information which controls the operation of the FEC Streaming Framework. |
| FEC Payload ID | See [3] |
| Source FEC Payload ID | See [3] |
| Repair FEC Payload ID | See [3] |
| FEC Object Transmission Information | See [3] |
| FEC Encoding ID | See [3] |
| Content Delivery Protocol | See [3] |

Table E.1 - Terms and Acronyms

# E.3	SMPTE 2022-1-based code

SMPTE 2022-1 [4] based coding MAY be applied for streams which meet the requirements of [5].

All requirements of [4] and [5] shall apply, with the modifications and exceptions as shown in Table E.2 below. Modifications/exceptions are classified as follows:

(R)	Additional requirement (normative)

(E)	Exception (normative)

(N)	Note (informative)

Table E.2: Modification/exceptions to [4] and [5]

| Section from [4] and [5] | Modification/exception |
| --- | --- |
| [4] 6.1 RTP/UDP/IP layer | (R) The SSRC SHALL have the same value in every packet of the stream |
| [4] 7 FEC scheme | (N) The term “FEC Scheme” used here does not have the same meaning as “FEC Scheme” in the present document or in [1]. |
| [4] 7.1 FEC packet arrangement | (E) When used with multiple layers, then the L * D block of packets protected by one or more FEC packets SHALL be wholly contained within a single source block of the Raptor code. |
| [4] 7.1 FEC packet arrangement | (E) Only the first FEC stream shall be supported |
| [4] 6.1 FEC buffer overhead and latency implications | (E) The limits defined in this section SHALL NOT apply. Receivers SHALL support values of L and D within the restrictions L*D <= 400 and L <= 40 (L is the burst size). Receivers MAY also support values of L and D outside this range. |
| [4] 7.4 FEC header format | (R) The D bit SHALL be set to 0 |
| [4] 7.4 FEC header format | (R) The SNBase ext bits SHALL NOT be used |
| [4] 7.5 FEC traffic shaping issues | (E) The requirements of this section SHALL NOT apply. |
| [4] 8 System Configuration | (E) In the case of multicast, the FEC Stream MAY use a separate IP multicast destination address to support layered sending |
| Annex B Non block aligned FEC arrangement | (E) The sending arrangement described in this Annex SHALL NOT be used. |

# E.4	Raptor code

## E.4.1	Introduction

The FEC Building Block [3] defined by the IETF Reliable Multicast working group describes an approach to the specification of protocols using FEC but separates the definition of the protocol from the specification of the FEC code itself. In the language of the FEC Building Block, separate specifications are provided for “Content Delivery Protocols” and for “FEC Schemes”, the former defining the protocols and the latter defining the actual FEC codes. The FEC Building Block describes rules that both kinds of specification shall follow so that they can be used together and so it provides the “glue” between Content Delivery Protocols and FEC Schemes.

Following this approach, this section is organized as a number of modular components. These are then combined to form complete protocols suitable for the DVB-IP services. These components include:

- An FEC Streaming Framework, equivalent to that defined in [1], which provides an overall protocol framework for the application of FEC to media streams. This is described in Section E.4.2.

- A number of FEC Schemes, which define protocol components according to the IETF FEC Building Block [3] suitable for various classes of application and which define how the Raptor FEC code is applied for streaming applications. These are defined in Section E.4.3.

Complete protocol specifications for multicast and unicast video with both MPEG-2 Transport Stream encapsulation and direct transport of audio and video encapsulated in RTP are then described in Section E.5. In both cases, the construction is based on the building blocks described above.

## E.4.2	FEC Streaming Framework

### E.4.2.1	Introduction

This section defines a framework for the definition of CDPs, in the sense of the FEC Building Block, which provides for FEC protection of streamed data flows over UDP. This section does not define a complete Content Delivery Protocol, but rather defines only those aspects that are expected to be common to all Content Delivery Protocols that support streaming data over UDP.

The framework defined in this section is not specific to a single streaming application protocol. The framework provides FEC protection for application protocol flows over UDP and for combined protection of multiple such flows. For example, multiple RTP flows may be protected together with the associated RTCP flows and potentially also other related flows such as security protocol packets.

Content Delivery Protocols which use this framework shall provide for communicating two kinds of information from sender to receiver:

- FEC Streaming Configuration Information

- FEC Object Transmission Information

FEC Streaming Configuration Information is information independent of the FEC Scheme being used that is needed by the FEC Streaming Framework, e.g. the definition of the UDP flows that are protected by the FEC Streaming Framework. The FEC Streaming Configuration Information is defined in this section and the means to transport it (for example with Service Discovery Information) shall be defined by each Content Delivery Protocol.

FEC Object Transmission Information is information which is specific to a particular FEC Scheme. The FEC Object Transmission Information is defined by each FEC Scheme. Content Delivery Protocols shall define a means to transport the FEC Object Transmission Information from sender to receiver.

The architecture outlined above is illustrated in Figure E.1.

+ - - - - - - -- - - - - - - - - - - - - - - - - +

|                                                |

+--------------------------------------------+

| |                                            | |

|               Application                  |

| |                                            | |

+--------------------------------------------+

| +---------------------------------+     |      |

|    Application/                 |     |

| | Transport Protocol (e.g. RTP)   |     |      |

|                                 |     |-Configuration/Coordination

| +---------------------------------+     |      |

^                    |

|                    |  UDP flows         |      |

v                    v

| +--------------------------------------------+ |    +----------------+

|                                            |      |                |

| |  FEC Streaming Framework (this section)    |------|   FEC Scheme   |

|                                            |      |                |

| +--------------------------------------------+ |    +----------------+

^

|                    |  UDP flows                |

v

| +--------------------------------------------+ |

|                                            |

| |                   UDP                      | |

|                                            |

+--------------------------------------------+

| +--------------------------------------------+ |

|                                            |

| |                  IP                        | |

|                                            |

| +--------------------------------------------+ |

Content Delivery Protocol

+ - - - - - - - - - - - - - - -  - - - - - - - - +

Figure E.1: FEC Streaming Framework Architecture

### E.4.2.2			Procedural overview

#### E.4.2.2.1	General

The mechanism defined in this section consists of three components:

(i) construction of a 'source block' from source media packets belonging to one or several UDP packet flows. The UDP flows MAY include, for example, RTP and RTCP packets and also other protocols related to the stream.

(ii) optional extension of source packets to indicate the source block and the position within the source block occupied by the data from and related to the source packet.

(iii) definition of repair packets, sent over UDP, which can be used by the FEC decoder to reconstruct missing portions of the source block.

The protected data may be from several different UDP flows that are protected jointly. In general, multiple source blocks will be constructed for a stream; each source block is constructed from different sets of source packets. For example, each source block may be constructed from those source packets related to a particular segment of the stream in time.

A receiver supporting this streaming framework SHALL support the packet format for FEC Source packets and SHALL also support the packet format for FEC Repair packets.

This section does not define how the sender determines which source packets are included in which source blocks. A specific Content Delivery Protocol MAY define this mapping or it MAY be left as implementation dependent at the sender, possibly including some memory constraints at receivers. However, a CDP specification SHALL define how a sender communicates to the receiver themaximum length of time that the sender will allow between a source packet and a repair packet that protects that source packet.

At the sender, the mechanism processes original UDP packets to create:

(i) a stored copy of the original packets in the form of one or more 'source block(s)'. The source block is a logical block of data to which the FEC code will subsequently be applied. It is constructed by concatenating 'Source Packet Information' (SPI) for each source packet. Generally, the SPI for a packet contains a short identifier for the flow the packet belongs to, a length indicator for the packet, the UDP payload and possible padding bytes.

(ii) FEC Source packets for transmission to the receiver.

The FEC Streaming Framework uses the FEC encoder specified by the FEC Scheme in use to generate the desired quantity of repair symbols from a source block. These repair symbols are then sent using the FEC repair packet format to the receiver. The FEC Repair packets are sent to a UDP destination port different from any of the original UDP packets' destination port(s) as indicated by the FEC Streaming Configuration Information.

The receiver recovers original source packets directly from any FEC Source packets received. The receiver also uses the received FEC Source Packets to construct a stored copy of the original packets in the same source block format as constructed at the sender.

If any FEC Source packets related to a given source block have been lost, then this copy of the source block at the receiver will be incomplete. If sufficient FEC source and FEC Repair packets related to that source block have been received, the FEC Framework may use the FEC decoding algorithm defined by the FEC Scheme to recover a (hopefully, but not necessarily, complete) copy of the source block. The SPI for the missing source packets can then be extracted from the completed parts of the source block and used to reconstruct the source packets to be passed to the application.

The receiver of FEC Source packets SHALL be able to identify the source block and the position within the source block occupied by the SPI derived from each packet. This information is know as FEC Source Packet Identification Information and may be communicated in several ways. The FEC Source Packet Identification Information  MAY be encoded into a specific field within the FEC Source packet format defined in this Annex, called the Source FEC Payload ID field. The exact contents and format of the Source FEC Payload ID field are defined by the FEC Scheme. Alternatively, the FEC Scheme or CDP MAY define how the FEC Source Packet Identification Information is derived from other fields within the source packets. This section defines the way that the Source FEC Payload ID field, if used, is appended to source packets to form FEC Source packets.

The receiver of FEC Repair packets SHALL also be able to identify the source block and the relationship between the contained repair data and the original source block. This information is known as FEC Repair Packet Identification information. This information SHALL be encoded into a specific field, the Repair FEC Payload ID field, the contents and format of which are defined by the FEC Scheme.

Any FEC Schemes to be used in conjunction with this framework SHALL be a systematic FEC Scheme and SHALL be based on source blocks. The FEC Scheme MAY define different FEC Payload ID field formats for FEC Source packets and FEC Repair packets.

#### E.4.2.2.2	Sender Operation

It is assumed that the sender has constructed or received original data packets for the session. These may be RTP, RTCP or other UDP packets. The following operations describe a possible way to generate compliant FEC Source packet and FEC repair packet streams:

1. A source block is constructed as specified in Section E.4.2.3.2, by concatenating the SPI for each original source packet. In doing so, the Source FEC Packet Identification Information of the FEC Source packet can be determined and included in the Source FEC Payload ID field, if used. In the SPI the identity of the packet's UDP flow is marked using a short 'UDP flow ID', defined in this Annex. The association of UDP flow specifications to UDP flow IDs is defined by the FEC Streaming Configuration Information.

2. The FEC Source packet is constructed according to Section E.4.2.3.3. The identity of the original flow is maintained by the source packet through the use of the same UDP ports and IP addresses which have been advertised by the Content Delivery Protocol (for example using DVB Service Discovery), as carrying FEC Source packets generated from an original stream of a particular protocol (e.g. RTP, RTCP, etc.). The FEC Source packet generated is sent according to normal UDP procedures.

3. The FEC encoder generates repair symbols from a source block and the FEC Streaming Framework places these symbols into FEC Repair packets, to be conveyed to the receiver(s). These repair packets are sent using normal UDP procedures to a unique destination port to separate them from any of the source packet flows. The ports to be used for FEC Repair packets are defined in the FEC Streaming Configuration Information.

#### E.4.2.2.3	Receiver Operation

The following describes a possible receiver algorithm, when receiving an FEC source or repair packet:

1. If an FEC Source packet is received (as indicated by the UDP flow on which was received):

a. The original source packet is reconstructed by removing the Source FEC Payload ID, if used. The resulting packet MAY be buffered to allow time for the FEC repair.

b. The Source FEC Packet Identification Information is determined, either from the Source FEC Payload ID, if used, or by other means.

c. The SPI for the resulting packet is placed into the source block according to the Source FEC Packet Identification Information and the source block format described in Section E.4.2.3.2. The IP addresses and UDP ports the packet was received on/sent from are used to determine the UDP flow ID within the SPI.

2. If an FEC Repair packet is received (as indicated by the UDP flow on which it was received), the contained repair symbols are associated with a source block according to the Repair FEC Payload ID.

3. If at least one source packet is missing and at least one repair packet has been received for a source block then FEC decoding may be desirable. The FEC decoder determines if the source block constructed in step 1 plus the associated repair symbols received in step 2 contains enough symbols for decoding of any or all of the missing source symbols in the source block and, if so, performs a decoding operation.

4. Any SPI that was reconstructed during the decoding operation is then used to reconstruct the missing source packets and these are buffered as normal received source packets (see step 1a above).

Note that the above procedure may result in a situation in which not all original source packets are recovered.

### E.4.2.3			Protocol Specification

#### E.4.2.3.1	General

This section specifies the protocol elements for the FEC Streaming Framework. The protocol consists of three components which are described in the following sections:

1. Construction of a source block from source packets. The FEC code will be applied to this source block to produce the repair data.

2. A format for packets containing source data.

3. A format for packets containing repair data.

The operation of the FEC Streaming Framework is governed by certain FEC Streaming Configuration Information. This configuration information is also defined in this section. A complete protocol specification that uses this framework SHALL specify the means to determine and communicate this information between sender and receiver.

#### E.4.2.3.2	Structure of Source Block

This clause defines the layout of the source block. A source block consists of the concatenation of SPI for at least one original source UDP packet.

Let

n	be the number of UDP packets in the source block. n MAY be determined dynamically during the source block construction process.

T	be the source symbol size in bytes. Note: this information is provided by the FEC Scheme as defined in SectionE.4.2.3.6.

i	the index to the ( i+1)-th UDP packet to be added to the source block, 0 <= i < n.

R[i]	denote the number of octets of the UDP payload of the i-th UDP packet.

l[i]	be a length indication associated with the i-th UDP packet – the nature of the length indication is defined by the FEC Scheme

L[i]	denote two octets representing the value of l[i] in network byte order (high order octet first)  of the i-th UDP packet.

f[i]	denote an integer 'UDP flow ID' identifying the UDP flow from which the i-th packet was taken

F[i]	denote a single octet representing the value of f[i]

s[i]	be the smallest integer such that s[i]*T >= (l[i]+3). Note s[i] is the length of SPI[i] in units of symbols of size T bytes.

P[i]	denote s[i]*T-(l[i]+3) zero octets. Note: P[i] are padding octets to align the start of each UDP packet with the start of a symbol.

SPI[i]	be the concatenation of F[i] ,L[i], R[i] and P[i].

Then, the source block is constructed by concatenating SPI[i] for i = 0, 1, 2, ... n-1. The source block size, S, is then given by sum {s[i]*T, i=0, ..., n-1}.

Source blocks are identified by integer Source Block Numbers and symbols within a source block by integer Encoding Symbol IDs. This section does not specify how Source Block Numbers are allocated to source blocks. Symbols are numbered consecutively starting from zero within the source block. Each source packet is associated with the Encoding Symbol ID of the first symbol containing SPI for that packet. Thus, the Encoding Symbol ID value associated with the j-th source packet, ESI[j], is given by

ESI[j] = 0, for j=0

ESI[j] = sum{s[i], i=0,...,(j-1)}, for 0 < j < n

The Source FEC Packet Identification Information consists of the identity of the source block and the Encoding Symbol ID associated with the packet.

A UDP flow is uniquely defined by an IP source and destination address and UDP source and destination port values. The assignment of UDP flow ID values to UDP flows is part of the FEC Streaming Configuration Information.

#### E.4.2.3.3	Packet format for FEC Source packets

The packet format for FEC Source packets SHALL be used to transport the payload of an original source UDP packet. As depicted in Figure E.2, it consists of the original UDP packet, followed, optionally, by the Source FEC Payload ID field, if used.

+------------------------------------+

|             IP header              |

+------------------------------------+

|            UDP header              |

+------------------------------------+

|       Original UDP Payload         |

+------------------------------------+

|  Source FEC Payload ID (optional)  |

+------------------------------------+

Figure E.2: Structure of FEC Source Packets

The IP and UDP header fields SHALL be identical to those of the original source packet. The Original UDP Payload field SHALL be identical to the UDP payload of the original source packet. The UDP payload of the FEC Source packet SHALL consist of the Original UDP Payload followed by the Source FEC Payload ID field.

The Source FEC Payload ID field, if present, contains information required for the operation of the FEC algorithm, in particular for the derivation of the Source FEC Packet Identification Information. The format of the Source FEC Payload ID and the derivation of the Source FEC Packet Identification Information are defined by the FEC Scheme. Note that the FEC Scheme or CDP may define a means to derive the Source FEC Packet Identification Information from other information in the source packet (for example the RTP Sequence number).In this case the Source FEC Payload ID field described here is not appended to the packet and the Source FEC packet is identical in every way to the original Source packet.

#### E.4.2.3.4	Packet Format for FEC Repair packets

The packet format for FEC Repair packets is shown in Figure E.3. The UDP payload consists of a Repair FEC Payload ID field and one or more repair symbols generated by the FEC encoding process.

+------------------------------------+

|             IP header              |

+------------------------------------+

|            UDP header              |

+------------------------------------+

|       Repair FEC Payload ID        |

+------------------------------------+

|          Repair Symbols            |

+------------------------------------+

Figure E.3: FEC Repair packet format

The Repair FEC Payload ID field contains information required for the operation of the FEC algorithm. This information is defined by the FEC Scheme. The format of the Repair FEC Payload ID field is defined by the FEC Scheme.

Any number of whole repair symbols may be contained within an FEC Repair packet, subject to packet size restrictions or other restrictions defined by the FEC Scheme. The number of repair symbols within a packet can be determined from the symbol length and the packet length. Partial repair symbols SHALL NOT be included in FEC repair packets.

#### E.4.2.3.5	FEC Streaming Configuration Information

The FEC Streaming Configuration Information is information that the FEC Streaming Framework needs in order to apply FEC protection to the UDP flows. A complete Content Delivery Protocol specification for streaming that uses the framework specified here SHALL include details of how this information is derived and communicated between sender and receiver.

The FEC Streaming Configuration Information includes identification of a number of UDP packet flows. Each UDP packet flow is uniquely identified by a tuple { Source IP Address, Destination IP Address, Source UDP port, Destination UDP port }.

A single instance of the FEC Streaming Framework provides FEC protection for all packets of a specified set of source UDP packet flows, by means of one or more UDP packet flows containing repair packets. The FEC Streaming Configuration Information includes, for each instance of the FEC Streaming Framework:

1. Identification of the UDP packet flow(s) carrying FEC Repair packets, known as the FEC repair flow(s).

2. For each source UDP packet flow protected by the FEC repair flow(s):

a. Identification of the UDP packet flow carrying source packets.

b. An integer identifier, between 0 and 255, for this flow. This identifier SHALL be unique amongst all source UDP packet flows which are protected by the same FEC repair flow.

3. The FEC Scheme that is to be applied

Multiple instances of the FEC Streaming Framework, with separate and independent FEC Streaming Configuration Information, may be present at a sender or receiver. A single instance of the FEC Streaming Framework protects all packets of all the source UDP packet flows identified in (2) above i.e. all packets on those flows SHALL be FEC Source packets as defined in Section E.4.2.3.3. A single source UDP packet flow SHALL NOT be protected by more than one FEC-SF instance.

A single FEC repair flow provides repair packets for a single instance of the FEC-SF. Other packets SHALL NOT be sent within this flow i.e. all packets in the FEC repair flow SHALL be FEC repair packets as defined in Section E.4.2.3.4 and SHALL relate to the same FEC Streaming Framework instance.

The FEC Streaming Framework SHALL be informed of the symbol size to be used for each source block. This information MAY be included in the FEC Streaming Configuration Information or it MAY be communicated by other means, for example within the FEC Repair Payload ID field. A complete Content Delivery Protocol specification SHALL specify how this information is communicated between sender and receiver.

#### E.4.2.3.6	FEC Scheme requirements

In order to be used with this framework, an FEC Scheme SHALL:

- adhere to the requirements of [3].

- be systematic

- be based on source blocks which are non-overlapping and contiguous within the stream

- specify how the Source Block Number and Encoding Symbol ID associated with a source packet are derived or communicated from sender to receiver (for example, within the Source FEC Payload ID field)

- specify how the symbol length is derived or communicated from sender to receiver (for example, as part of the FEC Object Transmission Information).

- specify how the length indication, l[i], included in the Source Packet Information, is derived from a UDP packet.

- specify how the Source Packet Information length, s[i], is derived from a UDP packet.

## E4.3	FEC Schemes for streaming

### E.4.3.1		Raptor FEC Scheme for arbitrary packet flows

This clause defines an FEC Scheme for Raptor protection of arbitrary packet flows over UDP.

#### E.4.3.1.1	Formats and Codes

##### E.4.3.1.1.1	FEC Object Transmission Information

This FEC Object Transmission Information elements for this FEC Scheme and their value ranges are as follows:

Maximum Source Block Length

A non-negative integer less than 216, in units of symbols

Encoding Symbol Size

A non-negative integer less than 216, in units of bytes

An encoding format for this information in a 4 octet field is defined in Figure E.4 below:

1                   2                   3

0 1 2 3 4 5 6 7 8 9 0 1 2 3 4 5 6 7 8 9 0 1 2 3 4 5 6 7 8 9 0 1

+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+

|       Symbol Size (T)         |   Max. Source Block Length    |

+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+

Figure E.4: Encoded Common FEC Object Transmission Information for Raptor FEC Scheme for arbitrary packet flows

##### E.4.3.1.1.2	FEC Payload ID

###### E.4.3.1.1.2.1	Source FEC Payload ID

The Source FEC payload ID is composed as follows:

1                   2                   3

0 1 2 3 4 5 6 7 8 9 0 1 2 3 4 5 6 7 8 9 0 1 2 3 4 5 6 7 8 9 0 1

+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+

|   Source Block Number (SBN)   |   Encoding Symbol ID (ESI)    |

+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+

Figure E.5: Source FEC Payload ID format for Raptor FEC Scheme for arbitrary packet flows

Source Block Number (SBN), (16 bits): An integer identifier for the source block that the source data within the packet relates to.

Encoding Symbol ID (ESI), (16 bits): The starting symbol index of the source packet in the source block.

The interpretation of the Encoding Symbol Identifier is defined by the FEC Streaming Framework (See SectionE.4.2).

###### E.4.3.1.1.2.2	Repair FEC Payload ID

The structure of the Repair FEC Payload ID is defined in Figure E.6 below:

1                   2                   3

0 1 2 3 4 5 6 7 8 9 0 1 2 3 4 5 6 7 8 9 0 1 2 3 4 5 6 7 8 9 0 1

+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+

|   Source Block Number (SBN)   |   Encoding Symbol ID (ESI)    |

+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+

|   Source Block Length (SBL)   |

+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+

Source Block Number (SBN), (16 bits): An integer identifier for the source block that the repair symbols within the packet relate to.

Encoding Symbol ID (ESI), (16 bits): integer identifier for the encoding symbols within the packet.

Source Block Length (SBL), (16 bits): The number of source symbols in the source block.

Figure E.6: Repair FEC Payload ID

The interpretation of the Source Block Number, Encoding Symbol Identifier and Source Block Length is defined by the FEC Code Specification.

#### E.4.3.1.2	Procedures

This FEC Scheme uses the procedures of the framework defined in Section E.4.2.2 to construct a source block to which the FEC code can be applied. The sender SHALL allocate Source Block Numbers to source blocks sequentially, wrapping around to zero after Source Block Number 216-1.

During the construction of the source block as per Section E.4.2.3.2,

-	The length indication, l[i], included in the Source Packet Information for each packet shall be the UDP payload length.

-	The value of s[i] in the construction of the Source Packet Information for each packet shall be the smallest integer such that s[i]*T >= (l[i]+3).

#### E.4.3.1.3	FEC Code specification

The Raptor FEC encoder defined in Section E7 SHALL be used. The source block passed to the Raptor FEC encoded SHALL consist of the Source Block constructed according to Section E.4.2.3.2 extended with zero or more padding symbols such that the total number of symbols in the source block is equal to the Maximum Source Block Length signaled in the FEC Object Transmission Information. Thus the value of the parameter K used by the FEC encoded is equal to the Maximum Source Block Length for all blocks of the stream. Padding symbols shall consist entirely of bytes set to the value zero.

The symbol size, T, to be used for source block construction and the repair symbol construction are is equal to the Encoding Symbol Size signaled in the FEC Object Transmission Information. The parameter T shall be set such that the number of source symbols in any source block is at most KMAX = 8192.

The Maximum Source Block Length parameter – and hence the number of symbols used in the FEC Encoding and Decoding operations - SHALL be set to one of the values specified in Section E.7. Recommended derivation of other parameters is presented in section E.4.3.1.6.

#### E.4.3.1.4	Encoding packet construction

As described in section E.4.2.3.4, each repair packet contains the following information:

-	Source Block Number (SBN)

-	Encoding Symbol ID (ESI)

-	Source Block Length (SBL)

-	repair symbol(s)

The number of repair symbols contained within a repair packet is computed from the packet length. The ESI value placed into a repair packet is given by the following formula:

ESIrepair = Irepair + K

Where Irepair is the index of the repair symbol in the sequence of repair symbols generated according to Section  E.7, where the first repair symbol has index 0, the second index 1 etc. and K is the number of source symbols (equal to the Maximum Source Block Length parameter).

The Source Block Length field of the Repair FEC Payload ID field SHALL be set to the number of symbols included in the Source Packet Information of packets associated with the source block i.e. before padding to the Maximum Source Block Length.

#### E.4.3.1.5	Transport

This sub-clause describes the information exchange between the Raptor encoder/decoder and any transport protocol making use of Raptor forward error correction for streaming.

The Raptor encoder for streaming requires the following information from the transport protocol for each source block:

-	The symbol size, T, in bytes

-	The number of symbols in the source block, K

-	The Source Block Number (SBN)

-	The source symbols to be encoded

The Raptor encoder supplies the transport protocol with encoding packet information consisting, for each repair packet, of:

-	Source Block Number (SBN)

-	Encoding Symbol ID (ESI)

-	Source Block Length (SBL)

-	repair symbol(s)

The transport protocol shall communicate this information transparently to the Raptor decoder.

A suitable transport protocol is defined in this specification.

#### E.4.3.1.6	Example parameters

##### E.4.3.1.6.1	Parameter derivation algorithm

This section provides recommendations for the derivation of the transport parameter T. This recommendation is based on the following input parameters:

-	B	the maximum source block size, in bytes. For further explanation, see below.

-	A	the symbol alignment factor, in bytes, i.e. symbol size T is a multiple of A.

-	P	the maximum repair packet payload size (not including Repair FEC Payload ID), in bytes, which SHALL be multiple of A

-    	KMAX	the maximum number of source symbols per source block. As defined in Section E.7, KMAX = 1281.

-	KMIN	a minimum target on the number of symbols per source block

-	GMAX 	a maximum target number of symbols per repair packet

A requirement on these inputs is that ceil(B/P) ≤ KMAX.  Based on the above inputs, the transport parameter T is calculated as follows:

Let,

G = min{ceil(P·KMIN/B), P/A, GMAX}	- the approximate number of symbols per packet

T = floor(P/(A·G))·A

The value of T derived above should be considered as a guide to the actual value of T used. It may be advantageous to ensure that T divides into P, or it may be advantageous to set the value of T smaller to minimize wastage when full size repair symbols are used to recover partial source symbols at the end of lost source packets (as long as the maximum number of source symbols in a source block does not exceed KMAX).  Furthermore, the choice of T may depend on the source packet size distribution, e.g., if all source packets are the same size then it is advantageous to choose T so that the actual payload size of a repair packet P’, where P’ is a multiple of T, is equal to (or as few bytes as possible larger than) the number of bytes each source packet occupies in the source block.

Recommended settings for the input parameters, A, KMIN  and GMAX are as follows:

A = 16			KMIN = 640			GMAX = 10

##### E.4.3.1.6.2	Examples

The above algorithm leads to transport parameters as shown in Table E.3 below, assuming the recommended values for A, KMIN and GMAX and P = 1424:

Table E.3: Example parameters settings

| Max source block size B | G | Symbol size T | G∙T |
| --- | --- | --- | --- |
| 16KB | 10 | 128 | 1280 |
| 32KB | 10 | 128 | 1280 |
| 128KB | 7 | 192 | 1344 |
| 256KB | 4 | 352 | 1408 |

### E.4.3.2		Raptor FEC Scheme for a single sequenced packet flow

This section defines an FEC Scheme for FEC protection of a single packet flow in which source packets each carry a unique sequence number. We call such a packet flow a “sequenced flow”. A primary example would be FEC protection of an RTP flow containing an MPEG-2 Transport Stream within which all data for the service is multiplexed. In this case the RTP Sequence Numbers can be used to derive the Source FEC Packet Identification Information.

Compared to the FEC Scheme defined in Section E.4.3.1, the primary advantage of this scheme is that it does not modify source packets in any way. As a result this FEC scheme can be used in the presence of legacy equipment which would not recognize source packets which had been modified according to the schemes defined in Section E.4.3.1.

In this FEC Scheme, the role played by the Source FEC Payload ID in the scheme of Section E.4.3.1 is replaced by the sequence number. The sequence numbers of packets within each flow to be protected SHALL be incremented by one for each packet in the stream.

The size of the Source Packet Information within a given Source Block for each packet within a given sequenced flow SHALL be the same and is derived from the size of the FEC Repair packets, which SHALL also all be the same size for a given source block.

#### E.4.3.2.1	Formats and Codes

##### E.4.3.2.1.1	FEC Object Transmission Information

See Section E.4.3.1.1.1

##### E.4.3.2.1.2	FEC Payload ID

###### E.4.3.2.1.2.1	Source FEC Payload ID

The Source FEC Payload ID field is not used by this FEC Scheme. Source packets are not modified by this FEC Scheme.

###### E.4.3.2.1.2.2	Repair FEC Payload ID

The Repair FEC Payload ID format for this FEC Scheme is shown in Figure E.7 below:

1                   2                   3

0 1 2 3 4 5 6 7 8 9 0 1 2 3 4 5 6 7 8 9 0 1 2 3 4 5 6 7 8 9 0 1

+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+

|    Initial Sequence Number    |      Encoding Symbol ID       |

+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+

|      Source Block Length      |

+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+

Figure E.7: Repair FEC Payload ID format

Initial Sequence Number (Flow i ISN) – 16 bits

This field specifies the lowest 16 bits of the sequence number of the first packet to be included in this sub-block. If the sequence numbers are shorter than 16 bits then the received Sequence Number SHALL be logically padded with zero bits to become 16 bits in length respectively.

Encoding Symbol ID (ESI) – 16 bits

This field indicates which repair symbols are contained within this repair packet. The ESI provided is the ESI of the first repair symbol in the packet.

Source Block Length (SBL) – 16 bits

This field specifies the length of the source block in symbols.

#### E.4.3.2.2	Procedures

This FEC Scheme uses the procedures of the framework defined in Section E.4.2 to construct a source block to which the FEC code can be applied. The sender SHALL allocate Source Block Numbers to source blocks sequentially, wrapping around to zero after Source Block Number 216-1.

During the construction of the source block as per Section E.4.2.3.2,

-	The length indication, l[i], included in the Source Packet Information for each packet shall be dependent on the protocol that is carried. Rules for RTP are specified below in Section E.4.3.2.2.3.

-	The value of s[i] in the construction of the Source Packet Information for each packet shall be equal to the number of repair symbols placed in each repair packet, which SHALL be the same for all repair packets of a block.

##### E.4.3.2.2.1	Derivation of Source FEC Packet Identification Information

The Source FEC Packet Identification Information for a source packet is derived from the sequence number of the packet and information received in any Repair FEC packet belonging to this Source Block. Source blocks are identified by the sequence number of the first source packet in the block. This information is signaled in all Repair FEC packets associated with the source block in the Initial Sequence Number field.

The length of the Source Packet Information (in bytes) for source packets within a source block is equal to length of the payload containing encoding symbols of the repair packets (i.e. not including the Repair FEC Payload ID) for that block, which SHALL be the same for all repair packets. The Source Packet Information Length (SPIL) in symbols is equal to this length divided by the Encoding Symbol Size (which is signaled in the FEC Object Transmission Information).

The set of source packets which are included in the source block is determined from the Initial Sequence Number (ISN) and Source Block Length (SBL) as follows:

Let,

I be the Initial Sequence Number of the source block

LP be the Source Packet Information Length in symbols

LB be the Source Block Length in symbols

Then, source packets with sequence numbers from I to I +LB/LP-1 inclusive are included in the source block.

Note that if no FEC Repair packets are received then no FEC decoding is possible and it is unnecessary for the receiver to identify the Source FEC Packet Identification Information for the source packets.

The Encoding Symbol ID for a packet is derived from the following information:

The sequence number, Ns, of the packet

The Source Packet Information Length for the source block, LP

The Initial Sequence Number of the source block, I

Then the Encoding Symbol ID for packet with sequence number Ns is determined by the following formula:

ESI=(Ns−I)⋅LP

Note that all repair packet associated to a given Source Block SHALL contain the same Source Block Length and Initial Sequence Number.

##### E.4.3.2.2.2	Derivation of repair packet Encoding Symbol IDs

The Encoding Symbol ID for a repair packet indicates which repair symbols the packet contains. This is given directly by the Encoding Symbol ID field of the Repair FEC Payload ID.

##### E.4.3.2.2.3	Procedures for RTP flows

In the specific case of RTP packet flows, then the RTP Sequence Number field SHALL be used as the sequence number in the procedures described above.

The length indication included in the Source Packet Information SHALL be the RTP payload length plus the length of the CSRCs, if any, and the RTP padding bytes, if any. Note that this length is always equal to the UDP payload length of the packet, minus 12.

#### E.4.3.2.3	FEC Code specification

The requirements of section E.4.3.1 apply.

#### E.4.3.2.4	Example parameters

##### E.4.3.2.4.1	Parameter derivation algorithm

It is recommended that the algorithm of Section E.4.3.1.6.1 is used.

In the case of RTP streams carrying MPEG-2 Transport Streams, then the maximum repair packet size should be set to

P = ceil((n·188 + 15)/A)·A

Where n is the nominal number of 188 byte TS packets per IP Source packet.

The maximum source block size is determined by application configuration at the sender.

##### E.4.3.2.4.2	Examples

The above algorithm leads to transport parameters for MPEG-2 Transport Streams as shown in Table E.4 below, assuming the recommended values for A, KMIN and GMAX:

Table E.4: Example parameters settings

| Maximum packets per protection period | Nominal TS packets per IP packet | Maximum Packet Size, P | Maximum Source Block Size, B | G | Symbol size T |
| --- | --- | --- | --- | --- | --- |
| 100 | 7 | 1344 | 134400 | 7 | 192 |
| 200 | 7 | 1344 | 268800 | 4 | 336 |
| 300 | 7 | 1344 | 403200 | 3 | 672 |
| 400 | 7 | 1344 | 537600 | 2 | 672 |

# E.5		FEC Decoder

## E.5.1		Decoder requirements (normative)

### E.5.1.1	Minimum decoder requirements

FEC decoders that are compliant to this Annex shall support processing of the SMPTE 2022-1 [4] packets. This means that whenever:

(1) an SMPTE 2022-1 FEC packet has been received and

(2) all but one of the media packets protected by this FEC packet have been received within the previous max-block-size source packets and/or within a time window beginning max-block-size-time before the current time and

(3) the time at which the remaining media packet is useful to the media decoder has not passed,

then, the SMPTE 2022-1 decoding operation shall be applied and the resulting recovered packet passed to the media decoder.

Note that the above requirement applies independently of the arrival time or order of the packets involved. The parameters max-block-size  and max-block-size-time are part of the FEC Configuration Information and are discussed further in Section E.6.

### E.5.1.2	Enhanced decoder requirements

FEC decoders may additionally support Raptor FEC packets. In this case, if a receiver receives a mathematically sufficient set of encoding packets (which may include both SMPTE 2022-1 FEC packets and Raptor FEC packets) for reconstruction of a source block within the previous max-block-size source packets and/or within a time window beginning max-block-size-time before the current time then the decoder shall recover the entire source block. Note that the example decoder procedures described in Section E.5.2 fulfil this requirement and thus a decoder is compliant to this Annex only if it can successfully decode given any set of packets with which the example decoder can also decode.

## E.5.2.	Hybrid decoding procedures (informative)

### E.5.2.1	Outline

In the case that a receiver receives FEC repair packets from multiple layers, including packets generated according to the codes of both sections E.3 and E.4, then combined decoding may be provided. This section outlines procedures which may be followed to achieve this.

Combined decoding proceeds in 3 steps:

Step 1: SMPTE 2022-1 decoding

In this step, the packets encoded according to SMPTE 2022-1[4], together with the received source packets, are processed as usual to recover zero of more source packets.

Step 2: Raptor decoding

In this step, if source packets are still missing, then packets encoded according to Raptor, together with the received source packets and any source packets which were recovered in Step 1, are processed using standard Raptor decoding procedures (for example as described in [2]) to recover zero of more source packets.

Step 3: Hybrid decoding

In this step, if source packets are still missing, then remaining (unprocessed) SMPTE 2022-1 [4]  packets are converted to a form in which they can be added to the Raptor decoding process, and Raptor decoding is then continued.

Conversion of SMPTE 2022-1 packets and their use in Raptor decoding are described in the following sections.

### E.5.2.2	Conversion of SMPTE 2022-1 packets

The objective of this conversion operation of SMPTE 2022-1 packets is to convert them into a form such that they can be included in the Raptor decoding process. According to SMPTE 2022-1, each FEC packet is constructed by applying a protection operation, based on the exclusive OR operation (XOR), to a number, D, of the source packets (the “protected packets”). The UDP payload of the SMPTE 2022-1 packet contains the following data (amongst other fields):

- An FEC header containing:

  - The Length Recovery field, which is the XOR of the RTP payload lengths of the protected packets

  - The XOR of the Payload Type (PT) fields of the RTP headers of the protected packets

  - The XOR of the Timestamp (TS) fields of the RTP headers of the protected packets

- The XOR of the RTP payloads of the protected packets

In the first step of the conversion operation, the fields of each received or recovered source packet protected by a received SMPTE 2022-1 FEC packet, are XORed into the corresponding fields of the FEC packet. After this operation, the fields of the FEC packet are each equal to the XOR of the corresponding fields of the remaining (unrecovered) protected packets (which we call the “unrecovered protected packets”).

In the second step of the conversion operation, for each remaining SMPTE 2022-1 FEC packet, the following fields are concatenated to form a ‘virtual’ Raptor repair packet payload:

- A single zero byte

- A two byte length indication, which is equal to the XOR of the RTP payload lengths of the unrecovered protected packets, taken directly from the SMPTE 2022-1 FEC packet

- A two-bit field, which is equal to the XOR of the RTP Version fields of the unrecovered protected packets. This is equal to zero if the number of unrecovered protected packets is even and 2 otherwise.

- Seven (7) zero bits, corresponding to the XOR of the RTP Padding (P), Extension (X), CSRC Count (CC) and Marker (M) bits of the unrecovered protected packets, which are all required to be zero according to SMPTE 2022-1.

- A seven (7) bit field equal to the XOR of the RTP Payload Type (PT) fields of the unrecovered protected packets (taken directly from the corresponding field of the SMPTE 2022-1 FEC header).

- A 16-bit field equal to the XOR of the RTP Sequence Number fields of the unrecovered protected packets. The Sequence Numbers of the unrecovered protected packets can be explicitly calculated based on the SNbase, offset and NA values of the FEC header of the FEC packet as per SMPTE 2022-1.

- A 32-bit field equal to the XOR of the RTP Timestamp (TS) fields of the unrecovered protected packets (taken directly from the corresponding field of the SMPTE 2022-1 FEC header).

- A 32-bit field equal to the XOR of the RTP SSRC fields of the unrecovered protected packets. This is equal to zero if the number of unrecovered protected packets is even and equal to the SSRC of the stream otherwise.

- The XOR of the RTP payloads of the unrecovered protected packets, taken directly from the remainder of the SMPTE 2022-1 FEC packet

- A number of zero-valued padding bytes, such that the total length of the “virtual” repair packet payload is equal to the length of the other Raptor repair packet payloads (which are required to all be the same according to SectionE.4.3.2.5).

The resulting “virtual” repair packet payload is then equal to the XOR of the Source Packet Information of the unrecovered protected packets.

### E.5.2.3	Extension of Raptor decoding

A possible Raptor decoding algorithm is described in Section C.7 of [2] in terms of a Gaussian Elimination process upon a matrix A. If decoding is not possible without use of the SMPTE 2022-1 packets, then this decoding process will fail during the second phase described in Section C.7 of [2]. At this point, the matrix A has less than L non-zero rows (Note, the symbol L here denotes the number of intermediate symbols of the Raptor code as defined in [2], not the L value associated with the SMPTE 2022-1 packets).

Let G be the number of symbols per packet (which can be calculated as the Raptor repair packet payload size divided by the symbol size). Then each “virtual” Raptor repair packet constructed above consists of exactly G new symbols, each of which is the XOR of exactly Ns source symbols (which we call the “unrecovered protected symbols”), where Ns is the number of unrecovered protected packets associated with the SMPTE 2022-1 FEC packet from which the “virtual” Raptor repair packet was constructed.

For each such new symbol, a new row is added to the decoding matrix A. This row is constructed as follows:

- The row is initialized to zero

- For each of the Ns unrecovered protected symbols, the LTEnc generator is used to determine the set of intermediate symbols whose sum is equal to the unrecovered protected symbol. For each such intermediate symbol a ‘1’ is XORed into the appropriate position of the new row

Phase two of the decoding process is then continued with these additional rows and symbols.

# E.6	FEC Content Delivery Protocols

This section defines several complete FEC Content Delivery Protocols, making use of the components defined in the foregoing sections.

## E.6.1	Multicast MPEG-2 Transport Stream over RTP

This section defines a Content Delivery Protocol for FEC protected multicast delivery of MPEG-2 Transport Streams over RTP.

### E.6.1.1	Control protocols

FEC Configuration information SHALL be delivered using the DVB Service Discovery mechanisms as described in Section 5. The DVB Broadcast Discovery record MAY contain the multicast address(es) and port(s) for one or more FEC layers. Receivers may choose which layers to join depending on capability and local configuration.

When the Raptor layer is provided, the Flow ID within the Source Packet Information for the MPEG-2 TS flow SHALL be zero.

### E.6.1.2	Transport protocol

The MPEG-2 Transport Stream shall be transported according to clause 7.1.1.

FEC protection of the MPEG-2 Transport Stream MAY be provided according to Sections E.3 and E.4 above. When a Raptor layer is provided, the FEC Scheme defined in E.4.3.2 SHALL be used.

## E.6.2		Unicast MPEG-2 Transport Stream over RTP

This section defines a Content Delivery Protocol for FEC protected unicast delivery of MPEG-2 Transport Streams over RTP.

### E.6.2.1	Control protocols

The receiver shall indicate in the Transport header of the RTSP SETUP request which FEC layers are requested by supplying port numbers that should be used for the FEC repair packets. Only requested FEC layers shall be sent to the receiver.

The server may supply the FEC parameters max-block-size, max-block-size-time and FEC Object Transmission Information in the Transport header of the RTSP SETUP response.

The Flow ID for the MPEG-2 TS flow SHALL be zero.

### E.6.2.2	Transport protocol

The MPEG-2 Transport Stream shall be transported according to clause 7.1.1.

FEC protection of the MPEG-2 Transport Stream MAY be provided according to Sections E.3 and E.4 above. When a Raptor layer is provided, the FEC Scheme defined in E.4.3.2 SHALL be used.

## E.6.3	Generic multicast video (informative)

This section defines a Content Delivery Protocol for FEC protected multicast delivery of arbitrary audio/video streams (for example H.264 encapsulated in RTP or MPEG-2 TS encapsulated in UDP).

### E.6.3.1	Control protocols

FEC Configuration information SHALL be delivered using the DVB Service Discovery mechanisms as described in Section 5. The DVB Broadcast Discovery record MAY contain the multicast address(es) and port(s) for one or more FEC layers. Receivers may choose which layers to join depending on capability and local configuration.

### E.6.3.2	Transport protocols

The audio/video stream is assumed to be carried by one or more UDP flows. FEC protection of these UDP flows MAY be provided using the procedures of Section E.4.2.2 and in particular the FEC Scheme defined in Section E.4.3.1.

## E.6.4	Generic unicast video (informative)

This section defines a Content Delivery Protocol for FEC protected unicast delivery of arbitrary audio/video streams (for example H.264 encapsulated in RTP). This section is provided to describe how FEC can be applied to future extensions to the DVB IPI Handbook which address direct encapsulation of audio/video streams in RTP.

### E.6.4.1	Control protocols

The receiver shall indicate in the Transport header of the RTSP SETUP request which FEC layers are requested by supplying port numbers that should be used for the FEC repair packets. Only requested FEC layers shall be sent to the receiver.

The server may supply the FEC parameters max-block-size, max-block-size-time and FEC Object Transmission Information in the Transport header of the RTSP SETUP response.

### E.6.4.2	Transport protocols

The audio/video stream is assumed to be carried by one or more UDP flows. FEC protection of these UDP flows MAY be provided using the procedures of E.3 and in particular the FEC Scheme defined in Section E.4.

# E.7. Raptor explicit encoding sequences

The Raptor code defined in this Annex is defined in terms of explicit encoding operation sequences which shall be applied to generate repair symbols from source symbols.

Note: The FEC code which results from these encoding sequences is identical to that generated by the procedures described in Annex C of [2]. As a results, the example decoder procedures described in [2] may be used.

The Maximum Source Block Size used with the FEC Schemes defined in Section E.4.3 SHALL be one of the following values:

101, 120, 148, 164, 212, 237, 297, 371, 450, 560, 680, 842, 1031, 1139, 1281

Explicit encoding operation sequences are provided for each of the block sizes indicated above, supporting highly efficient implementation of encoders for the Raptor code for these block sizes.

This section describes the notation used for the encoding sequences. The encoding sequences are provided as text files attached to this specification.

Each text file consists of two parts, a “pre-coding” section and a “repair symbol encoding” section. The two sections of the file are separated by a blank line.

The encoding sequence assumes that the data to be encoded is stored in a (virtual) block of memory. Each virtual memory location stores a complete symbol. At the start of the process, the source symbols are assumed to be stored consecutively in memory locations 0 to K-1 inclusive, where K is the block size.

Additional working memory locations are required to be available up to and including memory location L-1, where L is given in the following table for each value of K. Note that the L value here is exactly the value of L calculated according to Annex C of [2]. The additional working memory shall be initialized to zero.

| K | L |
| --- | --- |
| 101 | 127 |
| 120 | 149 |
| 148 | 181 |
| 164 | 197 |
| 212 | 251 |
| 237 | 277 |
| 297 | 337 |
| 371 | 419 |
| 450 | 499 |
| 560 | 613 |
| 680 | 739 |
| 842 | 907 |
| 1031 | 1103 |
| 1139 | 1213 |
| 1281 | 1361 |

Figure E.8: Total memory requirement in symbols (L) for different block sizes

Each line of the “pre-coding” section of the text file consists of a series of memory location indices (in decimal notation), separated by spaces and each optionally preceded by the character “>”. Each line is interpreted as follows:

Let,

A	be a working register which stores one symbol

n	be the number of memory location entries on the line

mi	be the ith entry of the line, for i = 0, …, n-1

C[x]	be the symbol at memory location x

0	be the zero symbol (all bits are zero)

	be the bitwise exclusive OR operation

The following algorithm should be followed for each line in sequence:

A := 0

FOR i = 0 to n-1

IF mi is preceded by “>” THEN

C[mi] := C[mi]  A

ELSE

A := A  C[mi]

ENDIF

Each line of the “repair symbol encoding” section of the file lists the memory locations which shall be XORed together to produce a repair symbol, the first line providing the list for the repair symbol with ESI K, the second for the repair symbol with ESI K+1 etc.

For example, when included within the pre-coding section of the file, the line:

4 8 3 5 > 7 6 > 10

Would result in the following symbol assignments:

C[7] := C[7]  C[4]  C[8]  C[3]  C[5]

and

C[10] := C[10]  C[6]  C[4]  C[8]  C[3]  C[5]

<END MODIFICATION>

# 5	References

- [DVB-A086] 			DVB Blue Book A086rev4 (2007) - Transport of MPEG 2 Transport Stream (TS) Based DVB Services over IP Based Networks (DVB-IPTV Phase 1.3 - Draft TS 102 034 1.3.1), see: http://www.dvb.org/technology/bluebooks/a086r4.tm3727r1.dTS102034.V1.3.1.DVB-IP1.3.pdf-xml.zip