| Introduction INTERNATIONAL TELECOMMUNICATION UNION |  |  |  | Focus Group On IPTV |
| --- | --- | --- | --- | --- |
| TELECOMMUNICATION STANDARDIZATION SECTOR STUDY PERIOD 2005-2008 |  |  | FG IPTV-C-0825 |  |
|  |  |  | English only |  |
| WG(s): 4 |  | 5th FG IPTV meeting: Geneva, 23 – 31 July 2007 |  |  |
| CONTRIBUTION |  |  |  |  |
| Source: | T-Systems (Deutsche Telekom) |  |  |  |
| Title: | The use of Any Source and Source Specific Multicast in IPTV (Comments to IPTV Network Control Aspects) |  |  |  |

This contribution describes the differences between Any Source Multicast (ASM) and Source Specific Multicast (SSM) for IPTV. It proposes to address Source Specific Multicast in “Working Document: IPTV Network Control Aspects” as well as in the architecture and requirements documents. It might be also relevant as input to the Y.ngn-mcastf document from SG13.

# Problem Description

Currently the term “Multicast” refers to different Multicast service models (e.g. Any Source Multicast (Group based Multicast), Source Specific Multicast and Overlay Multicast). The different service models have also strong impact on the underlying network and related issues like complexity, scalability, addressing, service separation, whole sales etc.

Goal of the contribution is to clarify the description of the different IP Multicast models (namely Any Source Multicast and Source Specific Multicast) and to support Source Specific Multicast in IPTV. For clarification the contribution uses the following definitions to distinguish between the different models:

- IPTV channel refers to: Any IPTV channel independent of the Multicast service model.

- Multicast Group/Group Based Multicast refers to: Network control is based on the IP-Multicast group only.

- Multicast Channel/Channel Based Multicast refers to: Network control is based on the IP-Multicast group address and the Unicast Address of the Multicast sender.

Currently some documents mix the use of the term “channel”, “group”, “multicast service”, etc. without a clear indication which type of IP Multicast is referred to. This can lead to misinterpretation of the upcoming standards.

# Any Source Multicast (Group Based Multicast)

Any Source Multicast (ASM) is the classical variant and uses a model where all decisions are made based on the multicast group address (G) only. Devices are able to join and leave multicast groups in order to receive IPTV streams. The receivers do not have knowledge about the Multicast senders within the network. When two IP Multicast senders are sending to the same IP Multicast group address (G) and a client joins (G), the client receives traffic from both IP Multicast senders.

Receiver

Join (*, G)

Sender S1

Sender S2

Sender S3

G

G

Any Source Multicast

enabled IPTV Network

G

Figure 1: Any Source Multicast

This model has some significant drawbacks for IPTV deployment:

  - Addressing: Due to the fact that only the IP Multicast address is used it is not possible to distinguish different senders on the network level. Therefore it is necessary to implement an entity which manages the use of IP Multicast addresses. This slows the deployment down because to setup new IPTV channels or change existing IPTV channels it is necessary to obtain a free (unused) IP Multicast address.

  - Security: In a group based model it is easy to disturb existing IPTV channels by simply sending traffic to the same group address. To avoid those attacks complex filter mechanisms are needed at the ingress and/or egress points of the network (and potentially at customer’s premises to avoid unwanted and disturbing IP Multicast traffic).

  - Redundancy: On the network level Any Source Multicast usually deploys PIM-SM (Protocol Independent Multicast – Sparse Mode) a central Rendezvous Point (RP) which represents a single point of failure. To overcome this problem redundant RPs and additional protocols are necessary.

  - Complexity: Taking a-c into account (as well as some other drawbacks) the use of Any Source Multicast adds complexity. To reduce the complexity and to overcome the problems with ASM a new model “Source Specific Multicast” was developed.

# Source Specific Multicast (Group and Source Based)

Source Specific was developed to overcome the problems encountered by Any Source Multicast. The main difference between the two models is the use of the source address of the multicast sender. In contrast to ASM with SSM it is possible to distinguish between a sender (S1) and a sender (S2) sending to the same IP Multicast address (G). Receivers not only specify the group address G but also the address of the sender (S), resulting in an IP Multicast channel (S,G). Because the Unicast address within a network uniquely identifies an end system, the combination of (S,G) is unique as well. A receiver joining (S1,G) does not receive the traffic from another system sending to the same IP Multicast address.

Receiver

Join (S1, G)

Sender S1

Sender S2

Sender S3

G

G

Source Specific Multicast

enabled IPTV Network

G

Figure 2: Source Specific Multicast

Source Specific Multicast solves the following problems:

  - Addressing: It is no longer necessary to centrally coordinate the use of IP Multicast addresses. Any sender can send to any existing IP Multicast group. The network and the receivers are able to specify a particular sender using an IP Multicast group. IP Multicast channels (S,G) are handled and routed individually by the network and the clients as well.

  - Security: Denial of Service (DoS, DDoS) attacks by simply sending to a Multicast group which is already in use are not longer possible because individual senders can be distinguished.

  - Complexity: Source Specific Multicast reduces the overall complexity within IPTV. It solves several issues related to security and addressing and also removes the need for centralized Rendezvous Points.

SSM is also optimized for One-to-Many deployment scenarios which are typically used within IPTV. In addition ASM and SSM can be used in parallel (deploying different address ranges) if necessary. The use of SSM eases the deployment of IPTV and reduces the complexity, allowing more flexible IPTV services.

# Proposal (Requirements and Recommendations)

Therefore we propose the following changes/additions to the existing document “IPTV Network Control Aspects”:

- Distinguish between the different variants of IP Multicast (wherever necessary) to avoid misinterpretation of the IPTV standards.

- Provide a description of the advantages of Source Specific Multicast and add requirements for the support of Source Specific Multicast in IPTV Multicast.

_________________