| Abstract: INTERNATIONAL TELECOMMUNICATION UNION |  |  |  | Focus Group On IPTV |
| --- | --- | --- | --- | --- |
| TELECOMMUNICATION STANDARDIZATION SECTOR STUDY PERIOD 2005-2008 |  |  | FG IPTV-C-0875 |  |
|  |  |  | English only |  |
| WG(s): 4 |  | 6th FG IPTV meeting: Tokyo, Japan, 15-19 October 2007 |  |  |
| CONTRIBUTION |  |  |  |  |
| Source: | ZTE Corporation, Huawei Technologies Co., Ltd. |  |  |  |
| Title: | Functions of service management system in Broadcast TV control |  |  |  |

Requirements for Multicast control of Broadcast TV are addressed in sub clause 7.4 in the working document FG.IPTV-DOC-0123 “IPTV network control aspects”. This joint document proposes to add some text for further clarifying Functions of service management system in sub clause 7.4.

- 1. Discussion:

Broadcast TV control has been addressed in sub clause 7.4 in IPTV Network Control Aspects of WG4. However, it doesn’t describe any specific functions or requirements for service management, so it is important to analyse these requirements.

Support for service management is fundamental to the operation of the IPTV service. These functions provide the ability to manage the IPTV in order to provide IPTV services with the expected quality, security, and reliability. Service management system is recommended to support functions in managing the channel access right assignment and updating, CDR records transfer and interface for OSS, see figure 1

Figure 1

The service management system can be decomposed into the following service related management functions:

User management: This function takes the form of a functional database representing the user’s information, such as user registration info, user status info, service subscription info, in the single user profile

Policy management:

This function is used to supervise the network elements and reconfigure them in order to introduce new services. It is adopted widely for different domains such as quality of service (QoS), security, and virtual private networks (VPNs).Typical usage of policies includes:

• Device configuration: queueing mechanisms, drop strategies, access filter lists, etc.

• Traffic classification/marking (e.g. DiffServ marking).

• Resource reservation and admission control based on user identity, application type, etc.

(e.g. with RSVP: ReServation Protocol).

• Service level agreement (SLA) between two adjacent domains.

Accounting management:

This function collects accounting information, in order to provide the IPTV service provider with appropriate resource utilization data, enabling the service provider to properly bill the users of the system.

Service customization:

This function enables users to discover, acquire, and user media content, in addition to providing a broad range of features for using media.

SP/CP management:

This function is used to deliver information technology infrastructure management services to multiple customers over a network on a subscription basis and maintain content lifetime and provider related information.

Security management:

This function is used to authorize the user for appropriate service privileges.

Performance management:

This function is used to provide user related statistics and performance analysis information.

Portal management:

This function allows an organization to integrate its business information, applications, and services in web-based IPTV Portal service.

STB management:

This function is responsible for receiving, converting and sending the picture and sound of the broadcast to the associated television

- 2. Proposal:

We think the above discussion is helpful to supplement sub clause 7.4 in Working Document network control aspect. This document proposes to add some text to describe the functions of Service Management System in clause 7.4 to read as follows:

- Service Management System

- Service management system is recommended to support some service related management functions of the Broadcast TV channel, including user management, policy management, SP/CP management, accounting management, service customization, security management, performance management, portal management, STB management.

- Service management system is recommended to run independently with access node NMS or OSS

- Service management system is recommended to be able to accept all BTV channels information be pushed from OSS

- Service management system is recommended to be able to push all BTV channels information to access node

- Service management system is recommended to be able to request customer channels access rights from OSS and transmit it to access node

- Service management system is recommended to be able to transfer a customer’s channel change request to OSS

- Service management system is recommended to be able to save access node’s CDR records and transmit automatically to OSS.

- _____________