| INTERNATIONAL TELECOMMUNICATION UNION |  |  |  | Focus Group On IPTV |
| --- | --- | --- | --- | --- |
| TELECOMMUNICATION STANDARDIZATION SECTOR STUDY PERIOD 2005-2008 |  |  | FG IPTV-C-0877 |  |
|  |  |  | English only |  |
| WG(s): 4 |  | 6th FG IPTV meeting: Tokyo, Japan, 15-19 July 2007 |  |  |
| CONTRIBUTION |  |  |  |  |
| Source: | ZTE Corporation |  |  |  |
| Title: | MLD for IPV6 requirements in IPTV multicast framework |  |  |  |

Abstract

This contribution propose the support of MLD (multicast listener discovery) protocol in IPV6 scenario in IPTV multicast framework (FG IPTV-DOC-0124).

Introduction

IPV6 is suggested to be supported in IPTV architecture and multicast functional requirements have been addressed and described a lot in our working document.

However, IGMP applies only to IPv4. In IPV6 scenario, IGMP is not suitable.  Standardized MLD (Multicast Listener Discovery) protocol must be used instead.

The purpose of Multicast Listener Discovery (MLD) is to enable each IPv6 router to discover the presence of multicast listeners (that is, nodes wishing to receive multicast packets) on its directly attached links, and to discover specifically which multicast addresses are of interest to those neighbouring nodes. This information is then provided to whichever multicast routing protocol is being used by the router, in order to ensure that multicast packets are delivered to all links where there are interested receivers.

If a router has more than one interface to the same link, it need perform the router part of MLD over only one of those interfaces. Listeners, on the other hand, must perform the listener part of MLD on all interfaces from which an application or upper-layer protocol has requested reception of multicast packets.

Some important aspects of MLD are following:

- MLD uses ICMPv6 .MLD is a sub-protocol of ICMPv6, that is, MLD message types are a subset of the set of ICMPv6 messages

- MLD is an asymmetric protocol, specifying different behaviors for multicast listeners and for routers. For those multicast addresses to which a router itself is listening, the router performs both parts of the protocol, including responding to its own messages.

Proposal

Following requirements are proposed to insert in Chapter 6.1.2

- The access node is recommended to support MLDV1 or MLD v2in IPV6 scenario.

- The access node is recommended to support MLD snooping and can optionally support MLD proxy.

Reference

[IETF RFC 2710] 		IETF RFC 2710 (1999), Multicast Listener Discovery (MLD) for IPv6

[IETF RFC 3810] 		IETF RFC 3810(2004), Multicast Listener Discovery Version 2 (MLDv2) for

IPv6

[IETF RFC 4604] 		IETF RFC 4604(2006), Using Internet Group Management Protocol Version 3 (IGMPv3) and Multicast Listener Discovery Protocol Version 2 (MLDv2) for Source-Specific Multicast.

[IETF RFC 4541] 		IETF RFC 4541(2006), Considerations for Internet Group Management Protocol (IGMP) and Multicast Listener Discovery (MLD) Snooping Switches.

[IETF RFC 4605] 		IETF RFC 4605(2006), Group Management Protocol (IGMP) / Multicast Listener Discovery (MLD)-Based Multicast Forwarding ("IGMP/MLD Proxying").

_______________