| Introduction INTERNATIONAL TELECOMMUNICATION UNION |  |  |  | Focus Group On IPTV |
| --- | --- | --- | --- | --- |
| TELECOMMUNICATION STANDARDIZATION SECTOR STUDY PERIOD 2005-2008 |  |  | FG IPTV-C-0895 |  |
|  |  |  | English only |  |
| WG(s): 3 |  | 6th FG IPTV meeting: Tokyo, Japan 15-19 October 2007 |  |  |
| CONTRIBUTION |  |  |  |  |
| Source: | Huawei Technologies Co., Ltd. |  |  |  |
| Title: | Alignment of with WG1 on terminologies for section 10.2.2 |  |  |  |

This contribution proposes some modification on section 10.2.2 to keep alignment with WG1.

# Discussion

During the 5th Geneva meeting, solid progresses were made on IPTV architecture. The high level architecture and the decomposition of some common functional entities were developed.

Specifically for IPTV applications, it was further decomposed into Service Discovery and Selection, Leaner TV application, On-Demand application etc. Currently the term IPTV application is used in section 10.2.2 of Document 122, while this section aims to address authorization for LTV service specifically. It is proposed to change IPTV application to LTV application to keep alignment with WG1 architecture decomposition and to reflect its intention. Further elaboration on right information is done also to solve the following two editor’s notes:

[Editor’s Note1: Need to keep alignment with WG1 architecture’s change.]

[Editor’s Note2: Need to elaborate the ‘Right Information’.]

# Proposal

Base on the discussion in chapter 2,it is proposed to modify section 10.2.2 as follows::

### 10.2.2 Service Authorization

- 10.2.2.1 Service Authorization for Linear TV service

The basic mechanism to authorize the Linear TV services is as follows:

    - The LTV application sends the rights information related with a particular user to the transport functions to which terminal is connected. The transport functions store the received rights information. The rights information includes the channel(es) a user can access.

    - When receiving a user request, the transport functions perform the service authorization procedures according to the stored rights to determine whether to grant access to the user.

The procedure is depicted below in  figure 10-1, defined as follows:

Figure ‎3-10-1 Service Authorization Information Flow

- User Subscribes or requests a TV channel via Service Control Functions.

- The LTV application gets the rights related with the subscribed channel from Application Profile Function, and sends the rights information to the transport functions via transport control functions. The transport functions store the rights information.

- The LTV application sends a response back to the terminal.

- The user requests a TV channel by sending a message, e.g. IGMP join, to the transport functions.

- When rights are not available or changed, the transport functions request LTV application via IPTV Service Control Function for the rights in real-time, and may store them locally. This step is not needed in case where the rights are available, therefore, shown as dotted lines.

- The transport functions perform service authorization according to the rights information.

- Based on the authorization, the transport functions grant access or deny the request

__________