| Introduction INTERNATIONAL TELECOMMUNICATION UNION |  |  |  | Focus Group On IPTV |
| --- | --- | --- | --- | --- |
| TELECOMMUNICATION STANDARDIZATION SECTOR STUDY PERIOD 2005-2008 |  |  | FG IPTV-C-0901 |  |
|  |  |  | English only |  |
| WG(s): 1 |  | 6th FG IPTV meeting: Tokyo, Japan, 15-19 October 2007 |  |  |
| CONTRIBUTION |  |  |  |  |
| Source: | Huawei Technologies Co., Ltd. |  |  |  |
| Title: | Improvement on section 8.1.2.1 on description of IPTV application functional block |  |  |  |

This contribution proposed to improve the descriptions to IPTV applications in clause 8.1.2.1 and 8.3.1.1 respectively.

# Discussion

During the 5th Geneva meeting, solid progresses were made on IPTV architecture. The decomposition of Application Functions, including IPTV applications, Content Preparation etc., was developed.

While the current description on IPTV applications functional block in high level architecture (see clause 8.1.2.1) covers only part of the actual functions of this block, most of the functions mentioned in clauses 8.3.1.1.1, 8.3.1.1.2, 8.3.1.1.3 and 8.3.1.1.4 are not covered there.

The description in clause 8.1.2.1 is as follows:

#### 8.1.2.1 IPTV applications functional block

Provides the server side functions of the IPTV Applications. One of the roles of this functional block is to allow the IPTV terminal to select, and purchase if necessary an item of content.

Some other descriptions in clauses 8.3.1.1.1, 8.3.1.1.2, 8.3.1.1.3 and 8.3.1.1.4 are as follows:

- In clause 8.3.1.1.1, it states the SD&S components “provide for the discovery and selection of the available IPTV services”.

- In clause 8.3.1.1.2, it says the CoD components “perform session management, service authorization, presentation of the content metadata, and execution of the service logic of the On-Demand service”.

- Clause 8.3.1.1.3 has the similar description for LTV service.

- In clause 8.3.1.1.4, it says other applications “provide for the delivery and presentation of additional IPTV services and their content, e.g. games, distant learning etc”.

- It also states in clause 8.3.1.1 that “All application functions may communicate with the Application Profile Function to support the personalisation of the IPTV services”

Therefore, in order to have an overview on the IPTV application functional block, above functions should be reflected to the description in clause 8.1.2.1.

# Proposal

It is proposed to modify the clause 8.1.2.1 as follows:

#### 8.1.2.1 IPTV applications functional block

Provides the server side functions of the IPTV Applications. One of the roles of this functional block is to allow the IPTV terminal to select, and purchase if necessary an item of content.

When receiving requests from IPTV Terminal Function, it performs application authorization and execution of IPTV service logic based on user profile, content metadata and other information retrieved from relevant entities. It also communicates with Content Delivery Functions and prepares for the delivery of media content to IPTV Terminal Function through Content Delivery Functions.

_____________