| Abstract INTERNATIONAL TELECOMMUNICATION UNION |  |  |  | Focus Group On IPTV |
| --- | --- | --- | --- | --- |
| TELECOMMUNICATION STANDARDIZATION SECTOR STUDY PERIOD 2005-2008 |  |  | FG IPTV-C-0955 |  |
|  |  |  | English only |  |
| WG(s): 4 |  | 6th FG IPTV meeting: Tokyo, Japan, 15-19 October 2007 |  |  |
| CONTRIBUTION |  |  |  |  |
| Source: | NTT Corporation, NEC Corporation |  |  |  |
| Title: | Comments on “IPTV Network Control Aspects” (FG IPTV-DOC-0123) |  |  |  |

Proposal for amending IPTV Network Control Aspects (FG IPTV-DOC-0123) for alignment to its scope.

# 1. Introduction

The scope of WG4 was discussed at the fourth FG IPTV meeting in Bled. WG4 agreed to work on low-level requirements to cover overall aspects of network control of IPTV inline with high-level requirements which were jointly developed in WG1. “IPTV Network Control Aspects” should be developed as a result of this work. It seems appropriate that WG4 concentrates on functional requirements as WG1 works on service requirements for IPTV. The scope section in the working document indicates to provide the list of requirements to address control and signalling related to IPTVnetworks. However, the current working document seems to contain a kind of implementation description beyond the agreed scope to list the requirements.

# 2. Discussion

One of the reasons for the confusion about the network control working document is that the relationships among Multicast, Broadcast, and Content Distribution are unclear. Though discussion with WG1 is necessary, the following clarification might be useful for discussiion.

- Content Distribution is a general function for the IPTV service. The IPTV service is provided with Content Distribution under the Content Distribution control.

- Broadcast is Content Distribution where the user’s control is limited passive control.

- Multicast is a mechanism to achievefficient content distribution flow from a single content source to multiple content destinations.

- Unicast is a mechanism to achieve content distribution flow from a single content source to a single content destination.

If the clarification is accepted, almost of the requirements in the current working document could be generalized and moved to the Content Distribution section, and the specific requirements applicable only for each of Broadcast, Multicast and Unicast could be specialized and retained in each section.

In any case, content describing implementation detailsis beyond the agreed scope. Such materials should be deleted or moved to the appropriate document. If such material is related to Multicast, that could be moved to the Multicast Framework document. If we cannot delete some texts, then some text might be move to the appendix for retaining useful information.

# 3. Proposal

Considering the scope of the document, contents beyond the scope such as implementation details should be moved to the appendix or deleted. Proposed dispositions of Working Document: IPTV Network Control Aspects are attached.

____________

Attachment

List of materials to be moved or deleted

# 1 Scope

# 2 References

# 3 Definitions

# 4 Abbreviations

# 5 Conventions

# 6 Framework of IPTV network control aspects

# 7 Control and signalling aspects

## 7.1 Network Control

### 7.1.2 Unicast Network Control

### 7.1.2 Multicast Network Control

The first paragraph of this section is appropriate as a functional requirement because the necessity multicast functionality and its control mechanism are important for the IPTV service. However, the following descriptions should be reconsidered item by item. The description of configuration details of multicast functionality is an implementation matter and should be deleted from the requirement document. The description of the user authentication function is appropriate as a functional requirement. The description of the selection of a multicast scheme is an implementation matter and should be deleted from the requirement document. The description of a multicast protocol proxy of an access network is an implementation matter and should be deleted.

#### 7.1.2.1 Multicast IP address management

- We are not sure whether multicast IP address management is a functional requirement. Careful study of how the multicast IP address is used for the IPTV service is needed. Multicast IP address management is too detailed to handle in a network control document and should be described in the Multicast Working Document.

#### 7.1.2.2 Connection Admission Control

- The necessity of CAC (Connection Admission Control) for multicasting is appropriate as a functional requirement for IPTV network control. However, the following descriptions should be handled in the Multicast Working Document.

#### 7.1.2.3 Attachment control network

Content here should be moved to Multicast Working Document.

#### 7.1.2.4 Multicast session identifier management

Content here should be moved to Multicast Working Document.

#### 7.1.2.5 Source Specific Multicast

Content here should be moved to Multicast Working Document.

## 7.2 Multicast Availability

Content here is applicable to multicast and unicast. We expect to generalize applicable content to unicast. In this case, the title of this section should be changed to “IPTV Network Availability”, for example.

### 7.2.1 Multicast Service failure recovery

Content here should be modified with its section title to that suitable for IPTV network failure recovery.

#### 7.2.1.1 Redundant system architecture & topology

#### 7.2.1.2 Robust network against Attacks

## 7.3 Multicast security Requirements

Content and the section title here should be generalized so that those are applicable to IPTV network security requirements.

## 7.4 Broadcast TV Control

- Description of access control is appropriate as functional requirement for IPTV network control. However, the other descriptions are only examples of controls, as described after “such as”.

## 7.5 Session Control

## 7.6 Stream Control

# 8 Content distribution aspects

- The description in this section is not clear. Clarifying the position of content distribution is necessary. Content distribution might be explained as a generalized network function for IPTV. Even if that is the case, content segmentation is not a requirement and should be moved to the appendix or deleted.

## 8.1 Content distribution network topology

Content here is not a requirement but an implementation specification. These should be moved to the appendix or deleted.

### 8.1.1 CDN-based IPTV media delivery mechanism

Manageability and interoperability are appropriate as requirements, but others are implementation hints and should be moved to the appendix or deleted.

### 8.1.2 Distributed content storage/cache and content serving

Content storage and/or cache serving are appropriate as a requirement, but distributed content serving is a kind of implementation and should be moved to the appendix or deleted, even if that is a kind of requirement, it is an optional requirement.

### 8.1.3 Centralized Content Location Management

Content here does not give requirements but implementation specifications. These should be moved to the appendix or deleted.

### 8.1.4 Content Distribution Protocols

Appropriate as functional requirements.

### 8.1.5 Statistical performance of Content Delivery Network

Appropriate as functional requirements.

## 8.2 Distributed Content Distribution.

Content here does not give requirement but implementation specifications. These should be moved to the appendix or deleted.

## 8.3 Distributed Edge Server Networks

Content here dies not give requirements but implementation specifications. These should be moved to the appendix or deleted.

# 9 IPTV Consumer Domain Attachment and Initialization

Appropriate as functional requirements.

# 10 Identification aspects

Content in this section is appropriate as functional requirements. Alignment with WG1 results is necessary.

# 11 Home, Access and Core network aspects

- Almost all content of this section should be handled in WG5. Only the hook point for providing access for the end system should be described at this section.

# 12 Network control aspects for non-NGN

# 13 Network control aspects for NGN

# 14 Network control aspects for IMS-based NGN

# 15 IPTV Inter-working

This section is an important part of the network control requirement document. However, this might be difficult to fix. FFS is sufficient in this stage.

# 16 Overlay Network

The overlay network is an example implementation for achieving network functionality such as multicasting. This is not a functional requirement and is beyond the scope of this working document. We propose to move this section to the appendix because the content is useful as an implementation suggestion.

# 17 Other aspects

Clarifying item by item whether the content is a requirement or implementation matter is necessary.

- Multicast VPN

We should confirm whether this item is a service requirement.

- Parental controls

Appropriate as functional requirements.

- Various home, access and core transport scenarios for multicasting

Not appropriate as functional requirements.

- Any other issues related to IPTV Network control aspects

We need to clarify item by item whether the content is a requirement or implementation matter.

_________________