| Introduction INTERNATIONAL TELECOMMUNICATION UNION |  |  |  | Focus Group On IPTV |
| --- | --- | --- | --- | --- |
| TELECOMMUNICATION STANDARDIZATION SECTOR STUDY PERIOD 2005-2008 |  |  | FG IPTV-C-0968 |  |
|  |  |  | English only |  |
| WG(s): 6 |  | 6th FG IPTV meeting: Tokyo, Japan, 15-19 October 2007 |  |  |
| CONTRIBUTION |  |  |  |  |
| Source: | NTT Corporation |  |  |  |
| Title: | Proposal on the Clause 5.2.5 of Working Document: IPTV Multimedia Application Platforms (FG IPTV-DOC-0146) |  |  |  |

This contribution proposes text to be added in FG IPTV-DOC-0146.

- Discussion

Background

The specification of ISDB-T multimedia coding scheme was first based on that of MHEG. But since March 1999, ARIB started developing an XML specification for multimedia coding scheme based on the functionalities of MHEG. This contributed to the development of an XML application language, BML (Broadcast Markup Language) that enables the presentation of an XML file on BML browsers. The ARIB data broadcasting system standards were enacted as STD-B24 in October 1999.

BML application can be based on synchronization and spatial-temporal relationships of multiple media formats, structural composition, event-action associations, navigation, and user interaction capabilities just as MHEG (see 5.3.1). BML consists of XMTML, CSS, DOM and ECMAScript, just as most Presentation Engines, and a difference is an application without using an execution engine. BML define additional API and object for broadcasting using ECMAScript.

The semantics of BML is based on the following minimum set of MHEG classes:

• Application Class

• Scene Class

• Link Class

• Action Class

|  | Presentation Engine (PE) |  | Execution Engine(EE) | Mono-media |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| ARIB (Japan) | Spec. | BML | Java (Under Study) | Video | Graphics | Audio | Character |
|  | Components | XHTML1.0 CSS1/2 DOM1/2 ECMAScript |  | MPEG-2 MPEG-1 MPEG-4 | JPEG MPEG-2 I PNG MNG | MPEG-2 AIFF-C | EUC-JP (8bit) |
| ATSC-DBASE (Terrestrial in US) | Spec. | XDML | Java | MPEG-2 MNG | JPEG PNG | AC-3 PCM | UTF-8 |
|  | Components | XHTML1.0 CSS2 DOM2 |  |  |  |  |  |
| DVB-MHP (Europe) | Spec. | DVB-HTML | DVB-J | MPEG-2 | JPEG MPEG-2 I PNG | MPEG-1 | UTF-8 |
|  | Components | XHTML1.0 CSS2, DOM2 ECMAScript | Java |  |  |  |  |
| CableLabs (CATV in US) | Spec. | OCAP HTML | Java | MPEG-2 | JPEG MPEG-2 I PNG, GIF AnimatedGIF SWF | MPEG-1 AC3 WAVE MS-PCM Format | UTF-8 |
|  | Components | HTML1.0 CSS2 DOM1/2 ECMAScript |  |  |  |  |  |

Table1. Data Broadcasting Method Comparison with Japan, US and Europe

- Proposal

It is proposed that the above text be added to clause 5.2.5 and Table1 to Appendix of Working Document: IPTV Multimedia Application Platforms, FG IPTV-DOC-0146.

______________