| INTERNATIONAL TELECOMMUNICATION UNION |  |  |  | Focus Group On IPTV |
| --- | --- | --- | --- | --- |
| TELECOMMUNICATION STANDARDIZATION SECTOR STUDY PERIOD 2005-2008 |  |  | FG IPTV-C-1029 |  |
|  |  |  | English only |  |
| WG(s): 6 |  | 7th FG IPTV meeting: Qawra, St Paul’s Bay, Malta, 11– 18 December 2007 |  |  |
| CONTRIBUTION |  |  |  |  |
| Source: | KDDI Corporation |  |  |  |
| Title: | Revised tables on digital video formats |  |  |  |

In the last meeting in Tokyo, the joint working group of WG 5 and WG 6 agreed that the working document on video codecs of WG 6 incorporated the contribution on digital video formats (FG IPTV-C-0925) as an appendix. This contribution supersedes the previous contribution with some editorial corrections. In addition, this contribution is provided as a Microsoft Word document for more smooth editing work.

Please note that the picture formats for VC-1 and AVS are not described in this document. Contributions are still requested from the countries that are responsible for these compression coding methods.

References (not changed)

[1] ANSI/SCTE 43 (2005)

[2] SCTE DVS 683r3 (Draft edition, Nov. 2006)

[3] ETSI TS 101 154 v1.8.1 (Jul. 2007)

[4] ARIB STD-B32 (Mar. 2007)

|  |  |  | US (SCTE) |  |  | Europe (DVB) |  |  | Japan (ARIB) |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Display aspect ratio | P/I | asp_idc | Frame rate (Hz) | Level | asp_idc | Frame rate (Hz) | Level | asp_ idc | Frame rate (Hz) | Level |
| 1920 x 1080 | 16:9 | P | 1 | 23.976, 24, 29.97, 30 | 4 | 1 | 23.976, 24, 25 | 4 |  |  |  |
| 1920 x 1080 | 16:9 | I | 1 | 29.97, 30 | 4 | 1 | 25, 29.97, 30 | 4 | 1 | 29.97 | 4 |
| 1440 x 1080 | 16:9 | P | 14 | 23.976, 24, 29.97, 30 | 4 | 14 | 23.976, 24, 25 | 4 |  |  |  |
| 1440 x 1080 | 16:9 | I | 14 | 29.97, 30 | 4 | 14 | 25, 29.97, 30 | 4 | 255 or 14 | 29.97 | 4 |
| 1280 x 1080 | 16:9 | P |  |  |  | 15 | 23.976, 24, 25 | 4 |  |  |  |
| 1280 x 1080 | 16:9 | I |  |  |  | 15 | 25, 29.97, 30 | 4 |  |  |  |
| 960 x 1080 | 16:9 | P |  |  |  | 16 | 23.976, 24, 25 | 4 |  |  |  |
| 960 x 1080 | 16:9 | I |  |  |  | 16 | 25, 29.97, 30 | 4 |  |  |  |
| 1280 x 720 | 16:9 | P | 1 | 23.976, 24, 29.97, 30, 59.94, 60 | 4 | 1 | 23.976, 24, 25, 29.97, 30, 50, 59.94, 60 | 4 | 1 | 59.94 | 4 |
| 960 x 720 | 16:9 | P |  |  |  | 14 | 23.976, 24, 25, 29.97, 30, 50, 59.94, 60 | 4 |  |  |  |
| 640 x 720 | 16:9 | P |  |  |  | 16 | 23.976, 24, 25, 29.97, 30, 50, 59.94, 60 | 4 |  |  |  |

Picture encoding format for H.264 HDTV

Profile for H.264

SCTE: High Profile for both HDTV and SDTV

DVB:  High Profile for HDTV, Main Profile for SDTV

Japan:  High Profile for HDTV, Main for SDTV (High can be used for SDTV)

Picture encoding format for H.264 SDTV in 30 Hz system

|  |  |  | US (SCTE) |  |  | Europe (DVB) |  |  | Japan (ARIB) |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Display aspect ratio | P/I | asp_idc | Frame rate (Hz) | Level | asp_ idc | Frame rate (Hz) | Level | asp_ idc | Frame rate (Hz) | Level |
| 720 x 480* | 4:3 16:9 | P | 3 5 | 23.976, 24, 29.97, 30, 59.94, 60, | 4 3 | 3 5 | 23.976, 24, 29.97, 30 | 3 |  |  |  |
| 720 x 480* | 4:3 16:9 | I | 3 5 | 29.97, 30 | 3 | 3 5 | 29.97, 30 | 3 | 3 5 | 29.97 | 3 or 3.1 or 3.2 |
| 640 x 480 | 4:3 16:9 | P | 1 | 23.976, 24, 29.97, 30, 59.94, 60 | 4 3 | 1 11 | 23.976, 24, 29.97, 30 | 3 |  |  |  |
| 640 x 480 | 4:3 16:9 | I | 1 | 29.97, 30 | 3 | 1 11 | 29.97, 30 | 3 |  |  |  |
| 544 x 480** | 4:3 16:9 | P | 5 | 23.976 | 3 | 5 13 | 23.976, 24, 29.97, 30 | 3 |  |  |  |
| 544 x 480** | 4:3 16:9 | I | 5 | 29.97 | 3 | 5 13 | 29.97, 30 | 3 |  |  |  |
| 480 x 480 | 4:3 16:9 | P |  |  |  | 11 7 | 23.976, 24, 29.97, 30 | 3 |  |  |  |
| 480 x 480 | 4:3 16:9 | I |  |  |  | 11 7 | 29.97, 30 | 3 |  |  |  |
| 352 x 480 | 4:3 16:9 | P | 7 | 23.976 | 3 | 7 9 | 23.976, 24, 29.97, 30 | 3 |  |  |  |
| 352 x 480 | 4:3 16:9 | I | 7 | 29.97 | 3 | 7 9 | 29.97, 30 | 3 |  |  |  |
| 352 x 240 | 4:3 16:9 | P |  |  |  | 3 5 | 23.976, 24, 29.97, 30, 59.94, 60 | 3 |  |  |  |
| 352 x 240 | 4:3 16:9 | I |  |  |  | 3 5 | 29.97, 30 | 3 |  |  |  |

*704 x 480 is also included in SCTE definition.

**528 x 480 is also included in SCTE definition.

Picture encoding format for H.264 SDTV in 25 Hz system

|  |  |  | US (SCTE) |  |  | Europe (DVB) |  |  | Japan (ARIB) |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Display aspect ratio | P/I | asp_idc | Frame rate (Hz) | Level | asp_idc | Frame rate (Hz) | Level | asp_idc | Frame rate (Hz) | Level |
| 720 x 576 | 4:3 16:9 | P |  |  |  | 2 4 | 25 | 3 |  |  |  |
| 720 x 576 | 4:3 16:9 | I |  |  |  | 2 4 | 25 | 3 |  |  |  |
| 544 x 576 | 4:3 16:9 | P |  |  |  | 4 12 | 25 | 3 |  |  |  |
| 544 x 576 | 4:3 16:9 | I |  |  |  | 4 12 | 25 | 3 |  |  |  |
| 480 x 576 | 4:3 16:9 | P |  |  |  | 10 6 | 25 | 3 |  |  |  |
| 480 x 576 | 4:3 16:9 | I |  |  |  | 10 6 | 25 | 3 |  |  |  |
| 352 x 576 | 4:3 16:9 | P |  |  |  | 6 8 | 25 | 3 |  |  |  |
| 352 x 576 | 4:3 16:9 | I |  |  |  | 6 8 | 25 | 3 |  |  |  |
| 352 x 288 | 4:3 16:9 | P |  |  |  | 2 4 | 25, 50 | 3 |  |  |  |
| 352 x 288 | 4:3 16:9 | I |  |  |  | 2 4 | 25 | 3 |  |  |  |

Picture encoding format for MPEG-2 HDTV

|  |  |  | US (SCTE) |  |  | Europe (DVB) |  |  | Japan (ARIB) |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Display aspect ratio | P/I | asp_idc | Frame rate (Hz) | Level | asp_idc | Frame rate (Hz) | Level | asp_idc | Frame rate (Hz) | Level |
| 1920 x 1080 | 16:9 | P | 1,3 | 23.976, 24, 29.97, 30 | HL | 3 | 23.976, 24, 29.97, 30 | HL |  |  |  |
| 1920 x 1080 | 16:9 | I | 1,3 | 29.97, 30 | HL | 3 | 29.97, 30 | HL | 3 | 29.97 | HL |
| 1440 x 1080 | 16:9 | P | 3 | 23.976, 24, 29.97, 30 | HL |  |  |  |  |  |  |
| 1440 x 1080 | 16:9 | I | 3 | 29.97, 30 | HL |  |  |  | 3 | 29.97 | HL |
| 1280 x 720 | 16:9 | P | 1,3 | 23.976, 24, 29.97, 30, 59.94, 60 | HL | 3 | 23.976, 24, 25, 29.97, 30, 50, 59.94, 60 | HL | 3 | 59.94 | HL |

Profile for MPEG-2 video

SCTE: Main Profile for HDTV and SDTV

DVB: Main Profile for HDTV and SDTV

ARIB: Main Profile for HDTV and SDTV

Picture encoding format for MPEG-2 SDTV in 30 Hz system

|  |  |  | US (SCTE) |  |  | Europe (DVB) |  |  | Japan (ARIB) |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Display aspect ratio | P/I | asp_ idc | Frame rate (Hz) | Level | asp_ idc | Frame rate (Hz) | Level | asp_ idc | Frame rate (Hz) | Level |
| 720 x 480* | 4:3 16:9 | P | 2 3 | 23.976, 24, 29.97, 30, 59.94, 60 | H14L | 2 3 | 23.976, 24, 29.97, 30, 59.94, 60 | H14L | N.A. 3 | 59.94 | H14L |
| 720 x 480* | 4:3 16:9 | I | 2 3 | 29.97, 30 | ML | 2 3 | 29.97,30 | ML | 2 3 | 29.97 | ML |
| 640 x 480 | 4:3 | P | 1,2 | 23.976, 24, 29.97, 30, 59.94, 60 | ML | 2 | 23.976, 24, 29.97, 30 | ML |  |  |  |
| 640 x 480 | 4:3 | I | 1,2 | 29.97, 30 | ML | 2 | 29.97, 30 | ML |  |  |  |
| 544 x 480** | 4:3 16:9 | P | 2 | 23.976 | ML | 2 3 | 23.976, 29.97 | ML |  |  |  |
| 544 x 480** | 4:3 16:9 | I | 2 | 29.97 | ML | 2 3 | 29.97 | ML | 2 3 | 29.97 | ML |
| 480 x 480 | 4:3 16:9 | P |  |  |  | 2 3 | 23.976, 29.97 | ML |  |  |  |
| 480 x 480 | 4:3 16:9 | I |  |  |  | 2 3 | 29.97 | ML | 2 3 | 29.97 | ML |
| 352 x 480 | 4:3 16:9 | P | 2 | 23.976 | ML | 2 3 | 23.976, 29.97 | ML |  |  |  |
| 352 x 480 | 4:3 16:9 | I | 2 | 29.97 | ML | 2 3 | 29.97 | ML |  |  |  |
| 352 x 240 | 4:3 16:9 | P |  |  |  | 2 3 | 23.976, 29.97 | ML | 2 3 | 29.97 | LL |

*704 x 480 is also included in SCTE definition.

**528 x 480 is also included in SCTE definition

Picture encoding format for MPEG-2 SDTV in 30 Hz system

|  |  |  | US (SCTE) |  |  | Europe (DVB) |  |  | Japan (ARIB) |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Display aspect ratio | P/I | asp_ idc | Frame rate (Hz) | Level | asp_ idc | Frame rate (Hz) | Level | asp_ idc | Frame rate (Hz) | Level |
| 720 x 576 | 4:3 16:9 | P |  |  |  | 2 3 | 25, 50 | H14L |  |  |  |
| 720 x 576 | 4:3 16:9 | I |  |  |  | 2 3 | 25 | ML |  |  |  |
| 544 x 576 | 4:3 16:9 | P |  |  |  | 2 3 | 25 | ML |  |  |  |
| 544 x 576 | 4:3 16:9 | I |  |  |  | 2 3 | 25 | ML |  |  |  |
| 480 x 576 | 4:3 16:9 | P |  |  |  | 2 3 | 25 | ML |  |  |  |
| 480 x 576 | 4:3 16:9 | I |  |  |  | 2 3 | 25 | ML |  |  |  |
| 352 x 576 | 4:3 16:9 | P |  |  |  | 2 3 | 25 | ML |  |  |  |
| 352 x 576 | 4:3 16:9 | I |  |  |  | 2 3 | 25 | ML |  |  |  |
| 352 x 288 | 4:3 16:9 | P |  |  |  | 2 3 | 25 | ML |  |  |  |

________________