| INTERNATIONAL TELECOMMUNICATION UNION |  |  |  | Focus Group On IPTV |
| --- | --- | --- | --- | --- |
| TELECOMMUNICATION STANDARDIZATION SECTOR STUDY PERIOD 2005-2008 |  |  | FG IPTV-C-1047 |  |
|  |  |  | English only |  |
| WG(s): 2 |  | 7th FG IPTV meeting: Qawra, St Paul’s Bay, Malta, 11 – 18 December 2007 |  |  |
| CONTRIBUTION |  |  |  |  |
| Source: | Pixelmetrix Corporation |  |  |  |
| Title: | Suggested modifications to FG-IPTV-DOC-0154 “Performance monitoring for IPTV” |  |  |  |

Introduction: This contribution presents a modification to the document FG-IPTV-DOC-0154 “Performance monitoring for IPTV”.

# Proposal: Replace Table 7-1 with the following table.

The proposed table contains the complete list of monitoring parameters. The major differences between the current and the proposed tables are as follows:

- New entries have been added to correlate with the parameters described in Section 7. The table thus provides a better view of all the monitoring parameters across the various points. The table should be updated whenever new parameters or monitoring points are added.

- A host of measurements are also removed from Point 5 as they are not applicable. These relate to the physical layer, IP layer and transport layer parameters. Point 5 refers to measurements after the set top box. The following parameters are therefore not relevant:

  - Packet Loss Metrics

  - Packet Jitter

  - IGMP Join/Leave Time

  - IP Flow List

  - TR 101 290

- Service Line-Up Parameters:

  - Buffering delay and decoding delay entries have been removed. They have no utility in IPTV monitoring. IGMP join/leave times and Channel zap times are more useful parameters.

- The Channel Attributes Parameters:

  - The Blackout and freeze frame detection entries have been removed and are redefined as one of the Video Quality parameters.

  - The Audio loss/presence entry has been removed and is redefined as one of the Audio Quality parameters.

  - The Video/Audio Bandwidth entry has been removed. This parameter should fall under the Channel Attributes parameter.

| ITU Monitoring Points | PT1 | PT2 | PT3 | PT4 | PT5 |
| --- | --- | --- | --- | --- | --- |
| Physical Layer Parameters |  |  |  |  |  |
| RF Integrity |  |  |  |  |  |
| Available Bandwidth |  |  |  |  |  |
| IP Layer/Network Parameters |  |  |  |  |  |
| Packet Loss |  Only for content over IP contribution |  |  |  |  |
| Packet Jitter |  Only for content over IP contribution |  |  |  |  |
| IGMP Join/Leave |  |  |  |  |  |
| IP Flow List |  |  |  |  |  |
| Link IP Layer Used Bandwidth |  |  |  |  |  |
| Link IP Layer Avail Bandwidth |  |  |  |  |  |
| End-to-End IP Layer Bandwidth |  |  |  |  |  |
| End-to-End IP Layer Avail Bandwidth |  |  |  |  |  |
| Loss Run Length Distribution |  |  |  |  |  |
| Error Free Interval Distribution |  |  |  |  |  |
| Other Packet Loss Metrics & Models |  |  |  |  |  |
| Streaming Jitter |  |  |  |  |  |
| Packet Loss Rate |  |  |  |  |  |
| Out of Order Packet Rate |  |  |  |  |  |
| Burst Loss Rate |  |  |  |  |  |
| Gap Loss Rate |  |  |  |  |  |
| Mean Gap Length |  |  |  |  |  |
| Mean RTP Burst Length |  |  |  |  |  |
| Loss Period Count |  |  |  |  |  |
| IP Maximum Loss Period |  |  |  |  |  |
| Retransmissions |  |  |  |  |  |
| Transport Layer Parameters |  |  |  |  |  |
| TR 101 290 P1 |  |  |  |  |  |
| TR 101 290 P2 |  |  |  |  |  |
| TR 101 290 P3 |  |  |  |  |  |
| Service Line-Up Parameters |  |  |  |  |  |
| Channel Line-Up |  |  |  |  |  |
| Service Meta-Data |  |  |  |  |  |
| Channel Zap Time |  |  |  |  |  |
| Correctness Rate |  |  |  |  |  |
| Connection Success Rate |  |  |  |  |  |
| Connection Time |  |  |  |  |  |
| Channel Attributes Parameters |  |  |  |  |  |
| Channel Attributes |  |  |  |  |  |
| Video Quality |  |  |  |  |  |
| Audio Quality |  |  |  |  |  |
| Ancillary Channel Assoc. Attributes |  |  |  |  |  |
| VOD Parameters |  |  |  |  |  |
| VOD Request Performance |  |  |  |  |  |
| Other Parameters |  |  |  |  |  |
| AAA Success Rate |  |  |  |  |  |
| STB Booting Time |  |  |  |  |  |

_____________