| Introduction: This contribution presents several suggested changes to the document FG-IPTV-DOC-0154 “performance monitoring for IPTV” INTERNATIONAL TELECOMMUNICATION UNION |  |  |  | Focus Group On IPTV |
| --- | --- | --- | --- | --- |
| TELECOMMUNICATION STANDARDIZATION SECTOR STUDY PERIOD 2005-2008 |  |  | FG IPTV-C-1048 |  |
|  |  |  | English only |  |
| WG(s): 2 |  | 7th FG IPTV meeting: Qawra, St Paul’s Bay, Malta, 11 – 18 December 2007 |  |  |
| CONTRIBUTION |  |  |  |  |
| Source: | Pixelmetrix Corporation |  |  |  |
| Title: | Suggested modifications to document FG-IPTV-DOC-0154 “Performance monitoring for IPTV” |  |  |  |

Proposal 1: Addition of the monitoring parameter “Blackout Detection” and “Freeze Frame Detection” to the section 7.5.2 “Video Quality”

### Blackout Detection

Blackout is defined as the loss of video for a particular service. The detection is done by analyzing the spatial variation in the picture after decoding the elementary stream. If the spatial variation is less than a certain threshold for a period of time, a blackout is considered to have occurred.

### Freeze Frame Detection

A freeze frame is defined as a still picture. This condition is detected by analyzing the temporal variation of the decoded pictures. If the temporal variation is less than a certain threshold for a period of time, a freeze frame condition is deemed to have occurred.

Proposal 2: Addition of the monitoring parameter “ Audio Tone/ Silence detection” to section 7.5.3 “Audio Quality”

Audio Tone/ Silence Detection:

Extended periods of audio tone/silence typically indicate some anomalies in the system. Monitoring for audio tone and silence is important as it can indicate the loss of audio from contribution feeds, encoder/multiplexer configuration errors etc.

___________