| Abstract INTERNATIONAL TELECOMMUNICATION UNION |  |  |  | Focus Group On IPTV |
| --- | --- | --- | --- | --- |
| TELECOMMUNICATION STANDARDIZATION SECTOR STUDY PERIOD 2005-2008 |  |  | FG IPTV-C-1049 |  |
|  |  |  | English only |  |
| WG(s): 2 |  | 7th FG IPTV meeting: Qawra, St Paul’s Bay, Malta, 11 – 18 December 2007 |  |  |
| CONTRIBUTION |  |  |  |  |
| Source: | NTT Corporation, NEC Corporation, Sumitomo Electric |  |  |  |
| Title: | Comments on DOC-0151 “ Quality of Experience Requirements for IPTV” |  |  |  |

This contribution provides comments on the latest working document DOC-0151 “Quality of Experience Requirements for IPTV.

1.  Introduction

The source companies jointly reviewed the latest working document DOC-0151 “Quality of Experience Requirements for IPTV” to brush up and finalize it as an Output Document of FG-IPTV.  The following section provides comments with some proposed resolutions.

The reviewing activities have been motivated by IPTV Working Group, TTC (the Telecommunication Technology Committee) in Japan and the source companies have voluntarily participated in this effort.

2.  Comments with some Resolutions

The followings are comments raised though the review activities on DOC-0151. Proposed modifications are detailed in the attached document.

(1) Clause 7.1

The meaning of  “quality” in the shorter GOPs’ description is ambiguous. In general, shorter GOPs are used for improving the random accessibility and the error recovery performance. Therefore, it is proposed that the sentence should be replaced with the following.

Current sentence: Shorter GOPs improve quality but reduce the improvement to bit rate from compression.

Proposed revision: Shorter GOPs improve quality, such as the performance of random accessibility and error recovery,  but reduce the improvement to bit rate from compression.

(2) Clause 7.1.1 - 7.1.3, North America/Europe/Asia Pacific

Source material formats can not be simply categorized by the three regions, North America, Europe and Asia Pacific. It is appropriate and good enough to specify standardized format types, such as NTSC and PAL/SECAM. Region specific description should be removed.

Proposed revision:

• 	NTSC (North America) or PAL (Europe / Asia Pacific)

- Editorial Comments

(1) Clause 6, the 2nd paragraph

Proposed revision:

Figure 6-1 Figure 1 shows factors contributing to QoE.

(2) Clause 7.1

Identation of itemization in Clause 7.1 seems incorrect.

Current sentences:

• Encoder quality and settings

– Group of Pictures (GOP) structure

• Shorter GOPs improve quality  but reduce the improvement to bit rate from compression.

• Longer GOPs improve maximum compression ratio, but increase channel change time and the amount of damage a lost packet will cause.

• Dynamic GOPs can be used to better handle scene changes and other effects but are not always implemented on STBs. In addition, dynamic GOPs can impact the variability of zapping latency and may complicate mechanisms to increase zapping speed considerably.

– Motion Vector Search Range

• Wider searches provide improved quality but at increased complexity and encoder delay

• Large search ranges are required for high motion content such as sports

– Rate Control

• Mode decisions greatly affect the bit rate

• Proprietary schemes are commonly used to gain competitive advantage

Proposed revision:

•  Group of Pictures (GOP) structure

- Shorter GOPs improve quality, such as the performance of random accessibility and error recovery,  but reduce the improvement to bit rate from compression.

- Longer GOPs improve maximum compression ratio, but increase channel change time and the amount of damage a lost packet will cause.

- Dynamic GOPs can be used to better handle scene changes and other effects but are not always implemented on STBs. In addition, dynamic GOPs can impact the variability of zapping latency and may complicate mechanisms to increase zapping speed considerably.

•  Motion Vector Search Range

– Wider searches provide improved quality but at increased complexity and encoder delay

– Large search ranges are required for high motion content such as sports

•  Rate Control

– Mode decisions greatly affect the bit rate

– Proprietary schemes are commonly used to gain competitive advantage

(3) Clause 7.1.1, Standard definition TV (SDTV): General minimum objectives

Proposed revision:

The recommended minimum bit rate values shown in Table 7-1 Table 1 represent .....

Unless indicated in Table 7-2 Table 6, recommended minimum bit rates ….

(4) Clause 7.1.3, the 4th paragraph

Proposed revision:

The recommended minimum bit rate values shown in Table 7-6 Table 6 represent …..”

(5) Clause 7.2, the 5th paragraph

Proposed revision:

…. in RFC3357 One-way Loss Pattern Sample Metrics20.

(6) Clause 7.2.3, the 2nd paragraph

Proposed revision:

Table 7-10 Table 10 below shows the loss period and ….

(7) Clause 7.3, Figure 7.3

In Figure 7-3, points from Table 7-8, 7-9, 7-10 and 7-11 are plotted. However, the table numbers have been lost perhaps through editing process. It is proposed to add the table numbers to the plotted points.

4.  Summary

It is proposed to consider and resolve the comments presented in Section 2 and Section 3 of this contribution.

_____________