| INTERNATIONAL TELECOMMUNICATION UNION |  |  |  | Focus Group On IPTV |
| --- | --- | --- | --- | --- |
| TELECOMMUNICATION STANDARDIZATION SECTOR STUDY PERIOD 2005-2008 |  |  | FG IPTV-C-1056 |  |
|  |  |  | English only |  |
| WG(s): 2 |  | 7th FG IPTV meeting: Qawra, St Paul’s Bay, Malta, 11 – 18 December 2007 |  |  |
| CONTRIBUTION |  |  |  |  |
| Source: | Nokia Siemens Networks |  |  |  |
| Title: | Mapping of IPTV services with Y.1541 QoS classes |  |  |  |

# Introduction

Section 7.2 of the Working Document on Traffic Management Mechanisms [DOC-152] for the Support of IPTV Services defines a mapping of IPTV services with Y.1541 QoS classes.  The IPTV services are taken from the Working Document on IPTV service scenarios [DOC-149]. This document lists a long list of services which are more or less related to IPTV.

When we map these services we have to consider that several of them are actually services bundles. Tele-learning for example can include interactive video telephone sessions with the teacher, streaming of content items, download of learning material, messaging and chatting sessions and web browsing. Tele-medicine can also include video telephone sessions with a doctor, access to information material, upload of patient supervision information to the doctor and emergency notification of rescue personal for example. Information and entertainment services can be provided as real-time or downloaded video clips, pictures, text messages, files and web pages for example. Interactive games can have different demands on the delay (e.g. a first person shooter, chess game, second life).

All the services usually have different components. For example VoD involves selection of a movie from a content guide which could be downloaded or via web pages, a transaction for renting the movie, streaming control and the actual content stream. The QoS requirements for these different components are different. User generated content can consist of the upload of the content to a network server which can be best effort and streaming of the content to other users from the server which ahs higher QoS requirements.

Some of the services are actually only variations of basic services. Live TV is for example a basic service. Pay per View is just a specific charging model for a live TV service in addition to the usual subscription or free to air model. Multi view is also a specific flavour which allows to select between different video streams in the same way one can select between different audio streams for different languages. Such variations usually do not change the QoS requirements.

Assigning the services to QoS classes as done in Table 7-2 of the Working Document on Traffic Management Mechanisms for the Support of IPTV Services is therefore a useless approach. Instead major service components like video streaming and content download can be assigned to QoS classes. However these cannot be normative assignments as the specific applications and other issues like the use of application layer error recovery mechanisms have to be considered.

# Proposed Changes

The following changes to section 7 of the Working Document on Traffic Management Mechanisms for the Support of IPTV Services are therefore proposed:

Add the following references to section 2:

[ITU-T FGIPTV.SERV]		ITU-T FGIPTV, Service scenarios for IPTV

[ITU-T FGIPTV.ALERR]		ITU-T FGIPTV, Application layer error recovery mechanisms for IPTV

# 7. IPTV service component mapping to IP performance classes

## 7.1 Network QoS classes

- ITU-T Recommendation Y.1541 [ITU-T Y.1541] defines classes of network QoS with objectives for IP performance parameters. Each QoS class defines a specific combination of limits on the performance values. Table 7-1 shows IP network QoS definitions, network performance objectives and guidance for applicability. Classes 6 and 7 are assigned provisionally in [ITU-T Y.1541] and are intended to support the performance requirements of high bit rate user applications that have more stringent loss/error requirements than those supported by classes 0 and 1.

- Table 7-1: IP network QoS class definitions and network performance objectives/Applications

| - QoS class | - IPTD | - IPDV | - IPLR | - IPER | - IPRR | - Applicability |
| --- | --- | --- | --- | --- | --- | --- |
| - 0 | - 100 ms | - 50 ms | - 1 x 10-3 | - 1 x 10-4 | - - | - Real-time, jitter sensitive, - low delay, highly interactive |
| - 1 | - 400 ms | - 50 ms | - 1 x 10-3 | - 1 x 10-4 | - - | - Real-time, jitter sensitive, medium delay, interactive |
| - 2 | - 100 ms | - U | - 1 x 10-3 | - 1 x 10-4 | - - | - Transaction data, low delay, - highly interactive |
| - 3 | - 400 ms | - U | - 1 x 10-3 | - 1 x 10-4 | - - | - Transaction data, medium delay, interactive |
| - 4 | - 1 s | - U | - 1 x 10-3 | - 1 x 10-4 | - - | - Low loss |
| - 5 | - U | - U | - U | - U | - - | - Best effort |
| - 6 | - 100ms | - 50 ms | - 1 x 10-5 | - 1 x 10-6 | - 1 x 10-6 | - High bit rate, strictly low loss, low delay, highly interactive |
| - 7 | - 400ms | - 50 ms | - 1 x 10-5 | - 1 x 10-6 | - 1 x 10-6 | - High bit rate, strictly low loss, medium delay, interactive |
| - U: undefined - QoS Classes 6 and 7 are assigned provisionally in [ITU-T Y.1541] |  |  |  |  |  |  |

## 7.2 Mapping of IPTV service components to Y.1541 classes

IPTV services as listed in [ITU-T FGIPTV.SERV] usually consist of several service components which can have different QoS requirements regarding the transport of related data. This section focuses on the mapping of some key service components to the IP QoS classes defined in [ITU-T Y.1541] and listed above in table 7-1.

Table 7-2 provides suggestions for the mapping. For the actual mapping the specific application and also the use of application layer error recovery mechanisms as for example defined in [ITU-T FGIPTV.ALERR] have to be taken into account.

- Table 7-2: Suggested mapping of key IPTV service components to Y.1541 QoS classes

| IPTV service component | Example IPTV services | IPTV QoS class |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | 5 | 4 | 3 | 2 | 1 | 0 | 7 | 6 |
| Streaming of live TV content | Linear TV including Pay per View and Multi-view |  |  |  |  |  |  |  |  |
| Streaming of video content | VoD, Network PVR, time-shift TV |  |  |  |  |  |  |  |  |
| Streaming of audio content | Music on Demand |  |  |  |  |  |  |  |  |
| Streaming control | VoD, Network PVR, time-shift TV |  |  |  |  |  |  |  |  |
| Download of video content | Push VoD, Near VoD |  |  |  |  |  |  |  |  |
| Upload of video content | User generated content |  |  |  |  |  |  |  |  |
| Download of data | Content guides, pictures, applications download |  |  |  |  |  |  |  |  |
| Access to web pages | Portals, information services |  |  |  |  |  |  |  |  |
| Streaming of live speech | voice call, audio conference |  |  |  |  |  |  |  |  |
| Streaming of live low resolution video content | video telephony, video conference |  |  |  |  |  |  |  |  |
| Interactive message exchange | Chatting |  |  |  |  |  |  |  |  |
| Message exchange | Messaging, Email |  |  |  |  |  |  |  |  |
| Payment Transactions | VoD rental |  |  |  |  |  |  |  |  |

# References

[DOC-149] ITU-T IPTV FG Document 149; Working Document: Service scenarios for IPTV

[DOC-152] ITU-T IPTV FG Document 152; Working Document: Traffic Management Mechanisms for the Support of IPTV Services

____________________