| Abstract INTERNATIONAL TELECOMMUNICATION UNION |  |  |  | Focus Group On IPTV |
| --- | --- | --- | --- | --- |
| TELECOMMUNICATION STANDARDIZATION SECTOR STUDY PERIOD 2005-2008 |  |  | FG IPTV-C-1060 |  |
|  |  |  | English only |  |
| WG(s): 1 |  | 7th FG IPTV meeting: Qawra, St Paul’s Bay, Malta, 11-18 December 2007 |  |  |
| CONTRIBUTION |  |  |  |  |
| Source: | NTT Corporation |  |  |  |
| Title: | Proposal for editorial modification of IPTV Architecture document |  |  |  |

This contribution proposes editorial modifications to the IPTV Architecture document (FG IPTV-DOC-0148).

# Introduction

The IPTV Architecture document FG-IPTV-doc-0148, which was developed at the 6th FG-IPTV meeting (15-19 October, Tokyo, Japan), will be completed at this meeting. This contribution proposes editorial modifications to inadequate descriptions to improve the document for completion.

# Discussion and Proposals

1) The number of Recommendation is wrong.

### 9.2.2   Differences between NGN-non-IMS-based and NGN-IMS-based IPTV architectures

The NGN-IMS-based IPTV architecture uses Core IMS and associated functions such as Service User Profile Function defined in ITU-T Recommendation Y.2021 (formerly Y.IFN) [4] to provide service control functions. The NGN-non-IMS-based IPTV architecture uses Service Control Functions other than Core IMS to provide service control functions.

2) Section 9.3.3 describes Non-NGN Service user profile functions. Therefore, ‘Service Stratum’ should be replaced with ‘Service Control Functions’.

### 9.3.3   Service user profile function

The service user profile function can be used for storing user profiles, subscriber-related location data, and presence status data in the Service Control Functions. The service user profile functional block performs basic data management and maintenance functions. The service user profile functional block is responsible for responses to queries for user profiles.

3)   Section 9.9 defines reference points. However, ‘Functional Block’ in the document is different from ‘Functional Entity’ and the separation of functions is not consistent. This means that the definitions of reference point may not be usable for discussing protocol in SG11. Therefore, this contribution proposes using another name such as ‘Functional Block Reference Point’ that can be distinguished from ‘Reference Point’.

- 9. 9 .1 Functional Block Reference Point E1

E1 is a functional block reference point between an ITF Application Client and IPTV Applications.

It is used to enable ITF to browse and select IPTV services, to make the service configuration.

- 9.9 .2 Functional Block Reference Point E2-Cu

E2-Cu is a logical end-to-end functional block reference point between a Unicast Content Client Function and a Content Delivery Control Function.

It is used to exchange content control messages, i.e., VCR commands them.

- 9.9 .3 Functional Block Reference Point E2-Cm

E2-Cm is a logical end-to-end functional block reference point between a Multicast Content Client Function and a Multicast Delivery.

It is used to exchange content control messages, i.e., IGMP commands.

- 9.9 .4 Functional Block Reference Point E3

E3 is a functional block reference point between a ITF Control Client and a IPTV Service Control Function.

It is used to enable ITF Control Client to initiate a service request to IPTV applications via the IPTV Service Control Function and to identify and prepare for the connection to the Content Delivery Functions, e.g., request for a suitable Content Delivery & Storage Function in VoD case and request for network parameters in the LTV case etc.

- 9.9 .5 Functional Block Reference Point H1

H1 is a logical functional block reference point between a Delivery Network Gateway Function and an Authentication & IP Allocation.

It is used to perform authentication and get necessary network parameters, e.g., IP address etc.,  when the devices within an End-User-Function attach to the access network.

- 9.9 .6 Functional Block Reference Point H2

H2 is a functional block reference point between a Delivery Network Gateway Function and an Access Network Functions.

It provides IP connectivity between them to deliver control messages and media streams.

- 9.9 .7 Functional Block Reference Point A1

A1 is a functional block reference point between IPTV Applications and an IPTV Service Control Function.

It is used to route/forward service signalling between an IPTV Service Control Function and IPTV applications and to forward signalling between IPTV Applications and other functions, such as ITF and Content Delivery Control Functions. Generally, a service request from ITF is forwarded to applications, and the service response, including service parameters, from applications is forwarded to ITF via this interface.

- 9.9 .8 Functional Block Reference Point A2

A2 is a functional block reference point between IPTV Applications and a Content Distribution & Location Function (CDLF).

It is used to enable IPTV applications to request some service parameters from CDLF. For Linear TV Application, it is used to request multicast network parameters, e.g., multicast address; for an On-Demand application, it is used to request the CDLF to identify a suitable CD&SF for media provisioning.

- 9.9 .9 Functional Block Reference Point A3

A3 is a functional block reference point between IPTV Applications and Content Preparation.

It is used to transmit the metadata stored in Content Preparation to IPTV applications.

- 9.9 .10 Functional Block Reference Point A4

A4 is a functional block reference point between IPTV Applications and the Application Profile Function.

It is used for IPTV applications to store and get application profiles. The application profile may include the user subscription. An example is for SD&S to get profiles for personalization.

- 9.9 .11 Functional Block Reference Point C1

C1 is a functional block reference point between a Content Preparation and a Content Delivery Control Functions.

It is used for Content Preparation to configure some policies, e.g., content distribution rules and selection criteria, for Content Delivery Control Functions

- 9.9 .12 Functional Block Reference Point C2

C2 is a functional block reference point between Content Preparation and Content & Data Sources.

It is used as an entry point between CP and SP and it used to exchange contents and metadata between them.

- 9.9 .13 Functional Block Reference Point D1

D1 is a functional block reference point between Content Delivery Control Functions and a Content Delivery & Storage Function.

It is used for CDCF to get the status information about CD&SF. The information may include the load status and the contents catalogue on each CD&SF. The CDCF identifies a suitable CD&SF according to some criteria after receiving a request from IPTV applications or IPTV SCF.

- 9.9 .14 Functional Block Reference Point S1

S1 is a functional block reference point between IPTV SCF and CDLF.

It is used to forward the service signalling, e.g., service request, media resource request, between ITF/IPTV applications and CDLF.

- 9.9 .15 Functional Block Reference Point S2

S2 is a functional block reference point between IPTV SCF and a Service User Profile Function.

It is used to enable IPTV SCF to store and get the service profile. The profile includes user information, e.g., user identity and security information, as well as a service profile specific to an IPTV application.

- 9.9 .16 Functional Block Reference Point S3

S3 is a functional block reference point between IPTV SCF and Resource Control.

It is used for IPTV SCF to request transport resources from Resource Control.

- 9.9 .17 Functional Block Reference Point S4

S4 is a functional block reference point between IPTV SCF and Authentication & IP Allocation.

It is used to exchange information between the Transport stratum and the Service stratum, e.g., IPTV SCF can get the ITF location from Authentication & IP Allocation.

- 9.9 .18 Functional Block Reference Point S5

S5 is a functional block reference point between IPTV SCF and a Content Delivery Control Function.

It is used to exchange messages for session management, e.g., session establishment, modification, or termination.

It may also be used to exchange content control messages, i.e., VCR commands between them.

- 9.9 .19 Functional Block Reference Point T1

T1 is a functional block reference point between Authentication & IP Allocation and Access Network Functions.

It is used for management of network configuration parameters, e.g., IP address, as well as authentication data.

- 9.9 .20 Functional Block Reference Point R1

R1 is a functional block reference point between Resource Control and Access Network Functions.

It is used for Resource Control to retrieve network parameters from the Access Network Function.

- 9.9 .21 Functional Block Reference Point Ud

Ud is a functional block reference point between a Unicast Delivery Function and Unicast Transport Functions.

It is used to deliver unicast streams from the Unicast Delivery Function to ITF via Unicast Transport Functions, and the Home Network.

- 9.9 .22 Functional Block Reference Point Md

Md is a functional block reference point between a Multicast Delivery Function and a Multicast Replication Function.

It is used to deliver media streams in multicast mode.

- 9.9 .23 Functional Block Reference Point Mc

Mc is a functional block reference point between a Multicast Delivery Function and a Multicast Control Point Function.

It is used to exchange messages for the establishment of multicast tree.

4)   The phrase ‘Allocate Delivery Resources’ in the high-level call flow of Figures I-1 and I-2 in Appendix I is unclear because there is no definition of Delivery Resources. These words probably mean Content Delivery & Storage Functions. They should be modified to ‘Allocate Content Delivery & Storage Functions’.

5)   In Figure J-I, Network Hierarchy for an IPTV Network (Linear TV example), there are some inadequate points:

  - Access Network Functions are depicted inside VSO. However, it is incorrect to depict networks inside a physical hierarchical object such as VSO. They should be depicted separately.

  - There is something wrong with the media flows links. Don’t the contents delivered from Multicast Delivery in VSO go through IP Multicast Replication? Do the contents delivered from Multicast Delivery in SHE go through VHO and VSO?

  - Service Control Functions are depicted inside SHE. However, the location of Service Control Functions is not limited to SHE.

These points in the figure should be modified, but discussing appropriate expressions and modifications will be laborious. Therefore, this contribution proposes removing this section so as not to delay the completion of the document at this meeting.

6)   There are some uncorrected and misaligned descriptions regarding the names of functions. For example, title of section 9.6.3.1 is ‘Content Distribution & Location Functions’, but the text explains Content Delivery Control Functions. In addition, ‘the Delivery & Storage Functions’ are described in the same section, but this name of function is not used any more. These descriptions should be corrected.

___________