| Introduction INTERNATIONAL TELECOMMUNICATION UNION |  |  |  | Focus Group On IPTV |
| --- | --- | --- | --- | --- |
| TELECOMMUNICATION STANDARDIZATION SECTOR STUDY PERIOD 2005-2008 |  |  | FG IPTV-C-1066 |  |
|  |  |  | English only |  |
| WG(s): 6 |  | 7th FG IPTV meeting: Qawra, St Paul’s Bay, Malta, 11-18 December 2007 |  |  |
| CONTRIBUTION |  |  |  |  |
| Source: | NTT |  |  |  |
| Title: | Proposed introduction text of Working Document: Service Navigation Systems (FG IPTV-DOC-0164) |  |  |  |

This contribution proposes text to be included in FG IPTV-DOC-0164.

- Discussion

# Service Navigation

Service Navigation is a process of presenting information that allows the end-user to discover, select and consume services.

Service Navigation is an integral part of IPTV because it is expected that there are many services possible on IPTV and without service navigation, it is very difficult, if not impossible, to obtain services.

A typical means of service navigation is by way of Service Navigation Interface, a user interface which is intended to provide information on available services, including content, that may be accessed by end-users for service navigation. Examples of Service Navigation Interfaces include EPG, IPG, ECG, and ESG.

Service Navigation comprises nodes, server and client, and three abstract functional entities: metadata, navigation logic, and presentation. Depending on the way these abstract entities are realized, there may be many different systems or models of service navigation.  In addition to these, (metadata) transport and interval management are the components to be taken into account.

Metadata

Service Navigation

Logic Declaration

Service Navigation

Logic Execution

Service Navigation Logic

Presentation

Declaration

Presentation

Execution

Presentation

Data

Figure X.0: Abstract components of Service Navigation

## Metadata

Metadata is structured, encoded data that describe characteristics of information-bearing entities to aid in the identification, discovery, assessment, and management of the described entities. Metadata has many applications and may vary in depth from merely identifying the content package title or information to populate an service navigation interface to providing a complete index of different scenes in a movie or providing business rules detailing how the content package may be displayed, copied, or sold.

## Navigation Logic

Navigation logic is a means to characterize the process in which service navigation is conducted through service navigation interface.

### Navigation Logic Description

Navigation logic description is the description of the navigation logic in a clearly defined language.

### Navigation Logic Execution

Navigation logic execution is the execution of the navigation logic described by the navigation logic description.

## (Navigation) Presentation

Navigation Presentation is

### Presentation Description

Presentation description is the description, in a clearly defined language, of how the navigation logic is to be presented and displayed, e.g., layout, design, and style, through service navigation interface.

### Presentation Execution

Presentation execution is the actual presentation and displaying of the service navigation logic through the service navigation interface according to the presentation description.

## Examples of Service Navigation Model

Various service navigation models are possible by combining the above functional components in various ways. Figure X.1 shows one of these, which is often called “web-based” because the service navigation interface is often realized by a “web-browser” application.

Metadata DB

Service Navigation

Logic Declaration

Service Navigation

Logic Execution

Presentation

Declaration

Presentation

Execution

Server Side

Terminal Side

Browser

HTML+CSS+

ECMAScript

Figure X1. An example of a Web-based Service Navigation

In this simple model, navigation logic is executed on the server side, meaning that the application on the client side only displays and does not interact dynamically with the end user.

Another example of service navigation is given in Figure X2, in which service navigation logic and presentation are all executed on the client side. The server side only delivers metadata. This is an extreme case of middleware-based service navigation model.

Metadata DB

Presentation

Execution

Server Side

Terminal Side

Browser

Service Navigation

Logic Declaration

Service Navigation

Logic Execution

Presentation

Declaration

Figure X.2: Middleware-based Service Navigation Model

As mentioned above, metadata transport and interval management need to be taken into account.

### Metadata Transport

Metadata over IP is usually transport in either of two ways: Unicast or Multicast.

### Interval Management

Interval management is a process in which temporal intervals between metadata delivery are managed. For example, interval management involves decisions such as, e.g., how often certain metadata is to be updated within a day.

Depending on the way these abstract entities are realized, there may be many different systems or models of service navigation.

- Proposal

This contribution proposes to add the above text to introduction section of Working Document: Service Navigation System, FG IPTV-DOC-0164.

------------------------------