| INTERNATIONAL TELECOMMUNICATION UNION |  |  |  | Focus Group On IPTV |
| --- | --- | --- | --- | --- |
| TELECOMMUNICATION STANDARDIZATION SECTOR STUDY PERIOD 2005-2008 |  |  | FG IPTV-C-1106 |  |
|  |  |  | English only |  |
| WG(s): 4, 6 |  | 7th FG IPTV meeting: Qawra, St Paul’s Bay, Malta, 11-18 December 2007 |  |  |
| CONTRIBUTION |  |  |  |  |
| Source: | WG 6 editors: Christian Bertin, Yasuaki Yamagishi |  |  |  |
| Title: | Protocols to be included in WG4 Working Document on IPTV-related Protocols: |  |  |  |

## 1. Transport Protocols for Content and Metadata provisioning

- FTP (for file transfer)

- RTP/UDP/IP (for streaming)

- Protocols coming from DVB-T, DVB-S, DVB-C, ISDB-T, ISDB-S, etc. (for streaming)

Metadata Protocols Extracted From Metadata Working Document (DOC-0131)

2. Metadata Delivery Protocols

- [ETSI TS 102 539] DVB-STP multicast transport protocol for push mode and the HTTP unicast protocol for pull mode.

- [ETSI TS 102 822-6-1] Unicast query/response based TV-Anytime metadata delivery using Simple Object Access Protocol (SOAP) over HTTP.

- [ETSI TS 102 822-8] Interchange data format for the delivery of TV-Anytime metadata and content referencing information from different data sources

- [ETSI TS 102 471] Metadata transport containers carrying ESG metadata instances are transported in FLUTE dynamic file delivery carousel sessions

In section 8 titled “Metadata delivery scheme” in IPTV Metadata Working Document (DOC-0131), there is the following description in relevant standard listing;

---

[ETSI TS 102 539] specifies push and pull modes of TV-Anytime content description delivery for container-based and query/response-based protocols. DVB adopts the DVB-STP multicast transport protocol for push mode and the HTTP unicast protocol for pull mode as defined in [ETSI TS 102 034]. A profile for unicast query/response delivery using Simple Object Access Protocol (SOAP) over HTTP specified in [ETSI TS 102 822-6-1] is also defined as optional. The query protocol can retrieve any portion of XML instance identified by XPath predefined or dynamically assigned by the content guide service. [ETSI TS 102 822-8] defines the interchange data format for the delivery of TV-Anytime metadata and content referencing information from different data sources.

As per clause 8 of [ETSI TS 102 471] Metadata transport containers carrying ESG metadata instances are transported in FLUTE dynamic file delivery carousel sessions as described for file delivery in [ETSI TS 102 472].

---

3 Metadata Transport Container

- [ETSI TS 102 822-3-2] Structure of the container carrying the TV-Anytime fragment.

- [ETSI TS 102 471] ESG fragments are encapsulated in the ESG Container that provides fragment aggregation, versioning and filtering.

In section 8.2 titled “Metadata Transport Container” in IPTV Metadata Working Document (DOC-0131), there is the following description in relevant standard listing;

---

[ETSI TS 102 822-3-2] specifies the structure of the container carrying the TV-Anytime fragment. It describes the format of the container and additional requirements on the delivery layer to provide container identification, information about the type of data a container carries and the current version of a container.

Per clauses 6 and 7 of [ETSI TS 102 471] ESG fragments are encapsulated in the ESG Container. The ESG Container provides fragment aggregation, versioning and filtering. ESG fragments may be represented in uncompressed, GZIP-compressed and BiM-compressed forms.

---

4 Metadata Change Notification

- [OASIS WSBN] Web Service Notification for a Web service, or other entity, to disseminate event notification to another set of clients to be used as metadata change notification scheme

- [IETF RFC3265] The SIP-specific framework for event notification also specifies a flexible way to provide generalized event notification services to be used as metadata change notification scheme

In section 8.2 titled “Metadata Change Notification” in IPTV Metadata Working Document (DOC-0131), there is the following description in relevant standard listing;

---

Web Services Notification [WSBN] provides a standardized way for a Web service, or other entity, to disseminate event notification to another set of clients. The SIP-specific framework for event notification [IETF RFC3265] also specifies a flexible way to provide generalized event notification services. Both types of event notification can be applied to the publish/subscribe service model for metadata delivery management systems.

---

5 References

[ETSI TS 102 539]	ETSI TS 102 539 V1.1.1 (2006), Digital Video Broadcasting: Carriage of Broadband Content Guide (BCG) information over Internet Protocol

[ETSI TS 102 822-6-1]	ETSI TS 102 822-6-1 V1.3.1 (2006), Broadcast and On-line Services: Search, select and rightful use of content on personal storage systems (“TV-Anytime”); Part 6: Delivery of metadata over a bi-directional network; Sub-part 1: Service and transport

[ETSI TS 102 822-8]	ETSI TS 102 822-8 V1.1.1 (2006), Broadcast and On-line Services: Search, select and rightful use of content on personal storage systems (“TV-Anytime”); Part 8: Phase 2 - Interchange Data Format

[ETSI TS 102 471]	ETSI TS 102 471 V1.2.1 (2006), Digital Video Broadcasting (DVB); IP Datacast over DVB-H: Electronic Service Guide (ESG)

[ETSI TS 102 822-3-2]	ETSI TS 102 822-3-2 V1.3.1 (2006), Broadcast and On-line Services: Search, select and rightful use of content on personal storage systems (“TV-Anytime”); Part 3: Metadata; Sub-part 2: System aspects in a uni-directional environment

[OASIS WSBN]	OASIS  (2006), Web Services Base Notification 1.3 (WS-BaseNotification)

[IETF RFC 3265]	IETFRFC 3265 (2002), Session Initiation Protocol (SIP)-Specific Event Notification

________________