| This document provides text for Working Document: Traffic Management Mechanism for the Support of IPTV Services v2. This was drafted based on FG IPTV-DOC-0037/WG2 and include the following updates: INTERNATIONAL TELECOMMUNICATION UNION |  |  |  | Focus Group On IPTV |
| --- | --- | --- | --- | --- |
| TELECOMMUNICATION STANDARDIZATION SECTOR STUDY PERIOD 2005-2008 |  |  | FG IPTV-DOC-0064 |  |
|  |  |  | English only |  |
| WG(s): 2 |  | 3rd FG IPTV meeting: Mountain View, USA, 22-26 January 2007 |  |  |
| OUTPUT DOCUMENT |  |  |  |  |
| Source: | Acting Editor |  |  |  |
| Title: | Working document: Traffic Management Mechanism for the Support of IPTV Services |  |  |  |

- Include editorial modifications from C279.

- As a disposition to FG IPTV-C-0293, remove Clause 8 and insert Editor’s Note to indicate that appropriate text will be later included based upon output from WG4 (relating to Streaming Protocols) and WG6 (relating to Codec definition).

- Update Clauses 2 and 4 based on the text and directions reflected in FG IPTV-C-0279.

- Add a generic description of Admission Control in Clause 6.1 based upon the text reflected in Section 6.1 of FG IPTV-C-0317.

- Excerpt text in Section 6.1 FG IPTV-C-0317 describing specific sequence details for multicast admission control processing to the living list document ---FG IPTV-DOC-0041/WG2 as a Study Point (SP).

- Correct Table 1 per FG IPTV-C-0346. In addition, add Editor’s Notes per proposal in same contribution to Tables 2 and 3 (i.e. Tables X and XX).

- Develop draft text for Clause 10.1 & 10.2 based upon proposal in FG IPTV-C-0382

- Use FG IPTV-C-0407 as the basis for developing draft text on Admission Control for Clause 6.1

- Use FG IPTV-C-0407 as the basis for developing draft text on buffer management for Clause 6.3.2.

- Use FG IPTV-C-0408 as the basis for developing draft text on the Transmission Scheduler for Clause 6.3.1.

- Use FG IPTV-C-0409 as the basis for developing a re-structuring of the current baseline draft.

- Edit Figure 6-2 to omit the Traffic Restoration function.

Traffic Management Mechanisms for the Support of IPTV Services

Summary

TBD

Keywords

TBD

Introduction

TBD

Contents

1.	Scope	4

2.	References	4

3.	Definitions	4

4.	Abbreviations	4

5.	Conventions	5

6.	Traffic Management Mechanisms	5

6.1	Control Plane	7

6.1.1	Admission Control	7

6.2	Data Plane	7

6.2.1	Traffic Conditioning	7

6.2.2	Transmission Schedulers	8

6.2.3	Buffer Management	8

6.3	Multicast	9

7.	IPTV Service Mapping to IP Performance Classes	9

7.1	Network QoS Classes	9

7.2	Association of IPTV Services with Y.1541 Classes	10

7.3	Mapping of IPTV Services with QoS Classes	11

8.	Video Coding and Transmission over IP Networks	12

9.	Traffic Management Capabilities: Core Networks	13

9.1	IP Network Traffic Management Capabilities	13

9.2	IP Traffic Management Capabilities applicable to IPTV	13

10.	Traffic Management Capabilities: Access Networks	13

10.1	Overview of Access Network Technologies	13

10.2	Access Traffic Management Capabilities applicable to IPTV	13

11.	Traffic Management Capabilities: Home Networks	14

# Scope

This working document describes a set of traffic management mechanisms which needs to be implemented at the transport network for the efficient support of IPTV services. The network supporting IPTV services will span a number of network segments, with the possibility of being established by different providers and may differ in their traffic management capabilities. Therefore, it is expected that the network provider (s) will implement a subset of these mechanisms to ensure IPTV service objectives are satisfied efficiently.

Traffic management mechanisms will be discussed for the home, access, and core networks.

# References

The following ITU-T Recommendations and other references contain provisions, which, through reference in this text, constitute provisions of this working document. At the time of publication, the editions indicated were valid. All Recommendations and other references are subject to revision; users of this working document are therefore encouraged to investigate the possibility of applying the most recent edition of the Recommendations and other references listed below. A list of the currently valid ITU-T Recommendations is regularly published.

The reference to a document within this working document does not give it, as a stand-alone document, the status of a Recommendation.

[RFC2250]	IETF RFC 2550 (1998), RTP Payload Format for MPEG 1 and MPEG 2 Video

[RFC3984]	IETF RFC 3984 (2005), RTP Payload Format for H.264 Video

[H.264]	ITU-T Recommendation H.264 (2005), Advanced video coding for generic audiovisual services

[Y.1541]	ITU-T Recommendation Y.1541 (2006), Network performance objectives for IP-based services

[TR59]	DSL Forum TR-59

# Definitions

This Working Document uses or defines the following terms

# Abbreviations

This Working Document uses the following abbreviations.

- BE	Best Effort

- DSCP	Differentiated Services Codepoint

IP	Internet Protocol

IPDV	IP packet Delay Variation

IPER	IP packet Error Ratio

IPLR	IP packet Loss Ratio

IPRR	IP Packet Reordering Ratio

IPTD	IP Packet Transfer Delay

- LL	Low Loss

- MPEG	Moving Picture Experts Group

NAL	Network Adaptation Layer

NALU	Network Adaptation Layer Unit

PDU	Protocol Data Unit

- PPV	Pay Per View

- PVR	Private Video Recorder

QoS	Quality of Service

- RTI	Real-Time Interactive

- RTMU	Real-Time Multicast & Unicast

RTP	Real Time Protocol

VCL	Video Coding Layer

VoD	Video on Demand

VoIP	Voice over IP

VTC	Video Teleconference

# Conventions

Tbd

# Traffic Management Mechanisms

This section provides an overview of the basic traffic management capabilities that are commonly deployed in a managed network. A provider may choose to deploy a subset of these capabilities depending on the performance objectives of applications supported by the network.

Traffic management is a set of generic network mechanisms for controlling the network service response to a service request, which can be specific to a network element, or for signalling between network elements, or for controlling and administering traffic across a network, mainly including bandwidth allocation, admission control, packet classification/marking, congestion management, congestion avoidance, traffic policing, and traffic shaping and line rate constraint, etc. Its basic processing sequence is shown in Figure xx

Transmission

Token bucket

Queue1

Queue2

QueueN

Queue0

Token

Classification/Marking

Traffic policing Traffic Shaping

Discarding

Discarding

Out queuing

CAR

GTS

RED

WRED

In queuing

Other

Processing

Origin address

Destination address

Destination port

Origin port

Protocol type

TOS

Congestion Avoidance

Queue

Packet should

be transmitted

by this interface

FIFO

PQ

CQ

WFQ

CBWF

Figure 6-1:	The basic processing sequence of traffic management

Figure 6-2. Architectural framework for QoS support

## Control Plane

### Admission Control

This mechanism controls the traffic to be admitted into the network. Normally the admission criteria are policy driven [IETF RFC 2753]. Whether traffic is admitted depends on an a priori service level agreement. In addition, the decision can depend on if adequate network resources are available so that newly admitted traffic does not overload the network and degrade service to ongoing traffic. For a service provider, maximal traffic should be admitted while the same level of QoS (including transaction performance as well as service reliability/availability expectations) is maintained for the existing traffic.

Call admission approaches related to transaction performance are typically parameter or measurement based. The parameter-based approach derives the worst-case bounds for a set of metrics (e.g., packet loss, delay and jitter) from traffic parameters and is appropriate for providing hard QoS for real-time services. This approach is typically exercised over a resource reservation request for securing necessary resource for an ensuing traffic flow.

In contrast, the measurement-based approach uses measurements of existing traffic for making an admission decision. It does not warrant throughput or hard bounds on packet loss, delay or jitter and is appropriate for providing soft or relative QoS. This approach has in general higher network resource utilization than the parameter-based one. Note that in principle, it is possible to have a hybrid approach such as using measurements to update the resources available in the parametric approach. Admission control can also be used to meet requirements for service reliability/availability over a specified period for the desired transaction types as negotiated in the SLA. Specifically, the desired service reliability/availability can be requested as a priority level for admission control that, in turn, determines the setup of a "connection" or link such as an LSP. Admission control policies give preference to traffic streams (e.g., for emergency communications) deemed to be more critical by a service provider under conditions of congestion. Admission control priority is a way of giving preference to admit higher priority traffic ahead of lower priority traffic.

For IPTV services, the admission control should be used closely with the hierarchical queue management and signaling for IPTV services. The hierarchical queue management supports the isolation for subscribers. A new IPTV service session of a subscriber should not affect the other subscribers’ sessions. The signaling mechanism can be layer 2 or layer 3 according to the admission control point. The representative L2 and L3 protocols are IGMP and PIM, respectively.

## Data Plane

### Traffic Conditioning

The following are required in access network for supporting multimedia service such as IPTV.

- QoS provisioning per subscriber and per service

- Traffic classification based on multiple header fields (5-tuple, 6-tuple, and etc.)

- Minimum bandwidth guarantee

- Traffic shaping/policing

- Advanced packet queuing and scheduling

- Traffic marking (DSCP, 802.1p, and etc.)

### Transmission Schedulers

This mechanism controls which packets to select for transmission on an outgoing link. Incoming traffic is held in a queuing system, which is made of, typically, multiple queues and a scheduler. Governing the queuing system is the queuing and scheduling discipline it employs. There are several key approaches:

– First-in, first-out queuing: Packets are placed into a single queue and served in the same order as they arrive in the queue.

– Fair queuing: Packets are classified into flows and assigned to queues dedicated to respective flows. Queues are then serviced in round robin. Empty queues are skipped. Fair queuing is also referred to as per-flow or flow-based queuing.

– Priority queuing: Packets are first classified and then placed into different priority queues. Packets are scheduled from the head of a given queue only if all queues of higher priority are empty. Within each of the priority queues, packets are scheduled in first-in, first-out order.

– Weighted fair queuing: Packets are classified into flows and assigned to queues dedicated to respective flows. A queue is assigned a percentage of output bandwidth according to the bandwidth need of the corresponding flow. By distinguishing variable-length packets, this approach also prevents flows with larger packets from being allocated more bandwidth than those with smaller packets.

– Class-based queuing: Packets are classified into various service classes and then assigned to queues assigned to the service classes, respectively. Each queue can be assigned a different percentage of the output bandwidth and is serviced in round robin. Empty queues are skipped.

– Hierarchical scheduling: This model provides IP QoS support across a QoS unware access and/or aggregation network, which is used in BNG (Broadband Network Gateway). A bottleneck point in the access topology can be represented by a node on the scheduler. The maximum number of levels of hierarchy is defined to 5 in DSL Forum TR-59 [TR-59].

### Buffer Management

Queue or buffer management deals with which packets, awaiting transmission, to store or drop. An important goal of queue management is to minimize the steady-state queue size while not underutilizing link as well as avoiding the lock-out phenomenon where a single connection or flow monopolizes the queue space [IETF RFC 2309]. Schemes for queue management differ mainly in the criteria for dropping packets and what packets drop. The use of multiple queues introduces further variation in the schemes, for example, in the way packets are distributed among the queues.

A common criterion for dropping packets is the queue reaching the maximum size. Packets are dropped when the queue is full. What packets drop depends on the drop disciplines, for example:

– "Tail drop" rejects the newly arriving packet. This is the most common strategy.

– "Front drop" keeps the newly arriving packet at the expense of the packet at the front of the queue.

– "Random drop" keeps the newly arriving packet at the expense of a randomly-selected packet from the queue. This scheme can be expensive since it requires a walk through the queue.

A scheme of dropping packets only when the queue is full tends to keep the queue in the full state for a relatively long period of time, which can have a catastrophic result in case of bursty traffic. There are schemes using a more dynamic criterion not based on the fixed maximum size of the queue and thus capable of performing active queue management. A prominent one is Random Early Detection (RED) [Floyd], which also helps address the full queue problem and avoid congestion. RED drops (incoming) packets probabilistically based on an estimated average queue size. The probability for dropping increases as the estimated average queue size grows. In other words, if the queue has been mostly empty in the recent past, incoming packets tend to be kept; if the queue has been mostly relatively full recently, however, incoming packets are likely to be dropped. More specifically, RED employs two thresholds for the average queue size. One specifies the average queue size below which no packets are dropped; the other specifies the average queue size above which all packets are dropped. For a queue of an average size between the two thresholds, the packet dropping probability is proportional to the average size. Naturally the effectiveness of RED depends on how the relevant parameters are set. There is no single set of parameters that work well for all traffic types and congestion scenarios. Thus appear RED variants, for example:

– Flow RED (FRED) [Lin et al., 1997], which introduces additional control to RED by providing differential drop treatment to flows based on their buffer usage. If the number of packets from a flow in the queue is lower than a flow-specific threshold, a newly arriving packet of the same flow will not be dropped. Otherwise, it is subject to drop treatment favoring flows with fewer packets in the buffer. Compared with RED, FRED is more flexible in protecting flows from using less- or more-than-fair share of buffer space and link bandwidth.

– Weighted RED, which introduces additional control to RED by providing differential drop treatment to packets based on their priority. The higher the priority of a packet is, the lower the probability it is to be dropped.

## Multicast

# IPTV Service Mapping to IP Performance Classes

## Network QoS Classes

- ITU-T Recommendation Y.1541 defines classes of network Quality of Service (QoS) with objectives for Internet Protocol performance parameters. Each QoS class defines a specific combination of limits on the performance values. The following Table 1 shows IP network QoS definitions, network performance objectives and guidance for applicability. Class 6 and 7 are intended to support the performance requirements of high bit rate user applications that have more stringent loss/error requirements than those supported by classes 0 through 4.

- Table 1 – IP network QoS class definitions and network performance objectives/Applications

| - QoS class | - IPTD | - IPDV | - IPLR | - IPER | - IPRR | - Applications (examples) |
| --- | --- | --- | --- | --- | --- | --- |
| - 0 | - 100 ms | - 50 ms | - 1 x 10-3 | - 1 x 10-4 | - - | - Real-time, jitter sensitive, - high interaction (VoIP, VTC) |
| - 1 | - 400 ms | - 50 ms | - 1 x 10-3 | - 1 x 10-4 | - - | - Real-time, jitter sensitive, - Interactive |
| - 2 | - 100 ms | - U | - 1 x 10-3 | - 1 x 10-4 | - - | - Transaction data, - highly interactive (Signalling) |
| - 3 | - 400 ms | - U | - 1 x 10-3 | - 1 x 10-4 | - - | - Transaction data, interactive |
| - 4 | - 1 s | - U | - 1 x 10-3 | - 1 x 10-4 | - - | - Low loss only (short transaction, bulk data, video streaming) |
| - 5 | - U | - U | - U | - U | - - | - Traditional applications of - default IP network |
| - 6 | - 100ms | - 50 ms | - 1 x 10-5 | - 1 x 10-6 | - 1 x 10-6 | - High bit rate, strictly low loss/error (TV broadcast on IP) |
| - 7 | - 400ms | - 50 ms | - 1 x 10-5 | - 1 x 10-6 | - 1 x 10-6 | - High bit rate, strictly low loss/error |

- Editor Note: further clarification is needed on the IPLR values for classes 6 and 7.

## Association of IPTV Services with Y.1541 Classes

- This section presents classified sets of IPTV services by network QoS requirements and associates them with IP QoS classes defined in Y.1541. We classify IPTV services into 5 categories. The first set is named Best Effort (BE) service. In this set, services are regarded as traditional default IP network applications. They do not need to be real-time, and errors can be recovered using transmission control protocol on the best effort network. So this equals to the QoS class 5 of Y.1541. The second set is Low Loss (LL) service set. This set is equal to the QoS class 4, because of its features of short transaction and bulk data. The next sets are Interactive (I) service, Real-Time Interactive (RTI) service, and Real-Time Multicast & Unicast (RTMU) service. And they are corresponding to QoS class 2/3, QoS class 0/1 and QoS class 6/7, respectively. And Table 2 also shows the IPTV service examples of each category.

- Table 2 - Association of IPTV services with Y.1541 QoS classes

| - IPTV service category | - IP QoS class | - IPTV service examples |
| --- | --- | --- |
| - Best Effort (BE) service | - QoS class 5 | - content download service |
|  |  | - regulatory information service |
|  |  | - T-information |
|  |  | - e-mail |
| - Low Loss (LL) service | - QoS class 4 | - Download VOD, MOD |
|  |  | - T-commerce |
|  |  | - T-learning based on VOD |
| - Interactive (I) service | - QoS class 2/3 | - messenger |
|  |  | - T-learning based on interactive |
| - Real-Time Interactive (RTI) service | - QoS class 0/1 | - VOIP, video phone |
|  |  | - multi player game |
| - Real-Time Multicast & Unicast (RTMU) service | - QoS class 6/7 | - linear/broadcast |
|  |  | - multi angle |
|  |  | - pay per view (PPV) |
|  |  | - networked personal video record (PVR, time-shift) |

Editor Notes:

- The above table is just a starting point.

- Each QoS class shows one of the candidate requirements.

- The consistency with Y.1541 needs to be considered.

- In determining the required value for a QoS class, considering the error correction functions, such as FEC, and their performance is necessary.

## Mapping of IPTV Services with QoS Classes

- This section defines the mapping of Y.1541 QoS classes on IPTV services currently identified..

- Editor Note: The list and definition of IPTV services still needs to be clarified further , according to the WG1 work progress on defining IPTV services. Thus, this table should be updated after the service list and each IPTV service scenarios are clearly defined. The identified services are examples pending WG 1 outcome.

- Table 3 - Mapping of IPTV services with Y.1541 QoS classes

| IPTV services | IPTV service category/QoS class |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | BE | LL | I |  | RTI |  | RTMU |  |
|  | 5 | 4 | 3 | 2 | 1 | 0 | 7 | 6 |
| Linear/broadcast TV (audio, video and data) |  |  |  |  |  |  |  |  |
| Multi-angle service |  |  |  |  |  |  |  |  |
| Time-shift TV (network PVR) |  |  |  |  |  |  |  |  |
| Pay Per View (PPV) (Near Vod) |  |  |  |  |  |  |  |  |
| Real VoD |  |  |  |  |  |  |  |  |
| Downloaded based video content distribution (Push VoD) |  |  |  |  |  |  |  |  |
| Content download service |  |  |  |  |  |  |  |  |
| Consumer originated content (video, etc. and applications) |  |  |  |  |  |  |  |  |
| Consumer originated broadcast |  |  |  |  |  |  |  |  |
| Linear broadcast audio |  |  |  |  |  |  |  |  |
| Music on demand including audio book |  |  |  |  |  |  |  |  |
| Pictures |  |  |  |  |  |  |  |  |
| T-learning based on VoD |  |  |  |  |  |  |  |  |
| T-learning based on interactive |  |  |  |  |  |  |  |  |
| Single player games (game on demand) |  |  |  |  |  |  |  |  |
| Multiplayer games |  |  |  |  |  |  |  |  |
| T-information (news, weather, transportation, etc.) |  |  |  |  |  |  |  |  |
| T-commerce (banking, stock, shopping, ticketing, auction, delivery, event etc.) |  |  |  |  |  |  |  |  |
| T-communication data (e-mail, SMS, web surfing, etc.) |  |  |  |  |  |  |  |  |
| T-communication interactive (messenger, channel chatting) |  |  |  |  |  |  |  |  |
| T-communication voice (VoIP, multiple video conference, video phone, etc.) |  |  |  |  |  |  |  |  |
| T-entertainment data (photo album, lottery, blog, etc.) |  |  |  |  |  |  |  |  |
| T-entertainment VOD (karaoke) |  |  |  |  |  |  |  |  |
| Presence service |  |  |  |  |  |  |  |  |
| Communication messaging |  |  |  |  |  |  |  |  |

Editor Notes:

- The above table is just a starting point.

- Each QoS class shows one of the candidate requirements.

- The consistency with Y.1541 needs to be considered.

- In determining the required value for a QoS class, considering the error correction functions, such as FEC, and their performance is necessary.

# Video Coding and Transmission over IP Networks

Editor’s Note:  Appropriate text will be included based upon output from WG4 (relating to Streaming Protocols) and WG6 (relating to Codec definition).

# Traffic Management Capabilities: Core Networks

## IP Network Traffic Management Capabilities

This sub-clause describes an overview of the IP traffic management capabilities including those related to integrated services and differentiated service.

## IP Traffic Management Capabilities applicable to IPTV

This sub-clause describes those IP traffic management mechanisms applicable to traffic management. The discussion should include IPTV delay and loss requirements and how they can be satisfied using the available techniques.

# Traffic Management Capabilities: Access Networks

## Overview of Access Network Technologies

Access networks should have means to control the following functions for traffic management:

- per packet/flow and per class basis CAC (Connection Admission Control)

- per user and per service QoS provisioning

- per packet/flow Mapping between DSCP and 802.1p

## Access Traffic Management Capabilities applicable to IPTV

Generic mechanisms are required for traffic management of upstream and downstream directions in an access network.

- Upstream

To support quality of service in access networks including broadcasting traffic, network nodes should have the following requirements in the upstream direction.

- Guarantee of minimum bandwidth

- Guarantee of bandwidth quality per service (e.g. VoIP, Video conference, and IPTV).

To address the above 2 requirements, the following functions should be supported:

- 5-tuple based Classification & Policing

- Priority Queuing

- Traffic shaping

- Downstream

To support quality of service in access networks including broadcasting traffic, network nodes should have the following requirements in the downstream direction.

- Guarantee of minimum bandwidth per service

To support the above requirement access networks should have the following functions in downstream side.

- per packet/flow and per class basis CAC (Connection Admission Control) for broadcast channels

- Traffic shaping/policing per subscriber

- Support of CoS (Class of Service) based DSCP

      - 5-tuple classification

      - DSCP marking

      - Mapping between DSCP and 802.1p

# Traffic Management Capabilities: Home Networks

This clause describes home network environment and the need for traffic management.

____________